| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–27 January 2024 | Document: JVET-AG0168-v1 |
| --- | --- |

| Title: | AHG9: Proposed update for the SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This contribution (JVET-AG0168) combines the proposed modifications for the SEI processing order SEI message in JVET-AG0166 and JVET-AG0165 and other related text changes as the update for JVET-AF2027.

Introduction

Contribution JVET-AG0165 proposes to modify the syntax structure for signlling the number of the processing-order-nested SEI messages and the processing orders for each of the target processing order id pon_target_po_id[ i ], adding flexibilities for multiple SEI processing order SEI messages with different values of po_id in CVS.

Contribution JVET-AG0166 proposes to group the SEI prefix data related syntax structure as a new SEI message, as the proponents view the wrapping flag and/or the prefix flag equal to 1 as the special cases for the SEI processing order SEI message and in general use-cases the two flags may not be equal to 1. The proposed grouping may further simplify the syntax structure of the SEI processing order SEI message.

# Proposal

This contribution (JVET-AG0168) combines the proposed modifications for the SEI processing order SEI message in JVET-AG0166 and JVET-AG0165 and other related text changes as the update for JVET-AF2027.

D.2.1	General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| SeiExtensionBitsPresentFlag = 0 |  |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| … |  |
| else if( payloadType = = 213 ) |  |
| sei_processing_order( payloadSize ) |  |
| else if( payloadType = = 214 ) |  |
| processing_order_nesting( payloadSize ) |  |
| else if( payloadType = = 215 ) |  |
| processing_order_prefix_matching( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 214 ) |  |
| processing_order_nesting( payloadSize ) |  |
| else if( payloadType = = 215 ) |  |
| processing_order_prefix_matching( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( SeiExtensionBitsPresentFlag ¦ ¦ more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| sei_reserved_payload_extension_data | u(v) |
| sei_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| sei_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Subpicture level information | The CVS containing the SLI SEI message and up to but not including the next CVS, in decoding order, that contains an SLI SEI message with different content |
| … | … |
| SEI processing order | Depending on the SEI messages indicated within the syntax. Each SEI message identified within the SEI processing order SEI message has the same persistence scope as if the SEI message was carried outside of the SEI processing order SEI message and not identified within an SEI processing order SEI message. For each value of po_id, the number of SEI messages and the type codes of the SEI messages indicated within the SEI processing order SEI message persist in decoding order from the first access unit in decoding order until the end of the CVS in output order. |
| Processing order nesting | Depending on the processing-order-nested SEI messages. Each processing-order-nested SEI message has the same persistence scope as if the SEI message was not nested. |
| Processing order prefix matching | Depending on the processing-order-prefix-matched SEI messages. Each processing-order-prefix-matched SEI message has the same persistence scope. |

...

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 47, 129, 132, 137, 142, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 179, 180, 200 to 202, inclusive, and 204 to 214215, inclusive.

...

## D.11 SEI processing order SEI message

D.11.1 SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

### D.11.2 SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for a group of types of SEI messages that may be present in a CVS.

The semantics of the SEI processing order SEI message uses the concept of types of SEI messages. SEI messages that have different payloadType values are considered different types of SEI messages. Additionally, different SEI messages that have the same payloadType value but are differentiated by values of syntax elements in the SEI payload are considered different types of SEI messages. Such differentiation by values of syntax elements in the SEI payload is to be performed by comparing values sent using popm_sei_prefix_data_bit[ i ][ j ][ n ] syntax elements in a processing order prefix matching SEI message (when present) or values sent as SEI messages within a processing order nesting SEI message (when present). For example, neural-network post-filter characteristics (NNPFC) SEI messages can be differentiated by having different nnpfc_id values.

When an SEI processing order SEI message with a particular value of po_id is present in any access unit of a CVS, an SEI processing order SEI message with that particular value of po_id shall be present in the first access unit of the CVS in decoding order. The number of SEI messages and the payloadType codes of the SEI messages indicated within each SEI processing order SEI message with the same value of po_id persists in decoding order from the current access unit until the end of the CVS in output order.

The SEI processing order prefix matching SEI message can carry one or more SEI prefix indications of a particular payloadType. When present, each SEI prefix indication is a bit string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload. These SEI prefix indications should provide sufficient information to determine the specific processing order for types of SEI messages having the same value of payloadType but a different preferred processing order.

po_id contains an identifying number to identify the SEI processing order SEI message.

Each SEI message in the group of SEI messages for which preferred processing order information is provided in an SEI processing order SEI message is identified by the syntax elements po_sei_payload_type[ i ], po_sei_wrapping_flag[ i ], po_sei_processing_order[ i ] and, po_sei_prefix_flag[ i ].  when present, po_num_bits_in_prefix_indication_minus1[ i ] and po_prefix_data_bit[ i ][ j ].

For each picture, there can be multiple persisting or activated SEI messages belonging to one or more groups of SEI messages.

NOTE 1 – Groups of SEI messages can be alternatives to each other, i.e., such that at most one group is chosen to be applied, or they can be complementary, i.e., such that more than one group is chosen and applied separately, with each group generating one output.

po_num_sei_messages_minus2 plus 2 indicates the number of types of SEI messages for which the preferred order of processing is indicated in the SEI processing order SEI message.

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with both of the following constraints should be present:

- pon_target_po_id[ j ] with any value of j is equal to po_id.

- There is a k-th loop entry in the SEI processing order nesting SEI message such that the payloadType of the k-th nested SEI message is equal to po_sei_payload_type[ i ] and pon_processing_order[ po_id ][ k ] is equal to po_sei_processing_order[ i ].

When po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] (and, when po_sei_prefix_flag[ i ] equal to 1, prefix data that matches the r-th values of popm_sei_prefix_data_bit[ po_id ] [ ir ][ j ]) should be present outside of the processing order nesting SEI message. However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ] or po_sei_wrapping_flag[ i ] is equal to 0 and po_sei_prefix_flag[ i ] is equal to 1 and no SEI message is present with with payloadType equal to po_sei_payload_type[ i ] that has prefix data that matches the values of popm_sei_prefix_data_bit[ po_id ][ ir ][ j ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.

NOTE 2 – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SEI processing order SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the type of SEI message with index i.

If the decoding system cannot interpret or does not support the functionality indicated by any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SEI processing order SEI message.

po_sei_payload_type[ i ] specifies the payloadType value of the i-th type of SEI message.

po_sei_prefix_flag[ i ] equal to 1 specifies that po_num_bits_in_prefix_indication_minus1[ i ] and some po_sei_prefix_data_bit[ i ][ j ] syntax elements are present. po_sei_prefix_flag[ i ] equal to 0 specifies that these syntax elements are not present.

po_sei_prefix_flag[ i ] equal to 1 specifies that one or more processing order prefix matching SEI messages with both of the following constraints should be present:

- popm_target_po_id[ j ] with any value of j is equal to po_id.

- There is a r-th loop entry in the SEI processing order prefix matching SEI message such that the payloadType of the r-th matched SEI message is equal to po_sei_payload_type[ i ] and popm_processing_order[ po_id ][ r ] is equal to po_sei_processing_order[ i ].

NOTE 3 – po_sei_prefix_flag[ i ] equal to 1 indicates the SEI prefix data to be carried within the processing order prefix matching SEI message.

SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 165, 177, 210, and 211. The value of po_sei_payload_type[ i ] for each i in the range of 0 to po_num_sei_messages_minus2 + 1, inclusive, shall be equal to a value in SeiProcessingOrderSeiList.

po_sei_processing_order[ i ] indicates the preferred order of processing of the i-th type of SEI message for which preferred processing order information is provided in the SEI processing order SEI message. For any two different integer values of m and n, po_sei_processing_order[ m ] less than po_sei_processing_order[ n ] indicates that the type of SEI message associated with index m should be processed before the type of SEI message associated with index n, and po_sei_processing_order[ m ] equal to po_sei_processing_order[ n ] indicates that there is no preferred order of processing between the types of SEI messages associated with indexes m and n (e.g., they can indicate different properties that are both applicable at that stage, or alternative processes that can be applied, or one can indicate a property and the other can indicate a process).

For i greater than 0, po_sei_processing_order[ i ] shall be greater than or equal to po_sei_processing_order[ i − 1 ].

po_num_bits_in_prefix_indication_minus1[ i ] and po_sei_prefix_data_bit[ i ][ j ], when present, have the same semantics as the num_bits_in_prefix_indication_minus1[ i ] and sei_prefix_data_bit[ i ][ j ] syntax elements of the SEI prefix indication SEI message, with prefix_sei_payload_type replaced by po_sei_payload_type[ i ].

When more than one SEI processing order SEI message with a particular value of po_id is present in a CVS, the values of po_num_sei_messages_minus2 and, for each value of i, the values of po_sei_wrapping_flag[ i ], po_sei_prefix_flag[ i ], po_sei_importance_flag[ i ], po_sei_payload_type[ i ], po_sei_processing_order[ i ] shall be the same as in the other SEI processing order SEI messages in the CVS with the same value of po_id.

po_byte_alignment_bit_equal_to_one shall be equal to 1.

## D.12 Processing order nesting SEI message

### D.12.1 Processing order nesting SEI message syntax

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_po_ids_minus1 | u(8) |
| for( i = 0; i <= pon_num_po_ids_minus1; i++ ) { |  |
| pon_target_po_id[ i ] | u(8) |
| pon_num_seis_minus1[ i ] | u(8) |
| for(ij = 0; ij <= pon_num_seis_minus1[ i ]; ij++ ) { |  |
| pon_processing_order[ i ] [ j ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |
| } |  |

### D.12.2 Processing order nesting SEI message semantics

The processing order nesting SEI message includes one or more SEI messages that should be applied only as parts of the processing chain identified by an one or more associated SEI processing order SEI messages and should not be applied in a manner that would contradict with the processing chain identified by the associated SEI processing order SEI messages.

The SEI messages contained in the processing order nesting SEI message are also referred to as the processing-order-nested SEI messages.

The persistence of all the SEI messages included in the same processing order nesting SEI message shall be the same.

pon_num_po_ids_minus1 plus 1 specifies the number of the SEI processing order SEI messages SEI associated with this processing order nesting SEI message.

pon_target_po_id[ i ] indicates the po_id of the i-th associated SEI processing order SEI message.

pon_num_seis_minus1[ i ] plus 1 specifies the number of the processing-order-nested SEI messages that are included in this SEI message associated with the i-th SEI processing order SEI message (with po_id equal to pon_target_po_id[ i ]).

pon_processing_order[ i ][ j ] specifies the position of the ij-th processing-order-nested SEI message within the processing order defined by the i-th associated SEI processing order SEI message (with po_id equal to pon_target_po_id[ i ]). When ij is greater than 0, pon_processing_order[ i ][ j ] shall be greater than or equal to pon_processing_order[ i ] [ j - 1 ].

For each associated SEI processing order SEI message with po_id equal to pon_target_po_id[ i ] for i in the range of 0 to pon_num_po_ids_minus1, inclusive, there shall be at least one value of ij in the range of 0 to pon_num_seis_minus1[ i ], inclusive, in the processing order nesting SEI message for which the i-th associated SEI processing order SEI message has some entry k for which all of the following are true:

–	po_sei_processing_order[ k ] is equal to pon_processing_order[ po_id ][ j ]

–	po_sei_payload_type[ k ] is equal to the payloadType value of the ij-th processing-order-nested SEI message corresponding to po_id.

–	When po_sei_prefix_flag[ k ] is equal to 1, po_sei_prefix_data_bit[ k ][ jm ] for jm in the range of 0 to po_num_bits_in_prefix_indication_minus1[ k ], inclusive, contain the same content as the po_num_bits_in_prefix_indication_minus1[ k ] plus 1 initial bits of the SEI message payload of the ij-th processing-order-nested SEI message corresponding to po_id.

The ij-th processing-order-nested SEI message should be applied as the k-th loop entry of the associated SEI processing order SEI message with po_id equal to pon_target_po_id[ i ].

## D.13 Processing order prefix matching SEI message

### D.13.1 Processing order prefix matching SEI message syntax

| processing_order_prefix_matching( payloadSize ) { | Descriptor |
| --- | --- |
| popm_number_po_ids_minus1 | u(8) |
| for( i = 0; i <= popm_num_po_ids_minus1; i++ ) { |  |
| popm_target_po_id[ i ] | u(8) |
| popm_num_sei_prefix_indications_minus1[ i ] | u(8) |
| for( j = 0; j <= popm_num_sei_prefix_indications_minus1[ i ]; j++ ) { |  |
| popm_processing_order[ i ][ j ] | u(8) |
| popm_num_bits_in_prefix_indication_minus1[ i ][ j ] | b(8) |
| for( n = 0; n <= popm_num_bits_in_prefix_indication_minus1[ i ][ j ]; n++ ) |  |
| popm_sei_prefix_data_bit[ i ][ j ][ n ] | u(1) |
| while( !byte_aligned( ) ) |  |
| popm_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |
| } |  |

### D.13.2 SEI processing order prefix matching SEI message semantics

The processing order prefix matching SEI message includes one or more prefix indications of SEI messages with payloadType equal to po_sei_payload_type[ i ] when po_sei_prefix_flag[ i ] equal to 1. The SEI messages with the prefix data matching the prefix indications contained in this SEI message should be applied only as parts of the processing chain identified by one or more associated SEI processing order SEI messages and should not be applied in a manner that would contradict with the processing chain identified by the associated SEI processing order SEI messages.

The SEI messages with SEI prefix data matching the SEI prefix indications contained in the processing order prefix matching SEI message are also referred to as the processing-order-prefix-matched SEI messages

popm_num_po_ids_minus1 plus 1 specifies the number of the SEI processing order SEI messages SEI associated with this processing order prefix matching SEI message.

popm_target_po_id[ i ] indicates the po_id of the i-th associated SEI processing order SEI message.

popm_num_sei_prefix_indications_minus1[ i ] plus 1 specifies the number of the prefix indications of the processing-order-prefix-matched SEI messages that are included in this SEI message associated with the i-th SEI processing order SEI message (with po_id equal to popm_target_po_id[ i ]).

popm_processing_order[ i ][ j ] specifies the position of the j-th processing-order-prefix-matched SEI message within the processing order defined by the i-th associated SEI processing order SEI message. When j is greater than 0, popm_processing_order[ i ][ j ] shall be greater than or equal to popm_processing_order[ i ][ j − 1 ].

For each associated SEI processing order SEI message with po_id equal to popm_target_po_id[ i ] for i in the range of 0 to popm_num_po_ids_minus1, inclusive, there shall be at least one value of j in the range of 0 to popm_num_sei_prefix_indications_minus1[ i ], inclusive, in the processing order prefix matching SEI message for which the i-th associated SEI processing order SEI message has some entry r for which all of the following are true:

–	po_sei_processing_order[ r ] is equal to popm_processing_order[ po_id ][ j ]

- po_sei_payload_type[ r ] is equal to the payloadType value of the j-th processing-order-prefix-matched SEI message corresponding to po_id.

–	When po_sei_prefix_flag[ r ] is equal to 1, popm_sei_prefix_data_bit[ po_id ][ r ][ n ] for n in the range of 0 to popm_num_bits_in_prefix_indication_minus1[ po_id ][ r ], inclusive, contain the same content as the popm_num_bits_in_prefix_indication_minus1[ po_id ][ r ] plus 1 initial bits of the SEI message payload of the j-th processing-order-prefix-matched SEI message corresponding to po_id.

The j-th processing-order-prefix-matched SEI message should be applied as the r-th loop entry of the associated SEI processing order SEI message (with po_id equal to popm_target_po_id[ i ]).

popm_num_bits_in_prefix_indication_minus1[ po_id ][ j ] and popm_sei_prefix_data_bit[ po_id ][ j ][ n ], for the j-th associated SEI processing order SEI message, when present, have the same semantics as the num_bits_in_prefix_indication_minus1[ j ] and sei_prefix_data_bit[ j ][ n ] syntax elements of the SEI prefix indication SEI message, with prefix_sei_payload_type replaced by po_sei_payload_type[ j ].

popm_byte_alignment_bit_equal_to_one shall be equal to 1.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).