| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0179-v1 |
| --- | --- |

| Title: | EE1-1.1-related: HOP filter complexity alignment with wider activation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Chaoyi Lin, Junru Li, Kai Zhang, Li Zhang (Bytedance) | Email: | yue.li@bytedance.com |
| Source: | Bytedance |  |  |

_____________________________

- Abstract

EE1-1.1 investigates a refined joint filter architecture for HOP by combining key components from several JVET-AF contributions including JVET-AF0102, JVET-AF0103, JVET-AF0182. EE1-1.1 tries to align the complexity of the joint filter architecture with the HOP.2 filter in NNVC-7.1 by increasing the number of residual blocks. This contribution proposes to achieve the complexity alignment in the other way around, i.e. by increasing the number of feature maps.

BD-rate changes on top of NNNC-7.1 HOP anchor (NN intra + HOP.2) are reportedly summarized as below:

RA: -0.16%, -0.38%, -0.69%, AI: -0.21%, -0.40%, -0.12%

kMAC/pixel: 442

Introduction

At JVET-AF meeting, several contributions have been submitted to the HOP category: JVET-AF0102 [1], JVET-AF0103 [2], JVET-AF0182 [3], JVET-AF0192 [4]. JVET-AF0182 simplifies HOP architecture by introducing group convolution into the residual block, JVET-AF0102 refines HOP architecture by reducing decomposition rank and alternating two types of residual blocks with different decomposition order, while JVET-AF0103 optimizes network head in HOP architecture by applying 1×1 convolution for sparse inputs including IPB and BS. EE1-1.1, investigates a joint filter architecture combining above elements as shown in Fig. 1. Note that the number of residual blocks has been increased to achieve the complexity alignment.

Fig. 1. HOP architecture in EE1-1.1 test.

Proposed HOP Filter Architecture

This contribution develops a filter architecture for HOP on top of EE1-1.1, where complexity alignment is achieved by increasing the number of feature maps rather than increasing the network depth. As shown in Fig. 2, numbers of feature maps before the activation layer in a residual block, i.e. C1 and C22, increase to 192 and 64, enabling an effect of wider activation.

Fig. 2. Proposed refined architecture for HOP EE1-1.1 test.

Inference

SADL is used for performing the inference of the proposed NN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.34M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | Total Parameters Number × 2 |
|  | Multiply Accumulate (kMAC/pixel) | 442 |
| Optional | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

Same training procedure as EE1-1.1 is applied. The network information in the training stage is provided in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | 200h |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |

Experimental results

The proposed model is integrated into NNVC-7.1 and tested according to the common test conditions defined in [4].

Compared with NNVC-7.0 HOP.2 anchor (nn intra + hop.2), the proposed method brings on average {-0.16%, -0.38%, -0.69%} and {-0.21%, -0.40%, -0.12%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 HOP2 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.08% | 0.78% | -0.28% | 72% | 61% |
| Class A2 | -0.21% | 0.06% | -0.44% | 74% | 62% |
| Class B | -0.18% | -1.12% | -0.43% | 71% | 60% |
| Class C | -0.16% | -0.63% | -1.53% | 73% | 58% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.38% | -0.69% | 72% | 60% |
| Class D | -0.08% | -0.57% | -2.13% | 72% | 58% |
| Class F | -0.18% | 0.01% | -0.28% | 68% | 61% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-7.1 HOP2 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.34% | -0.41% | -0.60% | 77% | 61% |
| Class A2 | -0.19% | -0.18% | -0.28% | 84% | 62% |
| Class B | -0.17% | -0.42% | 0.39% | 85% | 56% |
| Class C | -0.18% | -0.33% | -0.19% | 86% | 62% |
| Class E | -0.22% | -0.68% | -0.27% | 75% | 59% |
| Overall | -0.21% | -0.40% | -0.12% | 82% | 60% |
| Class D | -0.14% | 0.11% | -0.59% | 93% | 60% |
| Class F | 0.00% | -0.34% | 0.59% | 89% | 60% |

Conclusion

This contribution proposes an improved filter architecture for HOP on top of EE1-1.1, where the complexity alignment is achieved with wider activation. Proposed architecture achieves better performance than HOP.2. It is proposed to study this architecture in next round of EE.

References

- EE1-1.1.2: Complexity-performance trade off of decompostion, JVET-AF0102, D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

- EE1-1.1.3: Study on input feature set optimization, JVET-AF0103, D. Rusanovskyy, Y. Li, M. Karczewicz (Qualcomm)

- EE1-1.1.4.b: Group convolution for HOP, JVET-AF0182, Y. Li, K. Zhang, L. Zhang (ByteDance)

- Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AF2016, E. Alshina, R. -L. Liao, S. Liu, A. Segall, Hannover, DE, October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).