| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0186 (v2) |
| --- | --- |

| Title: | Non-EE2: FIBC Extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Wei Chen, Che-Wei Kuo, Ning Yan, Changyue Ma, Xianglin Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | { jhuhong-jheng, xiaoyuxiu, chenwei06, cheweikuo, yanning03, machangyue, xianglinwang }@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to support multiple template shapes and multiple models for the FIBC. Compared to ECM-11.0 anchors, simulation results are summarized as follows:

For class F sequences:

AI { -0.13%, -0.12%, -0.07%, 101.2%, 99.2% }, RA { -0.13%, -0.16%, -0.31%, 100.3%, 99.9% }

For TGM sequences:

AI { -0.06%, -0.02%, -0.09%, 101.0%, 100.1% }, RA { -0.08%, -0.13%, -0.05%, 100.4%, 99.9% }

# Proposal

In ECM-11.0 [1], the FIBC method is limited to the use of L-shaped template and single model for deriving linear model parameters. In this contribution, two modifications are proposed to the FIBC to improve its coding efficiency on screen content sequences (SCC): multiple template shapes and multiple models.

Firstly, besides the existing L-shaped template, two new FIBC modes are introduced to allow using top-only or left-only neighboring reconstructed samples, respectively, for the filter derivation.

Secondly, similar to MMLM, two linear models are proposed for the FIBC which are derived based on the classification of the template samples of the current CU according to their corresponding DC value.

# Simulation Results

The proposed method was implemented based on the ECM-11.0 [1] reference software. The performance was evaluated using the common test conditions as defined in [2].

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class F | -0.13% | -0.12% | -0.07% | 101.2% | 99.2% |
| Class TGM | -0.06% | -0.02% | -0.09% | 101.0% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.13% | -0.16% | -0.31% | 100.3% | 99.9% |
| Class TGM | -0.08% | -0.13% | -0.05% | 100.4% | 99.9% |

# Conclusions

This contribution proposes to support multiple template shapes and multiple models for the FIBC. The results show that the method provides promising coding gains with negligible encoding/decoding impacts. It is recommended to study the proposed method into EE.

# References

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, 25th Meeting, by teleconference, 12–21 January 2022

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

_______________________