| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0187-v1 |
| --- | --- |

| Title: | AHG16: Study text for common test conditions and evaluation procedures for generative face video coding (draft 1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sean McCarthy, Peng Yin (Dolby) Bolin Chen, Yan Ye (Alibaba) Shiqi Wang (City University of Hong Kong) | Email: | sean.mccarthy @dolby.com {chenbolin.chenboli, yan.ye}@alibaba-inc.com shiqwang@cityu.edu.hk |
| Source: | Dolby Laboratories, Inc., Alibaba, CityU |  |  |

_____________________________

# Abstract

This document proposes draft text to define common test conditions and software reference configurations for testing of generative video face coding. The conditions and configurations are tentatively recommended for use in applicable technical contributions to JVET meetings.

# Introduction

JVET-AG0042 (AHG 16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression) was discussed at the AHG16 generative face video coding (GFVC) conference call held on 1400-1600 UTC December 7, 2023. At that AHG meeting, it was agreed that testing conditions should be documented in the AHG16 report. The text in this contribution is intended for that purpose. Specifically, the text in this contribution is intended as draft text to define common test conditions (CTC) and software reference configurations for testing of generative face video coding. The conditions and configurations are tentatively recommended for use in applicable technical contributions to JVET meetings.

# Proposed CTC text

## Introduction

Common test conditions (CTC) are desirable to conduct experiments in a well-defined environment and facilitate the comparison of the outcome of experiments.

This document defines 2 test conditions for SDR video content and low-delay settings:

- Low delay, 8 bit

- Low delay, P slices only, 8 bit (optional)

[Ed: It is proposed to limit to low-delay currently and consider adding AI and RA in the future.]

[Ed: It is proposed to limit to SDR currently and add HDR/WCG in the future.]

The following sections define test sequences, quantization parameter values, encoder configuration files, and compile-time options to be used.

## Test sequences

Table 1 defines the set of test sequences to be used for low-delay conditions. Class A test sequences are selected from the VoxCeleb dataset [1]. Class B test sequences are selected from the CFVQA dataset [2]. All test sequences are 8-bit RGB 4:4:4 with sample width x height of 256 x 256.

Test sequences are available on and [Ed: add ftp site].

Test sequences are made available only to qualified JVET participants. Qualified participants may contact the JVET chairs for login information. Specific license information is provided with each test sequence.

[Ed: In Table 1, indicate which sequences are to be mandatory “M” or optional “O”].

Table 1. Test sequences

| Class | Sequence name | Frame count | Frame rate | Bit depth | Low-delay |
| --- | --- | --- | --- | --- | --- |
| A | VOXCELEB_001 | 250 | 25 | 8 | M |
| A | VOXCELEB_002 | 250 | 25 | 8 | M |
| A | VOXCELEB_003 | 250 | 25 | 8 | M |
| A | VOXCELEB_004 | 250 | 25 | 8 | M |
| A | VOXCELEB_005 | 250 | 25 | 8 | M |
| A | VOXCELEB_006 | 250 | 25 | 8 | M |
| A | VOXCELEB_007 | 250 | 25 | 8 | M |
| A | VOXCELEB_008 | 250 | 25 | 8 | M |
| A | VOXCELEB_009 | 250 | 25 | 8 | M |
| A | VOXCELEB_010 | 250 | 25 | 8 | M |
| A | VOXCELEB_011 | 250 | 25 | 8 | M |
| A | VOXCELEB_012 | 250 | 25 | 8 | M |
| A | VOXCELEB_013 | 250 | 25 | 8 | M |
| A | VOXCELEB_014 | 250 | 25 | 8 | M |
| B | VOXCELEB_015 | 125 | 25 | 8 | M |
| B | CFVQA_001 | 125 | 25 | 8 | M |
| B | CFVQA_002 | 125 | 25 | 8 | M |
| B | CFVQA_003 | 125 | 25 | 8 | M |
| B | CFVQA_004 | 125 | 25 | 8 | M |
| B | CFVQA_005 | 125 | 25 | 8 | M |
| B | CFVQA_006 | 125 | 25 | 8 | M |
| B | CFVQA_007 | 125 | 25 | 8 | M |
| B | CFVQA_008 | 125 | 25 | 8 | M |
| B | CFVQA_009 | 125 | 25 | 8 | M |
| B | CFVQA_010 | 125 | 25 | 8 | M |
| B | CFVQA_011 | 125 | 25 | 8 | M |
| B | CFVQA_012 | 125 | 25 | 8 | M |
| B | CFVQA_013 | 125 | 25 | 8 | M |
| B | CFVQA_014 | 125 | 25 | 8 | M |
| B | CFVQA_015 | 125 | 25 | 8 | M |

MD5 hashes and file names for each sequence are listed in Annex A. The hashes shall be used to verify that the correct version of the test sequence is used.

[Ed: MD5 hashes need to be generated and made available.]

Notes:

- When the encoder operates in 8-bit mode (InternalBitDepth=8) for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range. Similarly, when the encoder operates in 10-bit mode (InternalBitDepth=10) for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behavior is built into the reference encoder and no external conversion program is required.

- In the table above, “M” and “O” stand for mandatory and optional, respectively.

- “-” indicates that the test sequence is not requested in the JVET CTC or the given configuration.

## Quantization parameter values

For each video sequence, four quantization parameter values are to be used to generate low-delay anchor bitstreams: 37, 42, 47 and 52.

For GFVC bitstreams, the base picture is coded as intra using quantization parameter values 22, 32, 42 and 52, corresponding respectively to the four low-delay anchor bitstreams.

[Ed: Add quantization parameter values for GFVC fusion pictures.]

## Anchors

The VTM is provided as an anchor. Corresponding configuration files are provided, as described in section 2.6

## Reported Metrics

The attached SDR Excel sheets contain a reporting template in which bitrate, LPIPS (in RGB444 domain), DISTS (in RGB444 domain), RGB444_PSNR, RGB444_SSIM, YUV420_PSNR, YUV420_SSIM, and BD-rate [3] results are reported for the tested configuration against the anchor.

For 10-bit video, PSNR is calculated as 10*(log10((255 << 2) / MSE), 8-bit content is converted to 10-bit input in the encoder by shifting 2 bits to the left and 10-bit PSNR calculation is used to report testing results. This behavior is built into the reference encoder and no external conversion program is required.

In the attached Excel sheets, BD-rate is calculated using both piece-wise cubic and cubic interpolations, and the piece-wise cubic interpolation is used in the results summary.

## Configuration files

The following sections define encoder configuration files to be used for each test case. Parameters to be changed for each test point are:

- InputFile to reflect the location of the source video sequence on the test system

- FrameRate to reflect the frame rate of a given sequence as per Table 1

- SourceWidth to reflect the width of the source video sequence

- SourceHeight to reflect the height of the source video sequence

- FramesToBeEncoded to reflect the frame count of a given sequence as per Table 1

- QP to reflect the quantization parameter values defined in section 2.3.

- InputBitDepth to reflect the bit depth of a given sequence as per Table 1

The following configuration files are provided in the cfg/ folder of the VTM software package (available at [Ed: add link].

Test condition configuration files:

- “Low-delay B” (LB): encoder_lowdelay_gfvc.cfg

- “Low-delay P” (LP, optional): encoder_lowdelay_P_gfvc.cfg

## Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/CommonLib folder of the reference software. The default settings provided in the source code should be used.

## References

- Nagrani A, Chung J S, Zisserman A. Voxceleb: a large-scale speaker identification dataset[J]. arXiv preprint arXiv:1706.08612, 2017.

- Li Y, Chen B, Chen B, et al. Perceptual Quality Assessment of Face Video Compression: A Benchmark and An Effective Method[J]. arXiv preprint arXiv:2304.07056, 2023.

- Bjontegaard G. Calculation of average PSNR differences between RD-curves[J]. ITU SG16 Doc. VCEG-M33, 2001.

- Ding K, Ma K, Wang S, et al. Image quality assessment: Unifying structure and texture similarity[J]. IEEE transactions on pattern analysis and machine intelligence, 2020, 44(5): 2567-2581.

- Zhang R, Isola P, Efros A A, et al. The unreasonable effectiveness of deep features as a perceptual metric[C]//Proceedings of the IEEE conference on computer vision and pattern recognition. 2018: 586-595.

## Annex A. Test sequence checksums

For each test sequence included in the common test conditions, the MD5 hash is listed below. The list can be copied into a text file and hashes can be checked using the md5sum tool:

md5sum -c <filename>

[Ed: MD5 hashes need to be generated and made available.]

## Annex B: Objective distortion metrics for GFVC quality evaluations

The common test conditions described in this document requires a number of metrics to be reported for evaluation. In this Annex, further information about the Deep Image Structure and Texture Similarity (DISTS) [4] and Learned Perceptual Image Patch Similarity (LPIPS) [5] metrics are provided.

## B.1. PSNR

An implementation of the PSNR metric is provided in the [Ed: add link] software and shall be used for calculation of the metric.

## B.2. SSIM

An implementation of the SSIM metric is provided in the [Ed: add link] software and shall be used for calculation of the metric.

## B.3. DISTS

An implementation of the DISTS metric is provided in the [Ed: add link] software and shall be used for calculation of the metric.

An informative description of the DISTS metric follows:

[Ed: add informative description of DISTS]

## B.4. LPIPS

An implementation of the LPIPS metric is provided in the [Ed: add link] software and shall be used for calculation of the metric.

An informative description of the LPIPS metric follows:

[Ed: add informative description of LPIPS]

# Patent rights declaration(s)

Dolby Laboratories, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Alibaba and City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).