| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0194-v2 |
| --- | --- |

| Title: | Non-EE2: Reference filtering for inter-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Kirill Suverov | Tel: Email: | afilippov@ofinno.com vrufitskiy@ofinno.com |
| Source: | Ofinno LLC |  |  |

_____________________________

# Abstract

The contribution proposes an additional method of filtering (interpolated) reference samples for inter predicted blocks. The proposed technique involves filtering with spatial filters that are derived from templates of reconstructed and reference blocks. On top of ECM-11.0, the following BD-rate gain and run-time results are reportedly obtained for the proposed method {Y, U, V }: {-0.11%, -0.11%, -0.08%} and {-0.12%, 0.02%, 0.05%} for RA and LDB configurations, respectively.

# Introduction

In ECM-11.0, samples of a PU may be processed by local illumination compensation (LIC). The value of a predicted sample is obtained by calculating linear model parameters and applying these parameters to interpolated reference samples.

A flowchart of LIC process is shown in Figure 1.

Figure 1. LIC reference samples processing.

# Proposed method

When LIC flag is indicated an additional flag could be signalled to specify whether the proposed method is applied or not. When the proposed method is applied, reference samples of a block are not processed by LIC, but instead, a linear filter is applied to interpolated reference area to obtain predicted samples. Linear filter parameters are derived from template areas of current and reference blocks using spatially collocated samples as shown in Fig. 2. Linear filter parameters derivation is performed similarly to the filter parameter derivation in CCCM [1].

Figure 2. Correspondence of spatial positions of current block template samples and reference area samples in filter coefficients derivation.

Several filters with different parameter sets could be applied within the reference area. Selection of filter is performed based on classification that is performed for template areas of reference and current block

The following classification steps are performed for a sample at position (x,y):

- Calculation of differences between neighboring samples in horizontal and vertical directions (delta_x and delta_y, respectively);

- Derivation of gradient based on tangent value estimated from delta_x and delta_y (similarly, as it is performed in DIMD histogram derivation), thus obtaining magnitude and angular parameter (which is expressed as one of the intra prediction directional modes)

- Comparing the magnitude with a threshold. Position (x,y) is associated with class 0 if the magnitude is less than a threshold.

- Obtaining the absolute difference of angular parameters calculated for collocated samples of templates of reference area and current block. If the difference is less than a threshold, position (x,y) is associated with class 1.

Derivation of filter parameters is performed separately for sample positions associated with class 0 and sample positions associated with class 1.

Filter parameter selection for a sample of the reference area is performed when the magnitude of the gradient in the filtered position is greater than a threshold determined for a block. The threshold for a magnitude is obtained from the range of gradient magnitudes within the reference area.

Filter parameters are different for different color components. After reference samples of a color plane are processed by a derived filter, they are clipped to the range that depends on the bit depth specified for the color component (e.g., [0,1023] for a bit depth of 10 bits).

Reference filtering flag is signalled in AMVP mode in dependence of a LIC flag. The flowchart of indication in AMVP mode on decoder side is shown in Fig. 3. The method is not applied in AMVP-merge mode. When LIC flag is on and reference filter flag is off, samples of a reference block are processed by  LIC process. When LIC flag is on and reference filter flag is on, samples of a reference block are processed by the proposed method. When LIC flag is off neither LIC, not the proposed method is applied.

Fig. 3. Signaling mechanism for the reference filtering flag.

- The method is also applied in regular merge mode. Two cost values are calculated: C0=dSAD(1) and C1=dSAD(s). The dSAD metric is defined as follows:

dSADs=ijpi-pj-s·pi-pj

where pi,pj are neighboring template samples of a reference block; pi and pj are neighboring template samples of a reference block, s denotes an estimated value of the scale parameter that is obtained by dividing the sum of differences in the current template by the sum of differences in the reference template:

s=ΔΔ,

where Δ=ijpi-pj and Δ=ijpi-pj.

C0 is multiplied by α if the inherited LIC flag is false while C1 is multiplied by α if the inherited LIC flag is true, where α is a constant that can take values less than 1: α <1.

For merge candidates, that satisfy C0>C1 criteria, the LIC flag is set to true.

Then, histograms of gradients are calculated for the reference block template and for the current block template. The calculation of histograms of gradients is performed in a similar way as done in DIMD. The decision on whether reference filtering is applied is taken based on the difference of positions of maximums within histograms and estimation of amplitudes distribution near the maximum values as shown in Fig. 4.

Fig. 4. Histograms of Gradients.

If the difference between positions of maximums in HoGRef and HoGRec  exceeds a threshold, the reference filtering is not applied. Otherwise, if SRef / SRec > TR  or SRec / SRef > TR, a reference filtering is applied to a block.

# Simulation results

The proposed method was implemented on top of ECM-11.0 and are tested under CTC for enhanced compression tool testing [2] with Random-Access and Low-Delay B configurations. The simulation results are shown in Table 1.

Table 1. Simulation results of the proposed method in ECM-11.0

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  |  | DecVmPeak |  |
| Class A1 |  | -0.13% | 0.01% |  | -0.05% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |  | #DIV/0! |  |
| Class A2 |  | -0.11% | -0.15% |  | -0.15% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |  | #DIV/0! |  |
| Class B |  | -0.10% | -0.16% |  | -0.08% |  | #NUM! |  | #NUM! |  | #NUM! |  |  | #DIV/0! |  |
| Class C |  | -0.10% | -0.11% |  | -0.04% |  | #NUM! |  | #NUM! |  | #NUM! |  |  | #DIV/0! |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  | -0.11% | -0.11% |  | -0.08% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |  | #DIV/0! |  |
| Class D |  | -0.14% | -0.12% |  | -0.17% |  | #NUM! |  | #NUM! |  | #NUM! |  |  | #DIV/0! |  |
| Class F |  | #VALUE! | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  | #NUM! |  |  | #DIV/0! |  |
| Class TGM |  | #VALUE! | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  | #NUM! |  |  | #DIV/0! |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  |  | DecVmPeak |  |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B |  | -0.12% |  | 0.27% |  | 0.51% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |  | #DIV/0! |
| Class C |  | -0.14% |  | -0.08% |  | -0.40% |  | #NUM! |  | #NUM! |  | #NUM! |  |  | #DIV/0! |
| Class E |  | -0.08% |  | -0.27% |  | -0.11% |  | #NUM! |  | #NUM! |  | #NUM! |  |  | #DIV/0! |
| Overall |  | -0.12% |  | 0.02% |  | 0.05% |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |  | #DIV/0! |
| Class D |  | -0.12% | 0.22% |  | -0.73% |  | #NUM! |  | #NUM! |  | #NUM! |  |  | #DIV/0! |  |
| Class F |  | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |  | #DIV/0! |  |
| Class TGM |  | #VALUE! | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  | #DIV/0! |  |  | #DIV/0! |  |

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, and L. Zhang, “Algorithm description of Enhanced Compression Model 11 (ECM 11),” JVET-AF2025, October 2023.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, October 2023.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).