| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0195-v1 |
| --- | --- |

| Title: | Non-EE2: LIC model parameter inheritance for merge modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Han Huang, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA 92121 USA | Tel: Email: | chunchic@qti.qualcomm.com hanhuang@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to enable LIC (Local Illumination Compensation) model inheritance for merge modes to infer LIC model parameters from merged candidate motion with LIC enabled. When the new merge candidate is selected, the derivation process of LIC model parameters could be bypassed completely. The experimental results on top of ECM-11.0 are reported, as follows:

RA(Y/U/V): {-0.05%, 0.04%, -0.07%, EncT 99.1%, DecT 99.7%};

LDB(Y/U/V): {-0.13%, -0.15%, 0.01%, EncT 99.2%, DecT 99.6%};

# Introduction

In the merge candidate list construction process of ECM-11.0, LIC flag may be inferred from causally neighboring blocks and is set together with merge candidate motion. Then, the flag value is used to indicate whether LIC model parameters should be derived on-the-fly and applied to the motion-compensated predictors for either template or prediction block of the current block. Both the LIC flags and model parameters are stored in motion field and/or CU data. However, OBMC is the only coding tool that refers to it. Thus, the study in the proposal is to make the most of the stored LIC information by inheriting model parameters in addition to LIC flag value during the construction process of merge candidate lists. Compared with ECM-11.0 under EE2 common test conditions (JVET-AF2024), it is recorded with -0.05% and -0.13% of Y-BDBR for Random Access and Low Delay B configurations, respectively.

# Proposed method

The proposed method may duplicate merge candidates conditionally for those with LIC flag enabled and inherit the corresponding LIC model parameters in addition to merged motion information. The LIC flag derivation process (JVET-AF0128) adopted in the last meeting is bypassed for the new merge candidates. Thus, the derivation process of LIC model parameters for the new merge candidates could be bypassed completely in the motion compensation process of either template or prediction blocks. Besides, in the case of GPM merge modes, LIC model parameters from causally neighboring blocks could be inherited conditionally, when they have LIC flags determined to be true, in addition to motion information during the construction process of GPM merge candidate list. Accordingly, LIC could be enabled for GPM without the model parameter derivation in motion compensation.

# Experimental results

The proposed method was implemented on top of the ECM-11.0 software [1] and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AF2024). The testing results are summarized below.

- Performance results of the proposed method relative to ECM-11.0.

|  | Random Access Main 10 |  |  |  |  | Low Delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.12% | 0.07% | 99.3% | 98.7% | x | x | x | x | x |
| Class A2 | 0.00% | 0.19% | 0.05% | 99.0% | 100.3% | x | x | x | x | x |
| Class B | -0.07% | -0.13% | -0.23% | 99.1% | 99.6% | -0.16% | -0.10% | -0.01% | 98.9% | 98.7% |
| Class C | -0.06% | 0.07% | -0.06% | 99.0% | 100.2% | -0.04% | 0.02% | -0.31% | 99.3% | 100.3% |
| Class E | x | x | x | x | x | -0.20% | -0.45% | 0.45% | 99.6% | 100.3% |
| Overall | -0.05% | 0.04% | -0.07% | 99.1% | 99.7% | -0.13% | -0.15% | 0.01% | 99.2% | 99.6% |
| Class D | -0.04% | 0.09% | -0.11% | 99.8% | 100.3% | 0.02% | -0.73% | -0.70% | 98.3% | 98.7% |

# Reference

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).