| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0197-v2 |
| --- | --- |

| Title: | Non-EE2: Matrix based intra prediction replacing conventional intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Hongtao Wang, Vadim Seregin, Marta Karczewicz, Patrick Garus, Gleb Verba. 5775 Morehouse drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com hongtaow@qti.qualcomm.com vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In this proposal, a matrix based intra prediction replacing some directional intra modes is proposed where the weights are intra mode and block shape dependent which are applied to L shaped causal template to generate final prediction.

On top of the ECM-11.0, simulation result of the proposed method is:

AI: -0.46%, -0.42%, -0.42%, EncT: 102.2%   , DecT:  100.8%.

# Proposed method

In this method, a matrix of weights, which are defined for a block shape and intra mode, is introduced, those weights are multiplied by the neighbour reference template to derive the prediction samples replacing conventional intra prediction. The weights are applied to the reference samples of the L shaped causal neighborhood template as shown in the Fig 1.

Fig 1: L shaped neighborhood for a given predicted block

The reference samples in the causal neighborhood are denoted as r, and F(x,y) is the matrix of weights. Then the prediction P(x,y) can be derived as

P(x,y) = ∑k F(x,y,k)*r(k),

where k denotes  the index of the reference sample in the template

In the test, the template size is set to 4 (i.e., T1 = T2 = 4), and the template dimension is kept as 2*W and 2*H. The mode is only enabled for block sizes of W<=16 and H<=16, and for mode 0, 1, and (2+4*k) where k = {0, 1, 2,…., 16}, (i.e., every 4th mode). When enabled, this mode replaces the conventional intra-prediction mode, so no extra signalling is needed.

# Experimental results

The proposed method has been implemented on top of the ECM-11.0. The simulations were performed following the common test conditions [2] and summarized in the below table.

|  | All Intra Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.16% |  | -0.07% |  | -0.21% |  | 102.8% |  | 101.1% |  | 100.7% |  | 100.8% |  |
| Class A2 | -0.20% |  | -0.16% |  | -0.14% |  | 102.5% |  | 102.2% |  | 100.7% |  | 100.8% |  |
| Class B | -0.33% |  | -0.34% |  | -0.37% |  | 102.2% |  | 99.9% |  | 101.6% |  | 101.8% |  |
| Class C | -0.62% |  | -0.62% |  | -0.53% |  | 101.9% |  | 101.6% |  | 102.7% |  | 103.4% |  |
| Class E | -1.04% |  | -0.91% |  | -0.88% |  | 101.9% |  | 99.8% |  | 102.2% |  | 102.6% |  |
| Overall | -0.46% |  | -0.42% |  | -0.42% |  | 102.2% |  | 100.8% |  | 101.6% |  | 101.9% |  |
| Class D | -0.23% |  | -0.19% |  | -0.15% |  | 102.5% |  | 101.9% |  | 103.2% |  | 104.4% |  |
| Class F | -0.42% |  | -0.33% |  | -0.34% |  | 101.0% |  | 101.5% |  | 102.0% |  | 102.4% |  |
| Class TGM | #VALUE! |  | #VALUE! |  | #VALUE! |  | #NUM! |  | #NUM! |  | #NUM! |  | #NUM! |  |

# References

[1] ECM-11.0: https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

[2] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).