| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0202-v2 |
| --- | --- |

| Title: | Non-EE2: Geometry partitioning mode with inter prediction and intra block copy |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Li Zhang2 1Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents a method of geometry partitioning mode (GPM) with inter prediction and intra block copy (GPM-inter/IBC). In GPM-inter/IBC, the two sub-partitions divided geometrically are generated using inter prediction and IBC. On top of ECM-11.0, simulation results of the proposed method are reported as below:

RA: Class F {-0.03%, 0.02%, -0.07%; 100.7%, 99.9%}, Class TGM {-0.39%, -0.39%, -0.41%; 99.9%, 100.5%};

LDB: Class F{-0.21%, -0.37%, -0.12%; 101.1%, 101.5%}, Class TGM {-0.77%, -0.79%, -0.79%; 100.2%, 98.5%}.

Introduction

In current design of ECM, geometry partitioning mode (GPM) is used for inter prediction, intra prediction, and IBC. The combinations of prediction modes for two sub-partitions are supported as follows:

- GPM: Inter prediction + Inter prediction

- GPM with inter and intra prediction (GPM-Intra): Inter prediction + Intra prediction

- Spatial GPM (SGPM): Intra prediction + Intra prediction

- IBC with geometry partitioning (IBC-GPM): IBC + Intra prediction

- Bi-predictive IBC-GPM: IBC + IBC

However, the combination of inter prediction and IBC for GPM has not been supported yet.

Proposed method

In this contribution, GPM with inter prediction and intra block copy (GPM-inter/IBC) is proposed, which is missing in ECM-11.0. In GPM-inter/IBC, the two sub-partitions divided geometrically are generated using inter prediction and IBC. To generate the prediction signal of IBC, an IBC candidate list is constructed, and the index of the selected block vector is signalled.

Simulation results

The proposed method is implemented on top of ECM-11.0 [1]. Simulations are performed following the common test conditions of EE2 [2]. The simulation results of the proposed method compared to ECM-11.0 are shown in Table 1.

Table 1: Coding performance compared to ECM-11.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.03% | 0.02% | -0.07% | 100.7% | 99.9% | 100.1% | 100.0% |
| Class TGM | -0.39% | -0.39% | -0.41% | 99.9% | 100.5% | 99.5% | 100.4% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.21% | -0.37% | -0.12% | 101.1% | 101.5% | 100.6% | 99.6% |
| Class TGM | -0.77% | -0.79% | -0.79% | 100.2% | 98.5% | 100.3% | 97.0% |

Conclusions

In this contribution, geometry partitioning mode with inter prediction and intra block copy is proposed. It is recommended to study the proposed method in EE2.

Reference

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).