| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0203-v1 |
| --- | --- |

| Title: | AHG9/AHG16: Common text for proposed generative face video SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Bolin Chen,Yan Ye (Alibaba) Shanzhi Yin, Shiqi Wang (CityU) Sean McCarthy, Peng Yin, Guan-Ming Su, Anustup Kuma Choudhury, Walt Husak, Gary J. Sullivan (Dolby) | Email: | {jiechen.cj, chenbolin.chenboli, yan.ye}@alibaba-inc.com shanzhyin3-c@my.cityu.edu.hk shiqwang@cityu.edu.hk {smcca, pyin}@dolby.com |
| Source: | Alibaba, City University of Hong Kong and Dolby Laboratories, Inc. |  |  |

_____________________________

- Abstract

This contribution updates the common text of generative face video (GFV) SEI message proposed in JVET-AF0234 to 1) make the signaling of translator neural network optional by adding a gating flag; 2) support predictive signaling of matrix elements to align with the key-point coordinate signaling; 3) refine the interface between the translator neural network and generative neural network. The proposed GFV SEI message is intended for ultra-low bitrate face video compression applications such as video conferencing, live entertainment, and face animation.

Introduction

Versions of a GFV SEI message were proposed at the 29th and 30th JVET meetings in JVET-AC0088 [1] and JVET-AD0051 [2], respectively. At the 31st JVET meeting, JVET-AE0080 [3], JVET-AE0083 [4] and JVET-AE0280 [5] were proposed to support a common syntax design of generative face video SEI message and define the interface between the decoder (which outputs the base pictures and face representation parameters signalled the SEI message) and the generative neural network. During discussions, the interoperability issue between different facial representations and their associated generative networks was raised by experts. To solve it, JVET-AF0048 [6] submitted in the 32nd JVET meeting further analyzed the decoder design of generative face video coding system, and utilized a flow translator or a parameter translator to improve the decoder interoperability and parameter translatability. Based on that, JVET-AF0234[7] updated the common syntax design to support signaling of the parameter translator network and define the input/output of the translator and the interface between the translator network and generative network. In this meeting, JVET-AG0048 [8] further evaluated the rate-distortion performance of the parameter translator mechanism following the proposed test conditions in JVET-AG0042 [9]. And it was reported that different face parameters from different GFVC algorithms can be effectively adapted to one another while retaining promising coding performance. Thus, this contribution updates the common text proposed in JVET-AF0234 to make the design more mature. And the proposed GFV SEI message syntax was implemented on top of VTM-22.2 software.

Proposal

Similar with JVET-AF0234, the proposed GFV SEI message is designed to support carrying facial parameters for a variety of feature representations including 2D keypoints, 2D landmarks, 3D keypoints, facial semantics, and other formats. A facial parameter translator neural network may be signaled in the GFV SEI message to convert the signaled facial parameters into the fixed format of the parameters which can be used by the generative neural network in decoder side to generate the face pictures.

Compared with JVET-AF0234, three majors changes are made: 1) add a gating flag to make the signaling of translator network in GFV SEI message optional; 2) support predictive signaling of matrix elements to align with the key-point coordinate signaling; 3) refine the interface between the translator network and generative network. Besides, some editorial bugs were also fixed.

The updated text is provided in the following sections with changes on top of JVET-AF0234 highlighted.

Proposed SEI message scope

Table 1 - Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| … | … |
| Generative face video | The picture associated with the SEI message |

Syntax of generative face video SEI message

Table 2 - generative face video SEI message syntax

| generative_face_video ( payloadSize ) { | Descriptor |
| --- | --- |
| gfv_id | ue(v) |
| gfv_base_pic_flag /*indicate if current decoded output picture is a base picture*/ | u(1) |
| if( gfv_base_pic_flag ) { /*specify TranslatorNN( )*/ |  |
| gfv_nn_present_flag | u(1) |
| if( gfv_nn_present_flag ) { |  |
| gfv_nn_base_flag | u(1) |
| gfv_nn_mode_idc | ue(v) |
| if( gfv_nn_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_a | u(1) |
| gfv_nn_tag_uri | st(v) |
| gfv_nn_uri | st(v) |
| } |  |
| } |  |
| } else /* current decoded output picture is a driving picture*/ |  |
| gfv_drive_pic_fusion_flag /*indicate if DrivePicture is input to GenerativeNN( )*/ | ue(v) |
| gfv_coordinate_present_flag | u(1) |
| if( gfv_coordinate_present_flag ) { |  |
| gfv_coordinate_precision_factor_minus1 | ue(v) |
| gfv_num_kp_minus1 | ue(v) |
| gfv_kp_pred_flag | u(1) |
| gfv_coordinate_z_present_flag | u(1) |
| if(gfv_coordinate_z_present_flag ) |  |
| gfv_coordinate_z_max_value_minus1 | ue(v) |
| for( i = 0; i <= num_kp_minus1; i++ ) { |  |
| if(!gfv_kp_pred_flag) { |  |
| gfv_coordinate_x_abs[ i ] | u(v) |
| if( gfv_coordinate_x_abs[ i ] ) |  |
| gfv_coordinate_x_sign_flag[ i ] | u(1) |
| gfv_coordinate_y_abs[ i ] | u(v) |
| if( gfv_coordinate_y_abs[ i ] ) |  |
| gfv_coordinate_y_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_z_abs[ i ] | u(v) |
| if( gfv_coordinate_z_abs[ i ] ) |  |
| gfv_coordinate_z_sign_flag[ i ] | u(1) |
| } |  |
| } else { |  |
| gfv_coordinate_dx_abs[ i ] | u(v) |
| if(gfv_coordinate_dx_abs[ i ] ) |  |
| gfv_coordinate_dx_sign_flag[ i ] | u(1) |
| gfv_coordinate_dy_abs[ i ] | u(v) |
| if( gfv_coordinate_dy_abs[ i ] ) |  |
| gfv_coordinate_dy_sign_flag[ i ] | u(1) |
| if( gfv_coordinate_z_present_flag ) { |  |
| gfv_coordinate_dz_abs[ i ] | u(v) |
| if( gfv_coordinate_dz_abs[ i ] ) |  |
| gfv_coordinate_dz_sign_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| gfv_matrix_present_flag | u(1) |
| if(gfv_matrix_present_flag ) { |  |
| gfv_matrix_element_precision_factor_minus1 | ue(v) |
| gfv_num_matrix_types_minus1 | ue(v) |
| if( !gfv_base_pic_flag ) |  |
| gfv_matrix_pred_flag | u(1) |
| for( i = 0; i <= num_matrix_types_minus1; i++ ) { |  |
| gfv_matrix_type_idx[ i ] | u(6) |
| if( gfv_matrix_type_idx[ i ] = = 0 ¦ ¦ gfv_matrix_type_idx[ i ] = = 1 ) { |  |
| if( gfv_coordinate_present_flag ) |  |
| gfv_num_matrices_equal_to_num_kps_flag[ i ] | u(1) |
| if( !gfv_coordinate_present_flag ¦ ¦ !gfv_num_matrix_equal_to_num_kps_flag[ i ] ) |  |
| gfv_num_matrices_info[ i ] | ue(v) |
| }else if( gfv_matrix_type_idx[ i ] = = 2 ¦ ¦ gfv_matrix_type_idx[ i ] = = 3 ¦ ¦ gfv_matrix_type_idx[ i ] >= 7 ){ |  |
| if( gfv_matrix_type_idx[ i ] >= 7 ) |  |
| gfv_num_matrices_minus1[ i ] | ue(v) |
| gfv_matrix_width_minus1[ i ] | ue(v) |
| gfv_matrix_height_minus1[ i ] | ue(v) |
| }else if( gfv_matrix_type_idx[ i ] >= 4 && gfv_matrix_type_idx[ i ] <= 6 && !gfv_coordinate_present_flag ){ |  |
| gfv_matrix_for_3D_space_flag[ i ] | u(1) |
| for( j = 0; j < numMatrices[ i ]; j++ ) |  |
| for( k = 0; k < matrixHeight[ i ]; k++ ) |  |
| for( m = 0; m <matrixWidth[ i ]; m++ ) { |  |
| if( !gfv_matrix_pred_flag ) { |  |
| gfv_matrix_element_int[ i ][ j ][ k ][ m ] | ue(v) |
| gfv_matrix_element_dec[ i ][ j ][ k ][ m ] | u (v) |
| if( gfv_matrix_element_int[ i][ j ][ k ][ m ] ¦ ¦ gfv_matrix_element_dec[ i ][ j ][ k ][ m ] ) |  |
| gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ] | u(1) |
| } |  |
| else { |  |
| gfv_matrix_delta_element_int[ i ][ j ][ k ][ m ] | ue(v) |
| gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] | u (v) |
| if( gfv_matrix_element_int[ i][ j ][ k ][ m ] ¦ ¦ gfv_matrix_element_dec[ i ][ j ][ k ][ m ] ) |  |
| gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| if( gfv_nn_present_flag ) |  |
| if( gfv_nn_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| gfv_nn_reserved_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| gfv_nn_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

Semantics of generative face video SEI message

The generative face video (GFV) SEI message indicates facial parameters and specifies a facial parameter translator network, denoted as TranslatorNN( )), that may be used to convert various formats of facial parameters signaled in the SEI message into a fixed format of parameters, and a face picture generator neural network, denoted as GenerativeNN( ), that may be used to generate output pictures using the fixed format of facial parameters and previously decoded output pictures.

NOTE 1 – Facial parameters could be determined from source pictures prior to encoding. Such source pictures may be referred to as driving pictures.

NOTE 2 – Previously decoded output pictures input to GenerativeNN( ) may be a base picture (a decoded output picture that provides the reference texture from which the face pictures may be generated) and, optionally, a picture that can be fused by GenerativeNN( ) to improve background texture and facial details. When the current picture is not a base picture, the GFV SEI message may be used to generate a face picture based on the previously decoded base picture, the facial parameters conveyed by the GFV SEI message, and, optionally, the current decoded picture for fusion pupose.

Use of this SEI message requires the definition of the following variables:

- Input picture width and height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

- Luma sample array baseCroppedYPic and chroma sample arrays baseCroppedCbPic and baseCroppedCrPic for a decoded output picture, denoted as BasePicture, corresponding to a source base picture.

- Luma sample array driveCroppedYPic and chroma sample arrays driveCroppedCbPic and driveCroppedCrPic for a decoded output picture, denoted as DrivePicture, corresponding to a source driving picture.

- Bit depth BitDepthY for the luma sample array of the input pictures.

- Bit depth BitDepthC for the chroma sample arrays, if any, of the input pictures.

- A chroma format indicator, denoted herein by ChromaFormatIdc, as described in subclause ‎7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

gfv_id contains an identifying number that may be used to identify face feature information and specify a neural network that may be used as GenerativeNN( ). The value of gfv_id shall be in the range of 0 to 232–− 2, inclusive. Values of gfv_id from 256 to 511, inclusive, and from 231 to 232–− 2, inclusive, are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this edition of this document encountering a GFV SEI message with gfv_id in the range of 256 to 511, inclusive, or in the range of 231 to 232–− 2, inclusive, shall ignore the SEI message.

NOTE – Different values of gfv_id in different GFV SEI messages could be used to identify different faces when more than one face is present in an output picture, for example.

gfv_base_pic_flag equal to 1 indicates the current decoded output picture corresponds to a base picture. gfv_base_pic_flag equal to 0 indicates the current decoded output picture does not correspond to a base picture.

The following constraints apply to the value of gfv_base_pic_flag:

–	When a GFV SEI message is the first GFV SEI message, in decoding order, that has a particular gfv_id value within the current CLVS, the value of gfv_base_pic_flag shall be equal to 1.

–	When a GFV SEI message that has a particular gfv_id value has gfv_base_pic_flag being equal to 0, this SEI message pertains to the current decoded picture and all subsequent decoded pictures of the current layer, in output order, until the end of the current CLVS or up to but excluding the decoded picture that follows the current decoded picture in output order within the current CLVS and is associated with a subsequent GFV SEI message, in decoding order, having gfv_base_pic_flag equal to 0 and that particular gfv_id value within the current CLVS, whichever is earlier.

gfv_nn_present_flag equal to 1 indicates a neural network that may be used as a TranslatorNN( ) is contained or indicated by the SEI message. gfv_nn_present_flag equal to 0 indicates a neural network that may be used as a TranslatorNN( ) is not contained or indicated by the SEI message. When gfv_nn_present_flag is not present, it is inferred to be 0.

gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] specify a neural network that may be used as a TranslatorNN( ). gfv_nn_base_flag, gfv_nn_mode_idc, gfv_nn_reserved_zero_bit_a, gfv_nn_tag_uri, gfv_nn_uri, gfv_nn_payload_byte[ i ] have the same syntax and semantics as nnpfc_base_flag, nnpfc_mode_idc, nnpfc_reserved_zero_bit_a, nnpfc_tag_uri, nnpfc_uri, nnpfc_payload_byte[ i ], respectively.

gfv_drive_pic_fusion_flag, when present, equal to 1 indicates the current decoded picture, which corresponds to a driving picture that may be used for fusion, may be input to GenerativeNN( ). gfv_drive_pic_fusion_flag equal to 0 indicates the current decoded picture should not be input to GenerativeNN( ).

NOTE 3 – A gfv_drive_pic_fusion_flag value of 1 can be used, for example, to indicate that the current decoded picture can be used to improve face details or handle background changes.

NOTE 4 – Fusion takes the three inputs: the base picture, features from keypoints and/or matrices carried in the GFV SEI message, and the current decoded picture, and outputs a picture.

NOTE 5 – When current decoded picture corresponds to a driving picture, it should be marked as not for output purpose.

gfv_coordinate_present_flag equal to 1 indicates that coordinate information of keypoints is present. gfv_coordinate_present_flag equal to 0 indicates that coordinate information of keypoints is not present.

It is a requirement of bitstream conformance that when gfv_matrix_type_idx[ i ] for any i from 0 to gfv_num_matrix_types_minus1 is equal to 0 or 1, the value of gfv_coordinate_present_flag shall be equal to 1.

gfv_coordinate_precision_factor_minus1 plus 1 indicates the length, in bits, of gfv_coordinate_x_abs[i], gfv_coordinate_y_abs[i] and gfv_coordinate_z_abs[i].

gfv_num_kps_minus1 plus 1 indicates the number of keypoints. The value of gfv_num_kp_minus1 shall be in the range of 0 to 210 – 1, inclusive.

gfv_kp_pred_flag equal to 1 indicates syntax elements gfv_coordinate_dx_abs[ i ] ,gfv_coordinate_dy_abs[ i ], and gfv_coordinate_dz_abs[ i ] are present and syntax elements gfv_coordinate_dx_sign_flag[ i ], gfv_coordinate_dy_sign_flag[ i ] and gfv_coordinate_dz_sign_flag[ i ] may be present. gfv_kp_pred_flag equal to 0 indicates gfv_coordinate_x_abs[ i ] ,gfv_coordinate_y_abs[ i ], and gfv_coordinate_z_abs[ i ] are present and syntax elements gfv_coordinate_x_sign_flag[ i ], gfv_coordinate_y_sign_flag[ i ] and gfv_coordinate_z_sign_flag[ i ] may be present.

gfv_coordinate_z_present_flag equal to 1 indicates that z-axis coordinate information of the keypoints is present. coordinate_z_present_flag equal to 0 indicates that the z-axis coordinate information of the keypoints is not present.

gfv_coordinate_z_max_value_minus1 plus 1 indicates the maximum absolute value of z-axis coordinates of keypoints.

gfv_coordinate_x_abs[ i ] indicates the normalized absolute value of the x-axis coordinate of the i-th keypoint.

gfv_coordinate_x_sign_flag[ i ] specifies the sign of the x-axis coordinate of the i-th keypoint. When gfv_coordinate_x_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_y_abs[ i ] specifies the normalized absolute value of y-axis coordinate of i-th keypoint.

gfv_coordinate_y_sign_flag[ i ] specifies the sign of the y-axis coordinate of the i-th keypoint. When gfv_coordinate_y_sign_flag[i] is not present, it is inferred to be equal to 0.

gfv_coordinate_z_abs[ i ] specifies the normalized absolute value of z-axis coordinate of the i-th keypoint.

gfv_coordinate_z_sign_flag[ i ] specifies the sign of the z-axis coordinate of the i-th key point. When gfv_coordinate_z_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_dx_abs[ i ] indicates the absolute difference value of the normalized value of the x-axis coordinate of the i-th keypoint.

gfv_coordinate_dx_sign_flag[ i ] specifies the sign of the difference value of the x-axis coordinate of the i-th keypoint. When gfv_coordinate_dx_sign_flag[ i ] is not present, it is inferred to be equal to 0.

gfv_coordinate_dy_abs[ i ] specifies the absolute difference value of the normalized y-axis coordinate of the i-th keypoint.

gfv_coordinate_dy_sign_flag[ i ] specifies the sign of the difference value of the y-axis coordinate of the i-th keypoint. When gfv_coordinate_yd_sign_flag[i] is not present, it is inferred to be equal to 0.

gfv_coordinate_dz_abs[ i ] specifies the absolute difference value of the normalized z-axis coordinate of the i-th keypoint.

gfv_coordinate_dz_sign_flag[ i ] specifies the sign of the difference value of the z-axis coordinate of the i-th key point. When gfv_coordinate_dz_sign_flag[ i ] is not present, it is inferred to be equal to 0.

The variables coordinateDeltaX[ i ], coordinateDeltaY[ i ] and coordinateDeltaZ[ i ] indicating the delta x-axis coordinate, delta y-axis coordinate and delta z-axis oordinate of the i-th keypoint, respectively, are derived as follows:

coordinateDeltaX[ i ] = (1-2 * gfv_coordinate_dx_sign_flag[ i ]) * gfv_coordinate_dx_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaY[i] = (1-2 * gfv_coordinate_dy_sign_flag[ i ]) * gfv_coordinate_dy_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateDeltaZ[i] = (1-2 * coordinate_dz_sign_flag[ i ]) * gfv_coordinate_dz_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

The variables coordinateX[ i ], coordinateY[ i ] and coordinateZ[ i ] indicating the x-axis coordinate, y-axis coordinate and z-axis coordinate of the i-th keypoint, respectively, are derived as follows:

When gfv_kp_pred_flag is equal to 0,

coordinateX[ i ] = (1-2 * gfv_coordinate_x_sign_flag[ i ]) * gfv_coordinate_x_abs[ i ] * CroppedWidth1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateY[i] = (1-2 * gfv_coordinate_y_sign_flag[ i ]) * gfv_coordinate_y_abs[ i ] * CroppedHeight1<< (gfv_coordinate_precision_factor_minus1 +1)

coordinateZ[i] = (1-2 * gfv_coordinate_z_sign_flag[ i ]) * gfv_coordinate_z_abs[ i ] *(gfv_coordinate_z_max_value_minus1+1)1<< (gfv_coordinate_precision_factor_minus1 +1)

when gfv_kp_pred_flag is equal to 1,

if( gfv_base_pic_flag ) {
	coordinateX[ i ] = (( i > 0 ) ? coordinateX[ i - 1 ] : 0 ) + coordinateDeltaX[ i ]
	coordinateY[ i ] = (( i > 0 ) ? coordinateY[ i - 1 ] : 0 ) + coordinateDeltaY[ i ]
	coordinateZ[ i ] = (( i > 0 ) ? coordinateZ[ i - 1 ] : 0 ) + coordinateDeltaZ[ i ]
}
else {
	coordinateX[ i ] = BaseKpCoordinateX[ i ] + coordinateDeltaX[ i ]
	coordinateY[ i ] = BaseKpCoordinateY[ i ] + coordinateDeltaY[ i ]
	coordinateZ[ i ] = BaseKpCoordinateZ[ i ] + coordinateDeltaZ[ i ]
}

where BaseKpCoordinateX[ i ], BaseKpCoordinateY[ i ], BaseKpCoordinateZ[ i ] indicating the x-axis, y-axis and z-axis coordinates, respectively, of the i-th keypoint for the base picture are derived as follows:

if( gfv_base_pic_flag ) {
	BaseKpCoordinateX[ i ] = coordinateX[ i ]
	BaseKpCoordinateY[ i ] = coordinateY[ i ]
	BaseKpCoordinateZ[ i ] = coordinateZ[ i ]
}

gfv_matrix_present_flag equal to 1 indicates that matrix parameters are present. gfv_matrix_present_flag equal to 0 indicates that matrix parameters are not present.

gfv_matrix_element_precision_factor_minus1 plus 1 indicates the length, in bits, of gfv_matrix_element_dec[ i ][ j ][ k ][ m ].

gfv_num_matrix_types_minus1 plus 1 indicates the number of matrix types signalled in the SEI message. The value of gfv_matrix_type_num_minus1 shall be in the range of 0 to 26 – 1, inclusive.

gfv_matrix_pred_flag equal to 1 indicates syntax elements gfv_matrix_element_int [ i ][ j ][ k ][ m ] , gfv_matrix_element_dec [ i ][ j ][ k ][ m ] gfv_matrix_element_sign_flag [ i ][ j ][ k ][ m ] may be present. gfv_matrix_pred_flag equal to 0 indicates gfv_matrix_delta_element_int [ i ][ j ][ k ][ m ] , gfv_matrix_delta_element_dec [ i ][ j ][ k ][ m ] are present and syntax element gfv_matrix_delta_element_sign_flag [ i ][ j ][ k ][ m ] may be present. When gfv_matrix_pred_flag is not present, it is inferred to be 0.

gfv_matrix_type_idx[ i ] indicates the index of the i-th matrix type as specified in Table 3.

Table 3 - Specification of gfv_matrix_type_idx

| Value | Specification |
| --- | --- |
| 0 | Affine translation matrix with the size of 2*2 or 3*3. |
| 1 | Covariance matrix with size of 2*2 or 3*3. |
| 2 | Mouth matrix representing mouth motion. |
| 3 | Eye matrix representing the open-close status and level of eyes. |
| 4 | Head rotation paramters with the size of 2*2 or 3*3 representing the head rotation in 2D space or 3D space. |
| 5 | Head translation matrix with the size of 1*2 or 1*3 representing head translationin 2D space or 3D space. |
| 6 | Head location matrix with size of 1*2 or 1*3 representing the head location in 2D space or 3D space. |
| 7 | Compact feature matrix with the size being specified by gfv_matrix_width_minus1[i] and gfv_matrix_height_minus1[i]. |
| 8…31 | Other matrix that may be used as determined by the application with the size being specified by gfv_matrix_width_minus1[i] and gfv_matrix_height_minus1[i]. |
| 32…63 | Reserved |

NOTE 2 . The undefined matrxi type is used to represent the matrxi type rather than affine translation matrix, covariance matrix, rotation matrix, translation matrix and compact feature matrix. It is may be used by the user to extend the matrix type.

gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 1 indicates that the number of matrices of the i-th matrix type is equal to gfv_num_kps_minus1 + 1. gfv_num_matrices_equal_to_num_kps_flag[ i ] equal to 0 indicates the number of matrices of the i-th matrix type is not equal to gfv_num_coordinates_minus1 + 1.

gfv_num_matrices_info[ i ] provides information to derive the number of the matrices of the i-th matrix type.

gfv_matrix_width_minus1[ i ] plus 1 indicates the width of the matrix of the i-th matrix type.

gfv_matrix_height_minus1[ i ] plus 1 indicates the height of the matrix of the i-th matrix type.

gfv_matrix_for_3D_space_flag[ i ] equal to 1 indicates the matrix of the i-th matrix type is a matrix defined in three-dimensional space. gfv_matrix_for_3D_space_flag[ i ] equal to 0 indicates the matrix of the i-th matrix type is a matrix defined in two-dimensional space.

When gfv_matrix_width_minus1[ i ] is not present, it is inferred as follows:

––	If gfv_matrix_type_idx[ i ] is equal to 0, 1 or 4, and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, gfv_matrix_width_minus1[i] is inferred to be equal to 2

––	otherwise, if matrix_type_idx[ i ] is equal to 0, 1 or 4, and one of coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 0, gfv_matrix_width_minus1[ i ] is inferred to be equal to 1

––	otherwise (matrix_type_idx[ i ] is equal to 5 or 6), gfv_matrix_width_minus1[ i ] is inferred to be equal to 0

When gfv_matrix_height_minus1[ i ] is not present, it is inferred as follows:

––	If matrix_type_idx is equal to 0, 1, 4, 5 or 6, and one of gfv_coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is present and equal to 1, gfv_matrix_height_minus1[ i ] is inferred to be equal to 2.

––	otherwise (gfv_matrix_type_idx is equal to 0, 1, 4, 5 or 6, and one of gfv_coordinate_z_present_flag and gfv_matrix_for_3D_space_flag[ i ] is 0), gfv_matrix_height _minus1[ i ] is inferred to be equal to 1.

The variables matrixWidth[ i ] and matrixHeight[ i ] indicating the width and height of the matrix of the i-th matrix type are derived as follows

matrixWidth[ i ] = gfv_matrix_width_minus1[ i ] + 1

matrixHeight[ i ] = gfv_matrix_height_minus1[ i ] + 1

gfv_num_matrices_minus1[ i ] plus 1 indicates the number of matrices of the i-th matrix type.

The variable numMatrices[ i ] indicating the number of the matrices of the i-th matrix type is derived as follows:

if( gfv_matrix_type_idx[ i ] == 0 || gfv_matrix_type_idx[ i ] == 1 ) {

if( gfv_coordinate_present_flag )

numMatrices[ i ] = gfv_num_matrices_equal_to_num_kps_flag[ i ] ? gfv_num_kps_minus1 + 1 : ( gfv_num_matrices_info[ i ] < gfv_num_kp_minus1 ? gfv_num_matrices_info [ i ] + 1 : gfv_num_matrices_info [ i ] + 2 )

else

numMatrices[ i ] = gfv_num_matrices_info[ i ] + 1

}

else if( gfv_matrix_type_idx[ i ] >= 2 &&  gfv_matrix_type_idx[ i ] < 7)

numMatrices[ i ] = 1

else

numMatrices[ i ] = gfv_num_matrices_minus1[ i ] + 1

gfv_matrix_element_int[ i ][ j ][ k ][ m ] indicates the integer part of the value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type.

gfv_matrix_element_dec[ i ][ j ][ k ][ m ] indicates the decimal part of the value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type.

gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ] indicates the sign of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type. When gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ]is not present, it is inferred to be equal to 0.

gfv_matrix_delta_element_int[ i ][ j ][ k ][ m ] indicates the integer part of the difference value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type.

gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ] indicates the decimal part of the difference value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type.

gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ] indicates the sign of the difference value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type. When gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ]is not present, it is inferred to be equal to 0.

The variable matrixElementDeltaVal[ i ][ j][ k ][ m ] representing the difference value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type is derived as follows:

matrixElementDeltaVal[ i][ j ][ k ][ m ] = 1-2*gfv_matrix_delta_element_sign_flag[ i ][ j ][ k ][ m ]*(gfv_matrix_delta_element_int i  j  k  m  +gfv_matrix_delta_element_dec[ i ][ j ][ k ][ m ]1<< (gfv_matrix_element_precision_factor_minus1 +1))

The variable matrixElementVal[ i ][ j][ k ][ m ] representing the value of the matrix element at position (m, k) of the j-th matrix of the i-th matrix type is derived as follows:

when gfv_matrix_pred_flag is equal to 0

matrixElementVal[ i][ j ][ k ][ m ] = 1-2*gfv_matrix_element_sign_flag[ i ][ j ][ k ][ m ]*(gfv_matrix_element_int i  j  k  m  +gfv_matrix_element_dec[ i ][ j ][ k ][ m ]1<< (gfv_matrix_element_precision_factor_minus1 +1)) 
if( gfv_base_pic_flag ) 
	BaseMatrixElementVal[ i][ j ][ k ][ m ] = matrixElementVal[ i][ j ][ k ][ m ]

when gfv_matrix_pred_flag is equal to 1

matrixElementVal[ i][ j ][ k ][ m ] = BaseMatrixElementVal[ i][ j ][ k ][ m ] + matrixElementDeltaVal[ i][ j ][ k ][ m ] +

The following process is used to generate a video picture:

DeriveSigParam( ) 
TranslatorNN (sigKeyPoint , sigMatrix) 
DeriveInputTensors( ) 
if( gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 0) {
  if(ChromaFormatIdc == 0 ) 
    GenerativeNN( inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    GenerativeNN( inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
}
else if(gfv_base_pic_flag == 0 && gfv_drive_pic_fusion_flag == 1) {
  if(ChromaFormatIdc == 0 ) 
    GenerativeNN( inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix) 
  else 
    GenerativeNN( inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix) 
}

StoreOutputTensors( )

The process DeriveSigParam () for deriving the inputs of TranslatorNN () is specified as follows:

The keypoint coordinate array sigKeyPoint and the matrix sigMatrix are derived as follows:

if( gfv_coordinate_present_flag ) {
  for ( i = 0; i< = gfv_num_kps_minus1; i++ ) {
    sigKeyPoint[ i ][ 0 ] = coordinateX[ i ]
    sigKeyPoint[ i ][ 1 ] = coordinateY[ i ]
    if ( gfv_coordinate_z_present_flag ) 
      sigKeyPoint[ i ][ 2 ] = coordinateZ[ i ]
  }
}
else {
  for ( i = 0; i < =num_kps_minus1; i++ ) {
    sigKeyPoint [ i ][ 0 ] = 0
    sigKeyPoint [ i ][ 1 ] = 0
    if ( gfv_coordinate_z_present_flag ) 
      sigKeyPoint [ i ][ 2 ] = 0
  }
}
if( gfv_matrix_present_flag ) {
  for ( i = 0; i <= gfv_num_matrix_types_minus1; i++ ) {
    for ( j = 0; j < numMatrices[ i ]; j++ ) {
      for( k = 0; k < matrixHeight [ i ]; k++ ) {
        for ( l = 0;l < matrixWidth [ i ]; l++) {
          sigMatrix[ i ][ j ][ k ][ l ] = matrixElementVal[ i ][ j][ k][ l ] 
        }
      }
    }
  }
}
else {
  for ( i = 0; i <= gfv_num_matrix_types_minus1; i++ ) {
    for ( j = 0; j < numMatrices[ i ]; j++ ) {
      for( k = 0; k < matrixHeight [ i ]; k++ ) {
        for ( l = 0;l < matrixWidth [ i ]; l++) {
          sigMatrix [ i ][ j ][ k ][ l ] = 0
        }
      }
    }
  }
}

TranslatorNN( ) is a process to translate the various formats of the facial parameters carried in the SEI message to the fixed format of the facial parameters to be input to the generative network to generate the output picture

Inputs to TranslatorNN() are:

- sigKeyPoint and sigMatrix

Outputs of TranslatorNN() are:

- convKeyPoint and convKeyPointNum

- convMatrix and convMatrixNum, convMatrixWidth, convMatrixHeight

The process DeriveInputTensors( ) for deriving the inputs of GenerativeNN ( ) is specified as follows:

When gfv_base_pic_flag is equal to 1, the BasePicture input tensor inputBaseY, inputBaseCb and inputBaseCr are derived as follows:

for( x = 0; x < CroppedWidth; x++ ) {
  for ( y = 0; y < CroppedHeight; y++ ) {
    inputBaseY[ x ][ y ] = InpY( baseCroppedYPic[ x ][ y ] ) 
  }
}
if (ChromaFormatIdc !=0) {
  for( x = 0; x < CroppedWidth/ SubWidthC; x++ ) {
    for ( y = 0; y < CroppedHeight/ SubHeightC; y++ ) {
      inputBaseCb[ x][ y ] = InpC( baseCroppedCbPic[ x][ y ] )
      inputBaseCr[ x][ y ] = InpC( baseCroppedCrPic[ x ][ y ] )
    }
  }
}

When gfv_drive_pic_fusion_flag is equal to 1, the DrivePicture luma sample array inputDriveY, inputDriveCb and input DriveCr are derived as follows:

for( x = 0; x< CroppedWidth; x++ ) {
  for ( y = 0; y< CroppedHeight; y++ ) {
    inputDriveY[ x ][ y ] = InpY( driveCroppedYPic[ x ][ y ] )
  }
}
if (ChromaFormatIdc !=0) {
  for( x = 0; x< CroppedWidth/ SubWidthC; x++ ) {
    for ( y = 0; y < CroppedHeight/ SubHeightC; y++ ) {
      InputDriveCb[ x][ y ] = InpC( driveCroppedCbPic[ x][ y ] )
      InputDriveCr[ x][ y ] = InpC( driveCroppedCrPic[ x ][ y ] ) 
    }
  }
}

When gfv_base_pic_flag is equal to 0, the keypoint coordinate array inputDriveKeyPoint and the matrix inputDriveMatrix for the current picture are derived as follows:

for ( i = 0; i < = 14 convKeyPointNum; i++ ) {
    inputDriveKeyPoint[ i ][ 0 ] = convKeyPoint[ i ][ 0 ]
    inputDriveKeyPoint [ i ][ 1 ] = convKeyPoint[ i ][ 0 ]
    inputDriveKeyPoint [ i ][ 2 ] = convKeyPoint[ i ][ 0 ]
  }

for( j = 0; j < convMatrixNum; j++ ) {
  for( k=0; k< 3 convMatrixHeight; k++ ) {
    for ( m=0;m< 3 convMatrixWidth; m++) {
      inputDriveMatrix[ 0 ][ k ][ m ] = convMatrix [ 0 ][ k ][ m ] 
    }
  }
}

for( k=0; k< 1; k++ ) {
  for ( l=0;l< 3; l++) {
    inputDriveMatrix[ 1 ][ k ][ l ] = convMatrix [ 1 ][ k ][ l ] 
  }
}

When gfv_base_pic_flag is equal to 1, the keypoint coordinate array inputBaseKeyPoint and the matrix inputBaseMatrix for the base picture are derived as follows:

for ( i = 0; i < = 14 convKeyPointNum; i++ ) {
    inputBaseKeyPoint[ i ][ 0 ] = convKeyPoint[ i ][ 0 ]
    inputBaseKeyPoint [ i ][ 1 ] = convKeyPoint[ i ][ 0 ]
    inputBaseKeyPoint [ i ][ 2 ] = convKeyPoint[ i ][ 0 ]
  }
for( j = 0; j < convMatrixNum; j++) {
  for( k=0; k< 3; k++ ) {
    for ( l=0;l< 3; l++) {
      inputBaseMatrix[ 0 ][ k ][ l ] = convMatrix [ 0 ][ k ][ l ] 
    }
  }
}
for( k=0; k< 1; k++ ) {
  for ( l=0;l< 3; l++) {
    inputBaseMatrix[ 1 ][ k ][ l ] = convMatrix [ 1 ][ k ][ l ] 
  }
}

where the functions InpY( ) and InpC( ) are specified as follows:

InpY( x ) = x ÷ ( ( 1  <<  BitDepthY ) – 1 )

InpC( x ) = x ÷ ( ( 1  <<  BitDepthC ) – 1 )

GenerativeNN ( ) is a process to generate the sample values of an output picture corresponding to a driving picture. It is only invoked when gfc_base_pic_flag is equal to 0. Input value to GenerativeNN( ) and output values from GenerativeNN( ) is real numbers.

Inputs to GenerativeNN() are:

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaFormatIdc is equal to 0: inputBaseY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 0 and ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaFormatIdc is equal to 0: inputBaseY, inputDriveY, inputBaseKeyPoint, inputBaseMatrix, inputDriveKeyPoint, inputDriveMatrix

- When gfv_base_pic_flag is equal to 0 and gfv_drive_pic_fusion_flag is equal to 1 and ChromaFormatIdc is not equal to 0: inputBaseY, inputBaseCb, inputBaseCr, inputDriveY, inputDriveCb, inputDriveCr , inputBaseKeyPoint, inputBaseMatrix,, inputDriveKeyPoint, inputDriveMatrix

Outputs of GenerativeNN( ) are:

- A luma sample array genY

- When ChromaFormatIdc is not equal to 0, two chroma sample arrays genCb and genCr.

The process StoreOutputTensors( ) for deriving the output is specified as follows:

when gfv_base_pic_flag is equal to 0, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = OutY( genY[ x ][ y ] )
  }
}

if(ChromaFormatIdc != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = OutC( genCb[ x ][ y ] )
      outputCrPic[ x][ y ] = OutC( genCr[ x ][ y ] )
    }
  }
}

when gfv_base_pic_flag is equal to 1, the output sample array outYPic[ x ][ y ], outCbPic[ x ][ y ], and outCrPic[ x ][ y ] are derived as follows:

for(x=0; x< CroppedWidth; x++){
  for(y=0; y< CroppedHeight; y++){
    outputYPic[ x ][ y ] = baseCroppedYPic [ x ][ y ] 
  }
}

if(ChromaFormatIdc != 0) {
  for(x=0; x< CroppedWidth/ SubWidthC; x++){
    for(y=0; y< CroppedHeight/ SubHeightC; y++){
      outputCbPic[ x ][ y ] = baseCroppedCbPic [ x ][ y ] 
      outputCrPic[ x][ y ] = baseCroppedCbPic [ x ][ y ] 
    }
  }
}

where the functions OutY( ) and OutC( ) are specified as follows:

OutY( x ) = Clip3( 0, ( 1  <<  BitDepthY ) – 1 , x * ( ( 1  <<  BitDepthY ) – 1 ) 
OutC( x ) = Clip3( 0, ( 1  <<  BitDepthC ) – 1 , x * ( ( 1  <<  BitDepthC ) – 1 )

Conclusion

The contribution follows up JVET-AF0234 in which a parameter translator is introduced to translate various formats of facial parameters signaled in GFV SEI message into a fixed format of parameters supported by the generative neural network in the decoder side to resolve the interoperability issue. And the SEI message itself was implemented on top of VTM-22.2. It is recommended to include the proposed GFV SEI message into the next draft of VSEI TuC.

References

- Chen B, Chen J, et al. “AHG9: Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AC0088, January 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message for Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AD0051, April 2023.

- McCarthy S, et al. “AHG9: Generative Face Video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0080, July 2023.

- Chen B, Chen J, et al. “AHG9: Common SEI Message of Generative Face Video,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0083, July 2023.

- Chen B, Chen J, et al. “AHG9: Common Text for Generative Face Video SEI Message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AE0280, July 2023.

- Chen B, Chen J, et al. “A Study on Decoder Interoperability of Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AF0048, October 2023.

- Chen B, Chen J. et al, “AHG9: Common Text for Proposed Generative Face Video SEI message,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AF0234, October 2023.

- Yin. S, Chen B. et al, “AHG16: Interoperability Study on Parameter Translator of Generative Face Video Coding,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG0048, January 2023.

- Chen B, Chen J. et al, “AHG 16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression,” The Joint Video Experts Team of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, doc. no. JVET-AG0042, January 2023

- Feng D, Huang Y, Zhang Y, et al. A generative compression framework for low bandwidth video conference[C]//2021 IEEE International Conference on Multimedia & Expo Workshops (ICMEW). IEEE, 2021: 1-6.

- Siarohin A, Lathuilière S, Tulyakov S, et al. First order motion model for image animation[J]. Advances in Neural Information Processing Systems, 2019, 32.

- Siarohin A, Woodford O J, Ren J, et al. Motion representations for articulated animation[C]//Proceedings of the IEEE/CVF Conference on Computer Vision and Pattern Recognition. 2021: 13653-13662.

- Wang T C, Mallya A, Liu M Y. One-shot free-view neural talking-head synthesis for video conferencing[C]//Proceedings of the IEEE/CVF conference on computer vision and pattern recognition. 2021: 10039-10049.

- Chen B, Wang Z, Li B, et al. Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression[C]//2022 Data Compression Conference (DCC). IEEE, 2022: 13-22.

- Chen B, Wang Z, Li B, et al. Interactive Face Video Coding: A Generative Compression Framework[J]. arXiv preprint arXiv:2302.09919, 2023.

Patent rights declaration(s)

Alibaba may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

City University of Hong Kong may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).