| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0245-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0152 (EE2-1.3: SGPM with IntraTMP and IBC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Ivan Zupancic | Email: | ivan.zupancic@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of EE2 test 1.3 reported in JVET-AG0152 (EE2-1.3: SGPM with IntraTMP and IBC). It proposes further including block-vector based prediction obtained from IntraTMP or IBC modes of the neighboring blocks in the candidate list of SGPM.

The crosscheck results match the results of EE2 test 1.3 reported in JVET-AG0152.

# Introduction

Cross-check of the source code and full simulation runs have been conducted. Inspected source code follows the description in the proposal.

# Crosscheck results

The code of EE2 test 1.3 was run according to the CTC [1] for AI and RA configuration. The obtained crosscheck results are as follows:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | -0.14% | 0.00% | 99.2% | 99.8% | 100.0% | 100.1% |
| Class A2 | -0.08% | 0.01% | 0.03% | 99.2% | 100.6% | 100.1% | 100.1% |
| Class B | -0.04% | 0.03% | -0.06% | 99.7% | 100.6% | 100.0% | 99.9% |
| Class C | -0.04% | -0.10% | 0.03% | 99.5% | 100.1% | 100.0% | 100.0% |
| Class E | -0.09% | 0.00% | 0.03% | 99.0% | 100.9% | 100.0% | 100.0% |
| Overall | -0.05% | -0.03% | 0.00% | 99.4% | 100.4% | 100.0% | 100.0% |
| Class D | -0.03% | -0.14% | -0.09% | 99.6% | 101.5% | 100.0% | 100.0% |
| Class F | -0.42% | -0.45% | -0.46% | 99.3% | 101.2% | 100.0% | 100.1% |
| Class TGM | -1.11% | -1.03% | -1.10% | 98.6% | 103.4% | 100.0% | 100.2% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.14% | 0.11% | 100.4% | 99.1% | 99.9% | 100.2% |
| Class A2 | -0.05% | 0.04% | 0.02% | 100.2% | 98.6% | 100.9% | 100.1% |
| Class B | 0.00% | 0.16% | -0.06% | 99.9% | 101.7% | 99.4% | 100.2% |
| Class C | -0.01% | 0.03% | 0.03% | 99.4% | 101.7% | 101.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | 0.04% | 0.01% | 99.9% | 100.5% | 100.2% | 100.1% |
| Class D | -0.07% | 0.10% | -0.31% | 100.0% | 100.1% | 100.0% | 100.0% |
| Class F | -0.38% | -0.25% | -0.43% | 99.8% | 101.5% | 99.8% | 99.8% |
| Class TGM | -0.62% | -0.83% | -0.85% | 100.0% | 100.1% | 99.8% | 99.6% |

# Conclusion

Test EE2-1.3 was cross-checked and the results match with the results provided by the proponent. Inspection of the code shows consistency between the proposition and the implementation.

# References

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, Jul. 2023.

# Patent rights declaration(s)

Nokia does not have any current or pending patent rights relating to the technology described in this contribution.