| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0260-v1 |
| --- | --- |

| Title: | Cross-check report of JVET-AG0176: EE2: 2.6e and 2.6f on LIC improvement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yue Yu Zhihuang Xie | Tel: Email: | yue.yu@oppo.com xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

# Abstract

This document reports the cross-check results of JVET-AG0176, which is EE2-2.6e and 2.6f on LIC improvement. The partial results of crosschecker can match that provided by proponents.

# Introduction

In JVET-AF meeting, three individual tests, EE2-2.1, 2.2 and 2.5 were set up based on contributions JVET-AF0194 [1] and JVET-AF0191 [2]. JVET-AF0194 proposed to signal a LIC flag for inter-prediction merge modes, instead of inheriting the flag value from a merge candidate. The flag is signaled to indicate if the original inherited LIC flag or the reverse LIC flag value is used for a merge candidate. The flag is signaled for regular merge mode, affine merge mode and TM merge mode. Also, it is proposed to enable LIC with PU level BDMVR and BDOF.

In JVET-AF0191 [2], non-local illumination compensation (NLIC) was proposed. The linear model of the current block may be derived from the previously coded inter CUs by minimizing the difference between their reconstruction and prediction samples. When constructing the merge lists, up to 6 and 16 NLIC candidates (obtained from both spatial adjacent and non-adjacent positions) are inserted to the lists of regular merge and subblock merge respectively, and reordered with the existing merge candidates. The lengths of the output merge lists are kept unchanged. The same pattern used for non-adjacent merge mode is reused to locate the non-adjacent positions in the scheme. The following tests were conducted:

| 2.1 | A LIC flag for inter prediction merge modes |
| --- | --- |
| 2.2 | Enable PU level BDMVR and BDOF for bi-predicted LIC |
| 2.5 | Non-local illumination compensation |
| 2.6e | Test 2.2 + Test 2.5 |
| 2.6f | Test 2.1 + Test 2.2 + Test 2.5 |

This document reports the cross check results of EE2-2.26e and EE2-2.26f.

# Results

The simulation was conducted under the ECM CTC[3].

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | -0.12% | -0.06% | -0.04% | 101.5% | 102.4% | 99.2% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.05% | 0.03% | -0.10% | 100.8% | 102.3% | 100.1% | 100.1% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 1. EE2-2.6e over ECM-11.0

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.01% | 0.23% | 0.15% | 102.7% | 101.5% | 100.5% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Table 1. EE2-2.6f over ECM-11.0

# Conclusion

The crosschecker’s results can match proponent’s results.

# Reference

- Y. Zhang, V. Seregin, H. Wang, Z. Zhang, C. -C. Chen, H. Huang and M. Karczewicz, “Non-EE2: On LIC flag in merge mode”, JVET-AF0194, Oct. 2023.

- X. Xiu, C. Ma, N. Yan, H.-J. Jhu, C.-W. Kuo, W. Chen and X. Wang, “Non-EE2: Enhancements on local illumination compensation”, JVET-AF0191, Oct. 2023.

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AE2017, Jul. 2023.