| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, Online, 17-26 January2024 | Document: JVET-AG0295-v4 |
| --- | --- |

| Title: | AHG13: Frequency domain Film Grain Objective Metric with Adaptive Region Selection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Wenhao Zhang, Scott Labrozzi | Tel: Email: | xuewei.meng@disney.com wenhao.zhang2@disney.com scott.labrozzi@disney.com |
| Source: | Disney Streaming |  |  |

_____________________________

# Abstract

In the entertainment industry, film grain is widely present in TV and motion pictures. However, there are no widely used objective metrics to evaluate the quality of the synthesized grainy video, which highly limits the development of film grain related technologies. In the 32rd JVET meeting, we proposed a subband based film grain assessment method (SFGA) . Although SFGA not only enhances our understanding of film grain technology, but also provides a valuable tool for fine-tuning film grain parameters in the modeling and synthesis process, it has several shortcomings that need to be addressed.

- First, the SFGA is conducted on the grain-only image, which means that the clean/denoised image is needed. However, the clean image is usually unavailable, and the denoiser may be not strong enough to extract the film grain from the grainy image properly.

- Second, the SFGA is performed on the manually selected flat regions. However, in the practical use cases, manually selecting all the flat regions for each image is unrealistic. So adaptive region selection is essential.

In this proposal, we extend the frequency domain film grain objective metric to directly take effect on the grained video. Specifically, an automatic grain region selection scheme is proposed. Experimental results show that the film grain metric with region selection on content+grain has a better performance compared to the SFGA proposed in JVET-AF0209. This also greatly improves the completeness of the method and expands the application scenarios.

|  | SFGA in JVET-AF0209 |  | Proposed SFGA 2.0 |  |
| --- | --- | --- | --- | --- |
|  | PLCC | SROCC | PLCC | SROCC |
| Image1 | -0.777 | -0.790 | -0.841 | -0.851 |
| Image2 | -0.782 | -0.771 | -0.868 | -0.904 |
| Image3 | -0.830 | -0.741 | -0.807 | -0.842 |
| Image4 | -0.862 | -0.767 | -0.838 | -0.828 |
| Image5 | -0.807 | -0.689 | -0.784 | -0.805 |
| Image6 | -0.831 | -0.721 | -0.857 | -0.753 |
| Average | -0.815 | -0.747 | -0.833 | -0.831 |
| All | -0.759 | -0.777 | -0.809 | -0.821 |

# Introduction

In the entertainment industry, film grain [1][2][3] is widely present in TV and motion pictures. It is originally a characteristic of analog film, and is the result of the processes of exposing and developing silver halide crystals, i.e., light-sensitive crystals that when exposed to light capture an image on a film. With the development of digital camera sensors, film grain no longer exists. However, most professional photographers and filmmakers would rather stick with the film grain as analog aspect when it comes to producing artistic content to better capture the sought atmosphere and evoke the desired emotions. Hence, film grain is often added to the captured videos by post-processing methods.

However, when encoding these TV or motion picture videos, preserving film grain under limited bandwidth is a challenging task. To address this issue, film grain synthesis methods are proposed in video coding. A typical film grain framework can be described as shown in Figure 1. Film grain is removed in the encoder side and generated in the decoder side.

Figure 1. A typical framework of film grain synthesis.

Although film grain synthesis has been regarded as one of the most important features in state-of-the-art video coding standards, such as H.266/VVC and AV1. There are no widely used objective metrics to evaluate the quality of the synthesized grainy video, which highly limits the development of film grain related technologies.

In the 32rd JVET meeting, we proposed a subband based film grain assessment method (SFGA)[4], as shown in Figure 2. SFGA is designed as a full-reference film grain quality assessment tool. It initially transforms both reference and test images from the spatial domain to the frequency domain, then selects several sub-bands within the frequency domain, compares their sub-band noise power spectra, and subsequently combines the sub-band results. The PLCC and SROCC between the proposed metric and MOS are both larger than 0.75, outperforming existing metrics like NSS (Natural Scene Statistics), FFT (Fast Fourier Transform), PSNR, and SSIM.

Figure 2. Subband based film grain assessment method (SFGA) framework.

Although SFGA not only enhances our understanding of film grain technology, but also provides a valuable tool for fine-tuning film grain parameters in the modeling and synthesis process, it has several shortcomings that need to be addressed.

- First, the SFGA is conducted on the grain-only image, which means that the clean/denoised image is needed. However, the clean image is usually unavailable, and the denoiser may be not strong enough to extract the film grain from the grainy image properly.

- Second, the SFGA is performed on the manually selected flat regions. However, in the practical use cases, manually selecting all the flat regions for each image is unrealistic. So adaptive region selection is essential.

In this proposal, we propose a frequency domain film grain objective metric with adaptive region selection. Specifically, an adaptive region selection scheme is proposed. Experimental results show that the film grain metric with region selection on content+grain has a better performance compared to the last version proposed in JVET-AF0209 [4]. This also greatly improves the completeness of the method and expands the application scenarios.

# Method

The framework of the proposed SFGA with adaptive region selection is shown in Figure 3. Edge detection is first conducted to derive the edge map of the reference image. Then, with the edge map, flat region detection is performed to select N flat regions. Then Region 1 to Region N are input to the SFGA. Note that Region 1 to Region N represent the two regions from reference image R and test image T with the same location. Finally, the score of each region is fused to get the final score.

Figure 3. Subband based film grain assessment method (SFGA) with adaptive region selection.

## Edge Detection

There are lots of edge detection methods in image processing area, such as Sobel Detector, Canny Detector, and so on. Considering the purpose of edge detection is for the following flat region detection, the edge detection is conducted on each non-overlapped 32x32(can be adjusted) block. For each block, determine if it is a flat block, based on the gradient, variance, luma value and so on. The output of this step is a flat region map.

- For 10bit videos, if the luma value is smaller than 130 or larger than 850, film grain becomes almost unobservable. There is no point in assessing the quality of the film grain on these areas. Hence, these pixels are all non-flat pixels.

- If the variance of a block is smaller than a threshold, this region almost has no film grain. Hence, it is also be marked as non-flat regions.

As shown below, Figure 4 is the input image to be evaluated. Figure 5 is the output of flat region detection. The white regions are flat regions, and the black regions are non-flat regions.

Figure 4. Input image.

Figure 5. Edge map.

## Flat Region Detection

The purpose of this step is to select several flat MxM blocks from the flat map. Based on the experimental results, 256x256 and 128x128 block sizes are used for this step. As shown in Figure 6, the white blocks represent the selected flat regions.

Figure 6. Detected flat regions.

## Score Adaptive Fusion

As there are N blocks selected, there are N scores in total. The final score is calculated by the weighted average of these N scores,

Score= i=1NwiScorei ,

where w1, w2, …, wn are the weights. And i=1Nwi=1.

# Experimental Results

All the dataset and experiment conditions are identical to that in JVET-AF0209[4]. The results are shown in Table 1. It can be seen that the proposed film grain metric with region selection on content+grain in this proposal has a better overall performance compared to the last version proposed in JVET [4]. This also greatly improves the completeness of the method and expands the application scenarios.

Table 1. Experimental results of the proposed method.

|  | SFGA in JVET-AF0209 |  | Proposed SFGA 2.0 |  |
| --- | --- | --- | --- | --- |
|  | PLCC | SROCC | PLCC | SROCC |
| Image1 | -0.777 | -0.790 | -0.841 | -0.851 |
| Image2 | -0.782 | -0.771 | -0.868 | -0.904 |
| Image3 | -0.830 | -0.741 | -0.807 | -0.842 |
| Image4 | -0.862 | -0.767 | -0.838 | -0.828 |
| Image5 | -0.807 | -0.689 | -0.784 | -0.805 |
| Image6 | -0.831 | -0.721 | -0.857 | -0.753 |
| Average | -0.815 | -0.747 | -0.833 | -0.831 |
| All | -0.759 | -0.777 | -0.809 | -0.821 |

# Conclusion

In this proposal, a frequency domain film grain objective metrics with adaptive region selection was proposed. Experimental results show that the proposed film grain metric with region selection on content+grain in this proposal has a better performance compared to the SFGA proposed in JVET-AF0209. This also greatly improves the completeness of the method and expands the application scenarios.

# References

[1] X. Meng, W. Zhang, S. Labrozzi. AHG13: On Film Grain Synthesis Subjective Evaluation. JVET-AC0199, 29th JVET meeting, teleconference, Jan. 2023.

[2] X. Meng, W. Zhang, A. Norkin, J. Sole, A. Duenas. “On Film Grain Synthesis Subjective Evaluation and Further Work”. JVET-AD0238, 30th JVET meeting, Antalya, Apr. 2023.

[3] X. Meng, W. Zhang, S. Labrozzi. “AHG13: On Film Grain Objective Metrics”. JVET-AD0241, 30th JVET meeting, Antalya, Apr. 2023.

[4] X. Meng, W. Zhang, S. Labrozzi. “AHG13: Frequency domain Film Grain Objective Metrics”. JVET-AF0209, 32nd JVET meeting, Hannover, Oct. 2023.

[5] Mittal, A., Soundararajan, R., & Bovik, A. C. (2012). Making a “completely blind” image quality analyzer. IEEE Signal processing letters, 20(3), 209-212.

# Patent rights declaration(s)

Disney Streaming Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).