| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0308-v1 |
| --- | --- |

| Title: | CICP extensions for depth and alpha signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Podborski A. Tourapis | Tel: Email: | dpodborski@apple.com atourapis@apple.com |
| Source: | Apple Inc. |  |  |

_____________________________

# Abstract

This contribution extends the colour primaries and matrix coefficients code points in CICP to also allow the support of depth and alpha information. The intent of this extension is to enable carriage of such information in single layer bitstreams using existing video coding specifications, such as AVC and HEVC, without the need of specifying any additional metadata.

# Introduction

This proposal presents a method to label depth and alpha information in multimedia streams using Codec Independent Code Points (CICP). Although some existing coding specifications may have mechanisms to tag such information, they are often specific to the coding specification and require navigating through complex metadata structures. In addition, they often allow such tagging for auxiliary pictures only. The proposed approach leverages the flexibility of CICP, which is designed to be universal and straightforward.

In particular, we propose to add new entries to the colour primaries and matrix coefficients code points in the CICP specification, for identifying streams that contain depth or alpha information. This method simplifies the labeling process and offers a more efficient, backwards compatible, and accessible way to manage and process multimedia content with depth and alpha components across various codecs and applications.

# Proposed solution

## Depth and alpha signalled as new colour primaries

colour_primaries indicates either, when the image is a texture image, the chromaticity coordinates of the source primaries as specified in Table E.3 in terms of the CIE 1931 definition of x and y as specified in ISO 11664-1 or, when the image is not a texture image the type of the image.

When the colour_primaries syntax element is not present, the value of colour_primaries is inferred to be equal to 2 (the chromaticity is unspecified or is determined by the application). Values of colour_primaries that are identified as reserved in Table E.3 are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders shall interpret reserved values of colour_primaries as equivalent to the value 2.

Table E.3 — Colour primaries interpretation using colour_primaries syntax element

| Value | Primaries |  |  |  | Informative remark |
| --- | --- | --- | --- | --- | --- |
| 0 | Reserved |  |  |  | For future use by ITU-T ¦ ISO/IEC |
| 1 | primary | x | y |  | Rec. ITU-R BT.709-6 Rec. ITU-R BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-1 sRGB or sYCC IEC 61966-2-4 SMPTE RP 177 (1993) Annex B |
|  | green | 0.300 | 0.600 |  |  |
|  | blue | 0.150 | 0.060 |  |  |
|  | red | 0.640 | 0.330 |  |  |
|  | white D65 | 0.312 7 | 0.329 0 |  |  |
| … |  |  |  |  |  |
| 22 | primary | x |  | y | EBU Tech. 3213-E (1975) |
|  | green | 0.295 |  | 0.605 |  |
|  | blue | 0.155 |  | 0.077 |  |
|  | red | 0.630 |  | 0.340 |  |
|  | white D65 | 0.312 7 |  | 0.329 0 |  |
| 23..127 | Reserved |  |  |  | For future use by ITU-T ¦ ISO/IEC |
| 128 | Alpha map |  |  |  | This signal is an alpha map |
| 129 | Depth map |  |  |  | This signal is a depth picture |
| 130..255 | Reserved |  |  |  | For future use by ITU-T ¦ ISO/IEC |

Table E.5 — Matrix coefficients interpretation using the matrix_coeffs syntax element

| Value | Matrix | Informative remark |
| --- | --- | --- |
| 0 | Identity | The identity matrix. Typically used for GBR (often referred to as RGB); however, may also be used for YZX (often referred to as XYZ) IEC 61966-2-1 sRGB SMPTE ST 428-1 (2006) See Formulae (E-31) to (E-33) |
| 1 | KR = 0.212 6; KB = 0.072 2 | ITU-R Rec. BT.709-6 ITU-R Rec. BT.1361-0 conventional colour gamut system and extended colour gamut system (historical) IEC 61966-2-1 sYCC IEC 61966-2-4 xvYCC709 SMPTE RP 177 (1993) Annex B See Formulae (E-28) to (E-30) |
| … |  |  |
| 14 | ICTCP | Rec. ITU-R BT.2100-2 ICTCP See Formulae (E-62) to (E-64) for transfer_characteristics value 16 (PQ) See Formulae (E-65) to (E-67) for transfer_characteristics value 18 (HLG) |
| 15 | IPT-C2 | Society of Motion Picture and Television Engineers ST 2128 See Formulae (E-75) to (E-77) |
| 16 | YCgCo-Re | See Formulae (E-41) to (E-55) |
| 17 | YCgCo-Ro | See Formulae (E-41) to (E-55) |
| 18 | 1st colour component | A monochromatic representation with only the first colour component in the original representation present in the coded image. Typically used for Green only; however, may also be used for the Y tristimulus value (luminance), when the colour primaries are set to XYΖ, or to indicate that the representation contains only the first component of an image not representing texture. |
| 19 | 2nd colour component | A monochromatic representation with only the second colour component in the original representation present in the coded image. Typically used for Red only; however, may also be used for the X tristimulus value, when the colour primaries are set to XYΖ, or to indicate that the representation contains only the second component of an image not representing texture. |
| 20 | 3rd colour component | A monochromatic representation with only the third colour component in the original representation present in the coded image. Typically used for Blue only; however, may also be used for the Z tristimulus value, when the colour primaries are set to XYΖ, or to indicate that the representation contains only the third component of an image not representing texture. |
| 21 | Luma only KR = 0.2126; KB = 0.0722 | A monochromatic representation of the luma component of the Rec. ITU-R BT.709-6 representation. See Formula (E-28) |
| 22 | Luma only KR = 0.2627; KB = 0.0593 | A monochromatic representation of the luma component of the Rec. ITU-R BT.2020-2 non-constant luminance representation. See Formula (E-28) |
| 23 | I only - ICTCP | A monochromatic representation of the I component of ICTCP representation. See Formula (E-75) |
| 24..255 | Reserved | For future use by ITU-T ¦ ISO/IEC |

# Conclusion

We propose to add the presented extensions to CICP specification.

# Patent rights declaration(s)

Apple Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Rec. ITU-T H.264 | ISO/IEC 14496-10, Advanced video coding for generic audiovisual services.

- Rec. ITU-T H.265 | ISO/IEC 23008-2, High efficiency video coding.

- Rec. ITU-T H.273 | ISO/IEC 23091-2, Coding-independent code points for video signal type identification.