| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0319-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0116 (AHG12: GOP-based RPR encoder control for ECM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jani Lainema | Tel: Email: | - first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports a cross-verification of techniques described in JVET-AG0116 (AHG12: GOP-based RPR encoder control for ECM). The software provided by the proponents of JVET-AG0116 was studied and it was found to match the description given in the contribution.

Simulations were performed according to the JVET Common Test Conditions for QPs 32 and 37 using RA and LD-B configurations with the following ECM encoder configuration parameters to enable the proposed functionality:

RPR            : 1
UpscaledOutput : 2
GOPBasedRPR    : 1

All the BD-rate results matched with the ones reported in JVET-AG0116. Also, the runtime and peak memory measurements were close to the ones reported in the studied contribution (within a typical margin considering differences in computing platforms and computing load variations). Full results are provided in the attached spreadsheets.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).