| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0321-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AG0198 (Non-EE2: Coding Information-based ALF Classification) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Nan Hu 5775 Morehouse Dr, San Diego, CA 92121, USA | Tel: Email: | nanh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution is a crosscheck report of JVET-AG0198 (Non-EE2: Coding Information-based ALF Classification). The simulations were carried out based on the common test condition. The partial results are matched in both BD-rate and running time.

# Introduction

In ECM-11.0, ALF supports three classifiers for each 2x2 luma block: Laplacian classifier which is based on activity and directionality values calculated from Laplacian values in a surrounding window, band-based classifier which is based on quantizing the sample values of current block, and residual-based classifier which is based on quantizing the absolute residual values of current block.

In JVET-AG0198, the number of classes of existing ALF classifiers are reduced to 12 from 25. Each block is firstly classified into 2 noise levels. If the block is located at a CU or TU boundary or coded by intra mode, it is classified into noise level 1. Otherwise, the block is classified into noise level 0.

Then the class index is derived by combining the noise level and class index from existing ALF classifiers. For each classifier, the number of classes is changed into 12 x 2 = 24.

# Simulation results

The proposed method was implemented on top of ECM-11.0 and tested following common test condition described in JVET-AF2017.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | 0.00% | 0.00% | 0.00% | 97.4% | 97.8% | 98.9% | 99.1% |
| Class E | 0.00% | -0.01% | -0.01% | 100.3% | 97.9% | 98.6% | 99.5% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.00% | 0.00% | 0.00% | 100.0% | 100.6% | 98.8% | 98.9% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.02% | -0.01% | 96.4% | 95.3% | 99.7% | 100.0% |
| Class C | -0.10% | -0.03% | 0.02% | 99.3% | 95.8% | 98.7% | 99.3% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.04% | -0.05% | 101.2% | 99.4% | 99.2% | 99.0% |
| Class F | -0.04% | -0.04% | -0.09% | 96.6% | 96.6% | 100.0% | 99.7% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |
| Class C | -0.12% | -0.10% | -0.20% | 97.2% | 96.0% | 99.6% | 99.6% |
| Class E | -0.15% | -0.70% | -0.34% | 97.4% | 97.2% | 99.1% | 98.6% |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.05% | -0.18% | -0.41% | 99.8% | 99.0% | 99.7% | 99.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |