| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AG0324-v1 |
| --- | --- |

| Title: | AHG4/AHG13/AHG3: film grain tuning tool tutorial |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | P. de Lagrange | Email: | philippe.delagrange@interdigital.com |  |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution describes how to use the “FGC SEI designer” graphical tool to display and interactively edit an FGC SEI configuration file. That tool is available in the JVET gitlab (in “JVET AHG Film Grain Technologies” group). The intent is to encourage experimentation and showcase the capabilities of the FGC SEI in conjunction with the implementation provided in the same repository, which is asserted to be hardware-friendly and interoperable with different film grain models.

It is also asked whether interactive testing based on this tool could be used in the context of AHG4/AG5, to evaluate the grain fidelity aspect considered in the second category of the FGC SEI visual testing plan (JVET-AD2022).

# Introduction

A group called “JVET AHG Film Grain Technologies” has been created on the JVET gitlab (https://vcgit.hhi.fraunhofer.de/jvet-ahg-fgt) to contain software developments related to film grain that are either independent or not yet merged into reference software.

A recent addition is “FGC SEI designer”, a graphical interactive tool to edit an FCG SEI config file, that can be useful when automatic estimation is either not practical or needs to be refined. It can also be used to export a graphical representation of film grain parameters, for use e.g. in a document, and to showcase the capabilities of the FGC SEI.

As it is implemented as a single python script, and connected to a film grain synthesizer, it was added to the “VFGS” software repository (https://vcgit.hhi.fraunhofer.de/jvet-ahg-fgt/vfgs), rather than in an independent repository. External dependencies are matplotlib and numpy.

The tool is also available from InterDigital github (https://github.com/InterDigitalInc/VersatileFilmGrain), which is in fact its primary source.

The film grain synthesizer available in the same repository and connected to the graphical tool is intended to be hardware-friendly, actually designed as a hardware model with clearly separated hardware and firmware layers, and interoperable with different grain models and metadata formats, though this interoperability is not yet exercised by the graphical tool (this is part of future plans).

# Description of the tool

## Graphical display and edit

Figure 1 reproduces the main window of the tool, with arbitrary default film grain parameters when no configuration file is loaded yet.

Figure 1 – Main tool window

An FGC SEI configuration file can be loaded and saved through the “File” menu. Currently, only frequency-filtering mode is supported. The syntax used in the configuration file is the same as the VTM; there is a direct mapping between the interactive plot and FGC SEI model parameters.

Color component is selected with the “color component” menu.

Intensity interval boundaries, gain levels, and frequency cutoffs can be click-dragged. An interval is removed when it is reduced to zero-length, and is split by a double-click.

## Example configuration import / export

Figure 2 is an example film grain parameter representation exported with the FGC SEI designer tool, using the CrowRun.cfg file that was used for expert viewing experiment reported in JVET-AD0382. The graphical export is provided by the “save as” feature of matplotlib (the floppy disk icon of the toolbar in Figure 1).

Figure 2 – Film grain parameters used for CrowdRun in JVET-AD0382 (Luma, Cb, and Cr from top to bottom and left to right)

Notes:

- Horizontal axis corresponds to target image sample value (or “intensity”), normalized to 8-bits.

- Here, log2ScaleFactor equals 4, which means that the gain axis is scaled down by 2^4 = 16, for an 8-bit target image (for a 10-bit target image, is it scaled down 4 times less).

## Tool chain for experimentation

Encoder

Decoder

FGS

source picture

FilmGrainApp

decoded picture

regrained picture

Analyzer

.cfg

Preview window

FGC SEI designer (main window)

insert SEI messages

execute

execute

Figure 3 – Film grain experimental tool chain

Figure 3 depicts a tool chain that could be used to experiment with film grain synthesis, including the graphical tool described here. Items in green are still under construction.

The FGC SEI designer tool is connected to an external film grain synthesizer, so that immediate feedback of adjustments can reflected on a test video (the dashed “execute” arrow in Figure 3). A YUV viewer has to be used to watch the result until the preview window is available.

The configuration can also be saved and used in the FilmGrainApp program provided with the HM and VTM to insert FGC SEI messages in a bitstream.

Further work include:

- Estimate initial parameters when provided with the source and clean pictures (by e.g. connecting an external analyzer)

- Support of auto-regressive mode, and potentially translations between FF and AR modes

- Compare source / clean / re-grained pictures in a preview window

# Example of use

## Prepare original and degrained/decoded videos

The tool can interactively test film grain synthesis on a degrained video. This is done by automatically running the vfgs film grain synthesizer every time anything changes in the GUI. The test video is specified using the [File / Load YUV (clean)] menu, shown in Figure 4.

The degrained/clean video can be a decoded video without having to use any specific denoiser, since encoders likely remove grain by themselves. We suggest selecting a quality level (QP) where image quality is still very good but grain is removed in most pictures (see next section for more details). QP 26 for HM or VTM encodings, for example.

The temporary parameters are saved in __preview.cfg and result of film grain synthesis is output in the __preview_3840x2160_8b.yuv file in the current directory. The original and regrained images can be compared in a YUV viewer, preferably one that refreshes display upon file changes, so that the results of a change in GUI can be seen immediately.

Figure 4 – Loading a test video file

## Select a decoded picture for study

Grain may not be completely removed in all decoded pictures, for example some grain can remain on Intra pictures, and some badly impaired grain (visually annoying) can be present in the first temporal layers (e.g. Intra + 16, 8, 24), as illustrated in Figure 5.

A totally clean picture can be selected for easy grain estimation by comparison with the source, or one where grain is badly impaired to observe how synthetic grain magically removes the problem, as show in Figure 6.

The relevant test frame is selected with the slider on the bottom of the window, as shown in Figure 7.

Figure 5 – Example decoded video at POC #0, #16, #8, #11

Figure 6 – Example of how grain synthesis hides impaired grain
Left: decoded; right: decoded + grain synthesis

Figure 7 – Selecting the test frame using the slider

## Focus on a light area, tune amplitude and size

The experimentation can start with a flat configuration, for example load « Gaussian-1s.cfg » (using [File / Load cfg] menu), first focusing on luma, on a light area with a good amount a grain (e.g. sky, or white surface).

Then Log2ScaleFactor slider can be adjusted to 4 for example, and the gain (blue curve) can be dragged to match original grain amplitude. There is no need to be very precise at this stage, however it can be observed that the eye is rather sensitive to variations of grain amplitude; Figure 9 illustrates grain synthesis strength setting either 20% too low or too strong compared to the estimated sweet spot.

Figure 8 – Adjusting log2ScaleFactor, grain strength and size

Figure 9 – Tuning grain strength
Left: original - Middle: synthesis, too low – Right: synthesis, too strong

Then, the green curve can be adjusted (grain bandwidth or size) to best match original. This is more difficult to get right; it may help to use 100% and 200% viewing scales, and compare the grains themselves (difference between grainy and clean images). Amplitude may need to be refined at the same time, since reducing the bandwidth also reduces the total energy of synthesized grain.

The grain seed can be changed by touching the YUV frame slider on the bottom if the grain pattern needs to be changed.

On this example, we keep 14 for the light area as it looks right (grain is very fine here).

Figure 10 – Tuning grain size
Left: original - Right: synthesis, too large

## Split interval and focus on a darker area

In Figure 12, it can be observed that the parameters tuned for the light area do not work on darker areas. This illustrates the interest of the FGC SEI, that can adapt model parameters to light level.

The interval can be split by double-click, then one sub-interval can be disabled by right-click, and interval boundary can be dragged to isolate an area of interest (e.g. dark clothes), as explained in Figure 11.

double-click to split interval

right-click to temporary disable the light part

drag to adjust focus area

Figure 11 – Splitting interval and adjusting focus on a dark area

Figure 12 – Focusing on a dark area
Left: original – Middle: wrong parameters – Right: dark area isolated

Observing the grain pattern of the original (by difference with decoded) in Figure 13, it is evident that grain is both less strong and smoother/coarser in the dark. Grain amplitude (blue curve) and size (green curve) are then dragged to match original on the area under study, with the result shown in Figure 14.

Figure 13 – Tuning grain on a dark area
Left: original – Middle: original grain – Right: dark area tuned

Figure 14 – Parameter adjustment for dark area

## Create more intervals, and repeat on chroma

The reach the result shown in section 2.2, more work is needed: refine intervals (iterative split and tuning), and repeat the process on each chroma channel.

It is noted that sometimes, the grain is not fully consistent over the picture for a given intensity interval, and a compromise has to be found. This might be caused by film warping, or color space. It is then recommended to check the settings on the whole picture, out of the focus areas that were used for tuning.

# Conclusion

A graphical tool to display and adjust film grain parameters is presented, to encourage and ease experimentation of film grain synthesis technology, and demonstrate the capabilities of FGC SEI in conjunction with a hardware-friendly implementation.

This tool also enables interactive visual testing, that may be interesting for the “grain fidelity” aspect (second category) of the FGC SEI visual testing plan (JVET-AD2022).