| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG2035-v1 |
| --- | --- |

| Title: | Test conditions and evaluation procedures for generative face video coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Sean McCarthy Bolin Chen | Email: | sean.mccarthy@dolby.com chenbolin.chenboli@alibaba-inc.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document specifies test conditions and software reference configurations for testing of generative face video coding (GFVC). The conditions and configurations are recommended for use in applicable technical contributions to JVET meetings.

Changes yet to be integrated:

Summary: changes noted below corresponding to aspects of JVET-AG_notes_d9, JVET-AG0016, JVET-AG0042, JVET-AG0048, and JVET-AG0187

- JVET-AG_notes_d9

  - The ratio of bits that are used for base picture and other pictures should be reported. As the QP for base pictures is lower than for the I picture in VVC in some cases, this might cause different latency.

  - Information about the overlap in terms of quality should be added in the .xls

- JVET-AG0016 recommendations for initial testing conditions to be used for GFVC

  - 8-bit, SDR, low delay configurations

  - VVC QP: 37, 42, 47, 52

  - GFVC QP (for base pictures): 22, 32, 42, 52

- JVET-AG0187

  - Test sequences

  - Software:

  - Results template:

  - Annex B Objective metrics

Changes that have been integrated:

Summary: changes noted below corresponding to aspects JVET-AG_notes_d9, JVET-AG0016, JVET-AG0042, JVET-AG0048, and JVET-AG0187

- JVET-AG_notes_d9

- JVET-AG0016 recommendations for initial testing conditions to be used for GFVC

  - 8-bit, SDR, low delay configurations

  - 15 VOXCELEB test sequences and 18 CFVQA test sequences as identified in JVET-AG0042, where the 18 CFVQA sequences also include the 3 extra “bad case” sequences provided in JVET-AG0042-v3, all sequences are in 256x256 resolution

  - VVC QP: 37, 42, 47, 52

  - GFVC QP (for base pictures): 22, 32, 42, 52

- JVET-AG0187

  - Test sequences

    - Add ftp link for test sequences.

    - Add Frame Count, Frame Rate, and Bit Depth for CFVQA_016-018 in Table 1

    - Add MD5 hashes to Annex A

  - Software: Add link to CFVC branch of VTM including translator software (JVET-AG0048)

  - Results template: Attach Excel spreadsheet

  - Annex B Objective metrics

    - add informative description of DISTS

    - add informative description of LPIPS

# Introduction

Specified test conditions for generative face video are desirable to conduct experiments in a well-defined environment and facilitate the comparison of the outcome of experiments.

This document defines 2 test conditions for SDR video content and low-delay settings:

- Low delay, 8 bit

- Low delay, P slices only, 8 bit (optional)

The following sections define test sequences, quantization parameter values, encoder configuration files, and compile-time options to be used.

# Test sequences

Table 1 defines the sets of test sequences to be used for low-delay conditions. Class A test sequences are selected from the VoxCeleb dataset [1]. Class B test sequences are selected from the CFVQA dataset [2]. All test sequences are 8-bit RGB 4:4:4 with sample width x height of 256 x 256.

Test sequences in Table 1 are available on ftp://ftp.ient.rwth-aachen.de/JVET-AG_AhG16-GFVC/.

Test sequences are made available only to qualified JVET participants. Qualified participants may contact the JVET chair for login information. Specific license information is provided with each Class of test sequence.

Table 1. Test sequences

| Class | Sequence name | Frame count | Frame rate | Bit depth | Low-delay |
| --- | --- | --- | --- | --- | --- |
| A | VOXCELEB_001 | 250 | 25 | 8 | M |
| A | VOXCELEB_002 | 250 | 25 | 8 | M |
| A | VOXCELEB_003 | 250 | 25 | 8 | M |
| A | VOXCELEB_004 | 250 | 25 | 8 | M |
| A | VOXCELEB_005 | 250 | 25 | 8 | M |
| A | VOXCELEB_006 | 250 | 25 | 8 | M |
| A | VOXCELEB_007 | 250 | 25 | 8 | M |
| A | VOXCELEB_008 | 250 | 25 | 8 | M |
| A | VOXCELEB_009 | 250 | 25 | 8 | M |
| A | VOXCELEB_010 | 250 | 25 | 8 | M |
| A | VOXCELEB_011 | 250 | 25 | 8 | M |
| A | VOXCELEB_012 | 250 | 25 | 8 | M |
| A | VOXCELEB_013 | 250 | 25 | 8 | M |
| A | VOXCELEB_014 | 250 | 25 | 8 | M |
| B | VOXCELEB_015 | 125 | 25 | 8 | M |
| B | CFVQA_001 | 125 | 25 | 8 | M |
| B | CFVQA_002 | 125 | 25 | 8 | M |
| B | CFVQA_003 | 125 | 25 | 8 | M |
| B | CFVQA_004 | 125 | 25 | 8 | M |
| B | CFVQA_005 | 125 | 25 | 8 | M |
| B | CFVQA_006 | 125 | 25 | 8 | M |
| B | CFVQA_007 | 125 | 25 | 8 | M |
| B | CFVQA_008 | 125 | 25 | 8 | M |
| B | CFVQA_009 | 125 | 25 | 8 | M |
| B | CFVQA_010 | 125 | 25 | 8 | M |
| B | CFVQA_011 | 125 | 25 | 8 | M |
| B | CFVQA_012 | 125 | 25 | 8 | M |
| B | CFVQA_013 | 125 | 25 | 8 | M |
| B | CFVQA_014 | 125 | 25 | 8 | M |
| B | CFVQA_015 | 125 | 25 | 8 | M |
| B | CFVQA_016 | 125 | 25 | 8 | M |
| B | CFVQA_017 | 125 | 25 | 8 | M |
| B | CFVQA_018 | 125 | 25 | 8 | M |

MD5 hashes and file names for each sequence are listed in Annex A. The hashes shall be used to verify that the correct version of the test sequence is used.

Notes:

- When the encoder operates in 8-bit mode (InternalBitDepth=8) for a 10-bit source, each 10-bit source sample x is converted prior to encoding to an 8-bit value (x+2) / 4 clipped to the [0,255] range. Similarly, when the encoder operates in 10-bit mode (InternalBitDepth=10) for an 8-bit source, each 8-bit source sample x is converted prior to encoding to a 10-bit value 4*x. This behavior is built into the reference encoder and no external conversion program is required.

- In the table above, “M” and “O” stand for mandatory and optional, respectively.

- “-” indicates that the test sequence is not requested in the JVET CTC or the given configuration.

# Quantization parameter values

For each video sequence, four quantization parameter values are to be used to generate low-delay anchor bitstreams: 37, 42, 47 and 52.

For GFVC bitstreams, the base picture is coded as intra using quantization parameter values 22, 32, 42 and 52, corresponding respectively to the four low-delay anchor bitstreams.

# Configuration files

The following sections define encoder configuration files to be used for each test case. Two configuration files, a sequence-level configuration file and a VTM configuration file, are used for each test point

## Sequence-level configuration files

Sequence-level parameters to be changed for each test point are:

- InputFile to reflect the location of the source video sequence on the test system

- FrameRate to reflect the frame rate of a given sequence as per Table 1

- SourceWidth to reflect the width of the source video sequence

- SourceHeight to reflect the height of the source video sequence

- FramesToBeEncoded to reflect the frame count of a given sequence as per Table 1

- InputBitDepth to reflect the bit depth of a given sequence as per Table 1

The following configuration files are provided in the cfg/per-sequence folder of the VTM software package (available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc/gfvc_v1/-/tree/main/vtm

Test condition configuration files:

- RGB 4:4:4: inputrgb444.cfg

- YUV 4:2:0: inputyuv420.cfg

## VTM configuration files

The QP parameter is additionally to be changed for each test point as specified in section 3.

The following configuration files are provided in the cfg/ folder of the VTM software package (available at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc/gfvc_v1/-/tree/main/vtm

Test condition configuration files:

- “Low-delay B” (LB): encoder_lowdelay_vtm.cfg

- “Low-delay P” (LP, optional): encoder_lowdelay_P_vtm.cfg

## Compile-time settings

Compile-time settings are defined mostly in the TypeDef.h file located in the source/Lib/CommonLib folder of the reference software. The default settings provided in the source code should be used.

[Ed: Update compile time settings section to reflect that the current software needs to run on CUDA with Python. The C++ libraries cannot be used. Note that it might be possible to remove the hardcoded dependencies on CUDA]

# Anchors

The VTM is provided as an anchor. Corresponding configuration files are provided, as described in section 4

# Reported Metrics

The attached SDR Excel sheets contain a reporting template in which bitrate, LPIPS (in RGB444 domain), DISTS (in RGB444 domain), RGB444_PSNR, RGB444_SSIM, YUV420_PSNR, YUV420_SSIM, and BD-rate results are reported for the tested configuration against the anchor.

For 10-bit video, PSNR is calculated as 10*(log10((255 << 2) / MSE), 8-bit content is converted to 10-bit input in the encoder by shifting 2 bits to the left and 10-bit PSNR calculation is used to report testing results. This behavior is built into the reference encoder and no external conversion program is required.

In the attached Excel sheets, BD-rate is calculated using both piece-wise cubic and cubic interpolations, and the piece-wise cubic interpolation is used in the results summary.

# Results Template

An excel template file for reporting test results is attached to this document.

# References

- Nagrani A, Chung J S, Zisserman A. Voxceleb: a large-scale speaker identification dataset[J]. arXiv preprint arXiv:1706.08612, 2017.

- Li Y, Chen B, Chen B, et al. Perceptual Quality Assessment of Face Video Compression: A Benchmark and An Effective Method[J]. arXiv preprint arXiv:2304.07056, 2023.

- Ding K, Ma K, Wang S, et al. Image quality assessment: Unifying structure and texture similarity[J]. IEEE transactions on pattern analysis and machine intelligence, 2020, 44(5): 2567-2581.

- Zhang R, Isola P, Efros A A, et al. The unreasonable effectiveness of deep features as a perceptual metric[C]//Proceedings of the IEEE conference on computer vision and pattern recognition. 2018: 586-595.

# Annex A. Test sequence checksums

For each test sequence included in the common test conditions, the MD5 hash is listed below. The list can be copied into a text file and hashes can be checked using the md5sum tool:

md5sum -c <filename>

Table 2. Test sequences with MD5 hash

| Class | Sequence name | MD5 hash |
| --- | --- | --- |
| A | VOXCELEB_001 | 8ea5e795ed71f07f22e41164c3833ebd |
| A | VOXCELEB_002 | 6c8c677094f63d281d54eb863d261957 |
| A | VOXCELEB_003 | c093048c9ab6ca1657516ce229af0f65 |
| A | VOXCELEB_004 | 0b297082da57275b0471d6f62a5ae001 |
| A | VOXCELEB_005 | fd34a39acb3df265e33b7db4085f31a2 |
| A | VOXCELEB_006 | 19cdc47851996e68aa152c12bc2c0d1b |
| A | VOXCELEB_007 | afbb3c91d3bc51e820fb7d5612637d73 |
| A | VOXCELEB_008 | 6d9d2757871ef2c2695fd3829a8e64b3 |
| A | VOXCELEB_009 | f6283533d5d81b8136f666a505b313a3 |
| A | VOXCELEB_010 | 75ba2bfe9d960f0ebd6926ff3efc7e22 |
| A | VOXCELEB_011 | d4ce6a3e7f53b4462fbdfee357958f30 |
| A | VOXCELEB_012 | d0820289ebbed9054fc3e1900a68c76a |
| A | VOXCELEB_013 | ea1bd345f37c771bdf33503213a95f3f |
| A | VOXCELEB_014 | 6560c86c8d3258d7d5da5e0bc36eae88 |
| B | VOXCELEB_015 | afd51711787cd15689271f43ed2c7b17 |
| B | CFVQA_001 | 0ede421fcadd52286e53fc9a83a30c95 |
| B | CFVQA_002 | 584fed51a629ddee724660186dfecf43 |
| B | CFVQA_003 | e6167734f9d8315ffa57b9264b81ac67 |
| B | CFVQA_004 | ba9f002af7c86d9ad5f77629e20781c4 |
| B | CFVQA_005 | 3e4d0e18b1bd0f1beb720da249968162 |
| B | CFVQA_006 | ec939fbbf2004eb0f25d539e1b8016fa |
| B | CFVQA_007 | 4b8683292639d9a3ed3bc5fcbc710aeb |
| B | CFVQA_008 | 392dbf3bc333787bfb5d4ce2cdeb4cc0 |
| B | CFVQA_009 | cfd74fa97835a8e07c469f2f5e50d978 |
| B | CFVQA_010 | fba807a9ea81df342aad104acd763b5b |
| B | CFVQA_011 | 0d014908a198e389ab6a468f273b2987 |
| B | CFVQA_012 | 6ceb2f9f43f73b5dcd4f49a6e8c2aca8 |
| B | CFVQA_013 | 4a5fb91f18f273b3cfde5f3b045daf26 |
| B | CFVQA_014 | c737c521b1ff5bf7fe002e7026df9c60 |
| B | CFVQA_015 | 04b13ed4b415194395046e168e2dbf89 |
| B | CFVQA_016 | 84ff23036875b3e16e08ece723d5b48b |
| B | CFVQA_017 | 4289c58cb2a6d8d9d820ae8766eaf5c6 |
| B | CFVQA_018 | 27c5b06ed0f8cabf796f1830a2b6823f |

# Annex B: Objective distortion metrics for GFVC quality evaluations

The test conditions described in this document require the following objective distortion metrics to be reported for evaluation: PSNR, SSIM, DISTS (Deep Image Structure and Texture Similarity), and LPIPS (Learned Perceptual Image Patch Similarity).

DISTS [3] is a full-reference image quality model with explicit tolerance to texture resampling, which can transform images to multi-scale overcomplete representations and combine correlations of these spatial averages ("texture similarity") with correlations of the feature maps ("structure similarity").

LPIPS [4] is a full-reference image quality assessment method that can simulate and learn the characteristics of human visual perception via a deep neural network, and evaluate image quality based on these perceptual characteristics.

Implementations of the PSNR, SSIM, DISTS, and LPIPS metrics are provided in multiMetric_rgb444.py and  multiMetric_yuv420.py at https://vcgit.hhi.fraunhofer.de/jvet-ahg-gfvc/gfvc_v1/-/tree/main/evaluate.