| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0014-v3 |
| --- | --- |

| Title: | JVET AHG report: NNVC software development AhG14 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin, R. Chang, Y. Li, Y. Li, M. Santamaria, J. N. Shingala, Z. Xie | Tel: Email: | franck.galpin@interdigital.com renjiechang@tencent.com yue.li@bytedance.com yli30@qti.qualcomm.com maria.santamaria_gomez@nokia.com jay.shingala@ittiam.com xiezhihuang@oppo.com |
| Source: | AHG14 on NNVC software development |  |  |

# Abstract

This report summarizes the activities of the AHG14 on NNVC software development that have taken place between the 34th and 35th JVET meetings.

# Introduction

The mandates for the AhG are as follows:

- Coordinate development of the NNVC software and associated configuration files.

- Prepare and deliver NNVC-9.0 software version (and potential updates) and provide reference configuration encodings according to the NNVC common test conditions as described in JVET-AF2016. Study the impact of the addition of new dataset on the already integrated models.

- Investigate and bridge coding performance of NNVC VTM anchor compared to the latest VTM version and/or VTM anchor in ECM software.

- Investigate combinations of tools included in the NNVC software, prepare and release anchor data for all configurations of the software, including anchors for High and Low Operation Point (HOP/LOP) configurations.

- Study and maintain the SADL (Small Adhoc Deep-Learning Library). Identify gaps in functionality and develop improvements as needed.

- Coordinate with NNVC algorithm and software description (JVET-AH2019) editors to identify any mismatches between software and description document, suggest further updates to the description document as appropriate.

- Coordinate with AHG11 on items related to NNVC activities.

# Software development

## Location

NNVC repository is located at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM

NNVC software is based on VTM-11.0 with enabled MCTF including the update from JVET-V0056, GOP32, enabling deblocking in the RDO and JVET-AH0054 with improved MCTF.

NNVC-9.1 anchor at https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/nnvc-ctc/-/blob/master/Anchor%20performance/NNVC-current_Reporting_Template-with-anchor-data.xlsm?ref_type=heads is used for NNVC performance evaluation.

## Software changes

## NNVC-9.0

Several commits were merged in the NNVC repository.

The following changes were integrated:

- JVET-AH0080 (LOP3): training scripts + models + NNVC changes

- JVET-AH0081 (LOP3): models

- JVET-AH0064: improved MCTF, port from ECM

- JVET-AH0099 (rpr 1.5)

- JVET-AH0051 VLOP

- JVET-AH0096: LOP2 adaptive content: training scripts

- JVET-AH0088: new SADL

The following fixes and clean-up were performed:

- Build for ARM

- Fixes many implicit cast to pass MSVC strict build

- Clean model name

- Add support for Y4M

- Fix memory issue with padding

## NNVC-9.1

The following changes were integrated:

- JVET-AH0096: LOP2 adaptive content: models and NNVC inference

- JVET-AH00189 + JVET-AH0205: HOP4 models + training scripts + NNVC changes

- Extend SPS model ID to support new models

- SADL update for HOP4 support

The following fixes and clean-up were performed:

- Fixes on issues avec NNVC-9.0 integration

- Fixes legacy memory issues from VTM

- JVET-AH0096: fix parameter value for training

## Software version

NNVC-9.1 was tagged May 28th, 2024

NNVC-9.0 was tagged May 13th, 2024

NNVC-8.0 was tagged February 20th, 2024

NNVC-8.0rc3 was tagged February 20th, 2024.

NNVC-7.1 was tagged November 29th, 2023.

NNVC-7.0 was tagged November 3rd, 2023.

NNVC-6.1 was tagged September 25th, 2023 (fix).

NNVC-6.0 was tagged September 6th, 2023.

NNVC-5.1 was tagged July 19th, 2023.

NNVC-5.0 was tagged May 11th, 2023.

NNVC-3.0 (a.k.a VTM-11.0_nnvc3.0) was tagged December 1st, 2022.

NCS-1.0 (a.k.a NNVC-3.0wip2) was tagged September 4th, 2022 (first release containing the FilterSets, using NNVC 2.0 as a base).

VTM-11.0_nnvc-2.0 was tagged August 4th, 2022 (add deblocking in RDO).

VTM-11.0_nnvc-1.0 was tagged May 6th, 2021 (VTM-11.0 base with MCTF enabled).

# CTC performance

See configurations section for naming convention.

Please note that all results are using the Full CatRobot sequence (but using half the frames) which might give slightly different results due to the MCTF processing.

## Comparison to VTM

### NNVC-8.0 VTM vs NNVC-9.1 VTM

AI results are unchanged compared to NNVC-8.0.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.07% | -0.44% | -0.23% | -0.10% | -0.28% | -0.27% | 103% | 100% |
| Class A2 | -0.07% | -0.25% | -0.38% | -0.14% | -0.17% | -0.41% | 100% | 102% |
| Class B | -0.27% | -0.49% | -0.52% | -0.31% | -0.63% | -0.48% | 101% | 98% |
| Class C | -0.01% | -0.22% | 0.00% | -0.03% | 0.03% | -0.03% | 102% | 98% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.12% | -0.36% | -0.29% | -0.16% | -0.29% | -0.30% | 101% | 99% |
| Class D | 0.10% | -0.58% | -0.33% | -0.33% | -1.43% | -0.67% | 99% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 95% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.93% | -0.65% | -1.76% | -0.37% | -0.29% | -1.23% | 105% | 104% |
| Class C | 0.12% | 0.18% | -0.04% | -0.14% | 0.38% | 0.71% | 103% | 112% |
| Class E | -0.85% | -1.11% | -1.17% | -0.57% | -0.79% | -0.59% | 103% | 109% |
| Overall | -0.56% | -0.49% | -1.04% | -0.34% | -0.19% | -0.42% | 104% | 108% |
| Class D | 0.36% | -0.52% | -0.82% | 0.23% | -0.42% | -1.37% | 103% | 108% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 104% | 108% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 96% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 94% | 94% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 96% | 91% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 95% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 105% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 95% |

Note: Results from InterDigital, crosschecked by OPPO.

### NNVC-9.1 VTM vs NNVC-9.1 anchor

The NNVC-9.1 anchor includes LOP.3 filter and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -8.81% | -11.81% | -12.53% | -9.44% | -13.31% | -14.42% | 122% | 8345% |
| Class A2 | -7.55% | -12.21% | -8.04% | -8.14% | -12.90% | -7.89% | 120% | 7955% |
| Class B | -6.90% | -14.15% | -12.10% | -7.52% | -15.40% | -14.41% | 124% | 8291% |
| Class C | -6.53% | -13.48% | -11.91% | -7.52% | -14.12% | -12.83% | 120% | 7846% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.31% | -13.11% | -11.32% | -8.03% | -14.14% | -12.69% | 122% | 8113% |
| Class D | -6.71% | -11.81% | -11.15% | -6.82% | -12.00% | -12.02% | 122% | 7597% |
| Class F | -3.82% | -7.53% | -6.07% | -5.01% | -10.67% | -9.12% | 131% | 3845% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -5.49% | -9.96% | -7.87% | -6.52% | -9.48% | -12.65% | 115% | 7876% |
| Class C | -5.32% | -11.84% | -9.04% | -6.95% | -10.18% | -9.07% | 113% | 6950% |
| Class E | -5.66% | -4.34% | -3.26% | -7.88% | -1.94% | -4.39% | 130% | 8302% |
| Overall | -5.48% | -9.18% | -7.11% | -7.00% | -7.83% | -9.39% | 118% | 7654% |
| Class D | -6.05% | -9.91% | -7.88% | -7.22% | -4.80% | -9.28% | 111% | 6863% |
| Class F | -3.04% | -6.16% | -2.21% | -5.15% | -6.27% | -7.41% | 123% | 3760% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -9.23% | -14.16% | -14.92% | -9.43% | -16.70% | -16.37% | 177% | 5844% |
| Class A2 | -7.53% | -13.55% | -10.64% | -8.25% | -14.07% | -10.25% | 174% | 5098% |
| Class B | -7.62% | -14.07% | -14.34% | -8.02% | -15.93% | -16.06% | 173% | 4941% |
| Class C | -7.45% | -13.40% | -13.11% | -8.33% | -16.12% | -15.73% | 164% | 3691% |
| Class E | -10.96% | -16.02% | -16.23% | -11.83% | -16.26% | -17.00% | 173% | 5427% |
| Overall | -8.39% | -14.17% | -13.86% | -9.00% | -15.85% | -15.23% | 172% | 4862% |
| Class D | -7.26% | -11.65% | -12.23% | -7.61% | -14.59% | -15.44% | 156% | 3266% |
| Class F | -5.20% | -9.48% | -8.66% | -5.59% | -12.06% | -11.70% | 136% | 4106% |

Note: Results from Interdigital, crosschecked by OPPO.

### NNVC-9.1 VTM mode vs NNVC-9.1 HOP.4

The NNVC-9.1 where LOP.3 filter is replaced by HOP.4 and Intra Prediction tools activated.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |  |
| Class A1 | -15.50% | -13.07% | -21.94% | -16.86% | -18.06% | -24.05% | 309% | 158621% |  |
| Class A2 | -14.88% | -19.05% | -25.60% | -14.34% | -18.43% | -23.26% | 305% | 148560% |  |
| Class B | -12.86% | -23.89% | -16.58% | -11.95% | -21.96% | -15.22% | 311% | 157039% |  |
| Class C | -13.88% | -17.91% | -16.95% | -12.80% | -14.69% | -14.09% | 235% | 119442% |  |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -14.06% | -19.16% | -19.55% | -13.63% | -18.54% | -18.29% | 287% | 144665% |  |
| Class D | -14.88% | -17.91% | -18.12% | -11.21% | -13.74% | -12.83% | 234% | 125200% |  |
| Class F | -8.92% | -12.64% | -11.21% | -9.49% | -12.53% | -11.40% | 430% | 74404% |  |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |  |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |  |
| Class B | -10.06% | -10.48% | -4.81% | -9.97% | -4.02% | -1.83% | 281% | 143293% |  |
| Class C | -12.09% | -9.21% | -5.43% | -11.14% | -3.23% | 2.09% | 233% | 121119% |  |
| Class E | -11.42% | -6.10% | -3.59% | -10.72% | -5.98% | -4.87% | 549% | 131867% |  |
| Overall | -11.07% | -8.96% | -4.71% | -10.55% | -4.25% | -1.28% | 313% | 132698% |  |
| Class D | -13.33% | -7.49% | -2.98% | -10.68% | 1.01% | 9.81% | 228% | 112300% |  |
| Class F | -9.11% | -10.04% | -7.67% | -10.25% | -6.42% | -8.11% | 405% | 81716% |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -13.50% | -11.49% | -19.38% | -15.11% | -16.89% | -21.60% | 360% | #NUM! |
| Class A2 | -13.07% | -17.24% | -21.11% | -13.81% | -18.17% | -18.36% | 272% | #NUM! |
| Class B | -11.80% | -17.15% | -14.49% | -11.73% | -17.33% | -14.85% | 253% | #NUM! |
| Class C | -12.82% | -14.55% | -16.57% | -12.58% | -13.97% | -15.75% | 192% | #NUM! |
| Class E | -16.51% | -20.23% | -19.51% | -16.28% | -19.24% | -20.02% | 264% | #NUM! |
| Overall | -13.31% | -16.16% | -17.71% | -13.59% | -16.97% | -17.62% | 257% | #NUM! |
| Class D | -12.10% | -13.61% | -17.02% | -11.17% | -13.72% | -15.80% | 171% | #NUM! |
| Class F | -9.63% | -13.10% | -12.71% | -9.43% | -13.31% | -13.59% | 166% | #NUM! |

Note: Results from InterDigital, crosschecked by OPPO.

### NNVC-9.1 VTM mode vs NNVC-9.1 VLOP

The NNVC-9.1 where LOP.3 filter is replaced by VLOP and Intra Prediction tools activated.

|  |  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |  |  |  |  |
| Class A1 | -6.10% | -4.74% | -5.50% | -6.36% | -4.97% | -5.27% | 118% | 3201% |
| Class A2 | -5.24% | -4.48% | -3.39% | -5.19% | -4.01% | -1.87% | 117% | 3008% |
| Class B | -5.05% | -5.92% | -5.74% | -5.30% | -5.49% | -4.97% | 118% | 3017% |
| Class C | -5.07% | -6.05% | -5.52% | -5.47% | -5.72% | -4.13% | 118% | 2800% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | -5.30% | -5.43% | -5.16% | -5.54% | -5.15% | -4.19% | 118% | 2991% |
| Class D |  | -5.39% |  | -5.88% |  | -6.18% |  | -5.27% |  | -4.80% |  | -4.98% |  | 116% |  | 2855% |  |  |  |  |
| Class F |  | -2.85% |  | -4.21% |  | -3.48% |  | -3.37% |  | -4.35% |  | -2.50% |  | 121% |  | 1174% |  |  |  |  |
| Class H |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |  |  |  |  |
|  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Y-PSNR |  | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  | DecT CPU |  |  |  |  |  |
| Class A1 |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |  |  |  |  |
| Class A2 |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  | #DIV/0! |  |  |  |  |  |
| Class B | -3.66% | -4.81% | -3.13% | -4.43% | -7.41% | -6.02% | 113% | 4079% |
| Class C | -3.74% | -4.15% | -2.68% | -4.77% | -6.62% | -3.88% | 112% | 3624% |
| Class E |  | -3.72% |  | 0.58% |  | -0.44% |  | -4.97% |  | -1.40% |  | -3.85% |  | 113% |  | 3580% |  |  |  |  |
| Overall |  | -3.70% |  | -3.24% |  | -2.31% |  | -4.68% |  | -5.64% |  | -4.76% |  | 113% |  | 3795% |  |  |  |  |
| Class D |  | -4.46% |  | -2.04% |  | -0.73% |  | -5.27% |  | -5.46% |  | -3.49% |  | 108% |  | 3429% |  |  |  |  |  |
| Class F |  | -1.90% |  | -1.33% |  | -0.89% |  | -2.84% |  | -2.04% |  | -3.15% |  | 117% |  | 1761% |  |  |  |  |  |

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra Main10 |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |  |
| Class A1 | -7.49% | -8.00% | -8.23% | -7.78% | -9.35% | -8.33% | 159% | 2970% |
| Class A2 | -6.26% | -7.29% | -6.16% | -6.41% | -6.81% | -4.63% | 165% | 2456% |
| Class B | -6.48% | -7.64% | -8.22% | -6.58% | -7.43% | -7.36% | 161% | 2549% |
| Class C | -6.37% | -7.86% | -7.91% | -6.89% | -8.04% | -7.10% | 160% | 2054% |
| Class E | -9.27% | -8.43% | -7.81% | -9.29% | -7.53% | -6.13% | 157% | 2823% |
| Overall | -7.05% | -7.82% | -7.74% | -7.27% | -7.80% | -6.80% | 160% | 2519% |
| Class D | -6.42% | -7.61% | -8.31% | -6.52% | -7.95% | -8.06% | 153% | 1943% |
| Class F | -4.39% | -6.21% | -5.97% | -4.40% | -5.66% | -4.67% | 134% | 1839% |

Note: Results from Qualcomm crosschecked by InterDigital, OPPO.

### NNVC-9.1 VTM vs NNVC-9.1 LOP2 adaptive content

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -10.05% | -10.93% | -15.36% | -11.89% | -12.13% | -14.22% | 236% | 6826% |
| Class A2 | -7.44% | -14.06% | -10.75% | -6.93% | -12.79% | -8.64% | 235% | 6185% |
| Class B | -7.85% | -17.68% | -15.71% | -6.12% | -16.58% | -13.56% | 236% | 7166% |
| Class C | -6.96% | -15.00% | -15.18% | -6.88% | -13.50% | -13.15% | 237% | 6142% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.97% | -14.89% | -14.51% | -7.64% | -14.11% | -12.60% | 236% | 6613% |
| Class D | -7.27% | -14.40% | -15.26% | -6.61% | -12.21% | -12.92% | 236% | 5540% |
| Class F | -4.08% | -10.03% | -10.20% | -4.25% | -10.91% | -10.91% | 255% | 3628% |

Note: Results from Nokia crosscheck by xxx,

### NNVC-9.1 VTM vs NNVC-9.1 RPR 2x

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 7.24% | -11.66% | -7.75% | -35.92% | -31.51% | -23.94% | 76% | 60% |
| Class A2 | -19.19% | -23.88% | -15.45% | -40.34% | -28.89% | -33.49% | 78% | 68% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -2.39% | -7.11% | -4.64% | -15.25% | -12.08% | -11.49% | 90% | 83% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Note: Results from InterDigital, crosschecked by xxx.

Please note that some rate-distortion curves are not monotonic and cause bad bdrate estimation.

### NNVC-9.1 VTM RPR 2x vs NNVC-9.1 NNSR

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1387.23% | -14.29% | -13.06% | -12.15% | 4.13% | -24.17% | 147% | 7636% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |

Please note that some rate-distortion curves are not monotonic and cause bad bdrate estimation.

Note: Results from InterDigital crosscheck by xxx,

## Comparison to previous NNVC anchor

### NNVC-8.0 anchor vs NNVC-9.1 anchor

The results reflect LOP/nnvc progress between NNVC 8.0 (as anchor) and NNVC 9.1 (as test).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.04% | -2.39% | -2.41% | -1.05% | -2.66% | -4.67% | 103% | 109% |
| Class A2 | -0.88% | 0.23% | 1.54% | -1.45% | -1.81% | -0.81% | 105% | 115% |
| Class B | -0.61% | -0.82% | 0.14% | -1.08% | -3.42% | -3.23% | 103% | 109% |
| Class C | 0.06% | 1.35% | 2.93% | -0.47% | -1.22% | 0.03% | 102% | 107% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.57% | -0.35% | 0.65% | -0.98% | -2.36% | -2.17% | 103% | 110% |
| Class D | 0.62% | 2.93% | 4.18% | -0.61% | -0.26% | 0.55% | 102% | 101% |
| Class F | -0.08% | 2.26% | 3.86% | -0.59% | -0.29% | 1.36% | 101% | 111% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -1.10% | 3.48% | 4.33% | -0.70% | 6.23% | 4.30% | 108% | 108% |
| Class C | 0.37% | 2.66% | 7.10% | 0.33% | 7.33% | 10.62% | 106% | 109% |
| Class E | -0.33% | 3.36% | 7.97% | -0.48% | 7.76% | 9.19% | 112% | 116% |
| Overall | -0.42% | 3.18% | 6.16% | -0.30% | 6.98% | 7.63% | 108% | 110% |
| Class D | 0.87% | 3.34% | 7.48% | 0.41% | 9.41% | 9.45% | 104% | 105% |
| Class F | 0.04% | 2.85% | 7.71% | -0.68% | 4.50% | 7.73% | 105% | 105% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-8.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.68% | -1.78% | -2.99% | -0.47% | -2.33% | -4.43% | 102% | 107% |
| Class A2 | -0.42% | -1.01% | 0.74% | -0.88% | -2.35% | -1.07% | 102% | 109% |
| Class B | -0.29% | -1.22% | -0.90% | -0.60% | -3.14% | -3.12% | 100% | 103% |
| Class C | 0.09% | 0.39% | 1.59% | -0.41% | -2.35% | -1.49% | 99% | 102% |
| Class E | -0.28% | -2.24% | -1.38% | -1.01% | -3.35% | -3.39% | 100% | 104% |
| Overall | -0.29% | -1.09% | -0.50% | -0.65% | -2.73% | -2.68% | 100% | 105% |
| Class D | 0.43% | 1.23% | 3.33% | -0.17% | -1.25% | -0.11% | 102% | 99% |
| Class F | 0.08% | 1.52% | 1.99% | -0.46% | -1.06% | -0.16% | 100% | 102% |

Note: Results from InterDigital, crosschecked by OPPO.

## Comparison to NNVC-9.1 anchor

### NNVC-9.1 vs NNVC-9.1 HOP.4

The results below reflect LOP/HOP performance differences (LOP set as anchor, HOP as test).

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -7.32% | -1.69% | -10.91% | -8.17% | -5.85% | -11.57% | 254% | 1901% |
| Class A2 | -7.92% | -7.75% | -19.12% | -6.71% | -6.17% | -16.82% | 254% | 1867% |
| Class B | -6.44% | -11.72% | -5.26% | -4.78% | -8.28% | -0.97% | 250% | 1894% |
| Class C | -7.88% | -5.13% | -5.77% | -5.70% | -0.63% | -1.43% | 196% | 1522% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -7.30% | -7.16% | -9.30% | -6.09% | -5.33% | -6.39% | 236% | 1783% |
| Class D | -8.77% | -6.96% | -8.00% | -4.70% | -1.95% | -1.19% | 192% | 1648% |
| Class F | -5.40% | -5.74% | -5.57% | -4.77% | -2.25% | -2.61% | 327% | 1935% |
| Class H | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -4.85% | -0.58% | 3.15% | -3.71% | 6.63% | 11.85% | 244% | 1819% |
| Class C | -7.16% | 2.91% | 3.66% | -4.53% | 7.72% | 11.87% | 207% | 1743% |
| Class E | -6.10% | -1.79% | -0.04% | -3.05% | -3.92% | -0.25% | 423% | 1588% |
| Overall | -5.94% | 0.28% | 2.52% | -3.82% | 4.35% | 8.83% | 265% | 1734% |
| Class D | -7.75% | 2.21% | 5.28% | -3.83% | 6.27% | 19.44% | 206% | 1636% |
| Class F | -6.31% | -4.23% | -5.47% | -5.40% | -0.20% | -0.92% | 330% | 2173% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -4.68% | 3.02% | -5.16% | -6.22% | -0.31% | -6.40% | 204% | 1902% |
| Class A2 | -6.01% | -3.84% | -11.56% | -6.02% | -4.25% | -8.88% | 156% | 1811% |
| Class B | -4.56% | -3.45% | -0.16% | -4.01% | -1.72% | 1.52% | 146% | 1809% |
| Class C | -5.87% | -1.24% | -4.07% | -4.63% | 2.73% | -0.06% | 117% | 1574% |
| Class E | -6.28% | -5.06% | -4.02% | -4.96% | -3.63% | -3.78% | 153% | 1726% |
| Overall | -5.40% | -2.22% | -4.41% | -5.01% | -1.24% | -2.77% | 150% | 1755% |
| Class D | -5.25% | -2.16% | -5.67% | -3.86% | 1.13% | -0.90% | 110% | 1514% |
| Class F | -4.76% | -4.01% | -4.46% | -4.06% | -1.37% | -2.16% | 123% | 1811% |

Note: Results from InterDigital, crosschecked by OPPO.

### NNVC-9.1 vs NNVC-9.1 VLOP

The results below reflect LOP/VLOP performance differences (LOP set as anchor, VLOP as test).

|  | Random access Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  |  | DecT CPU |
| Class A1 | 2.97% | 8.41% |  | 8.07% |  | 3.39% |  | 10.44% |  | 10.88% |  | 94% |  |  | 50% |
| Class A2 | 2.52% | 9.31% |  | 5.15% |  | 3.20% |  | 10.92% |  | 6.59% |  | 96% |  |  | 49% |
| Class B | 1.98% | 9.72% |  | 7.32% |  | 2.39% |  | 12.05% |  | 11.20% |  | 95% |  |  | 50% |
| Class C | 1.57% | 8.72% |  | 7.39% |  | 2.19% |  | 10.18% |  | 10.34% |  | 95% |  |  | 50% |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 2.18% | 9.11% |  | 7.05% |  | 2.70% |  | 11.00% |  | 9.99% |  | 95% |  |  | 50% |
| Class D | 1.41% | 6.80% |  | 5.70% |  | 1.64% |  | 8.37% |  | 8.06% |  | 95% |  |  | 49% |
| Class F | 1.03% | 3.79% |  | 2.84% |  | 1.76% |  | 7.63% |  | 7.75% |  | 91% |  |  | 43% |
| Class H | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR |  | V-PSNR |  | Y-MSIM |  | U-MSIM |  | V-MSIM |  | EncT |  |  | DecT CPU |
| Class A1 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #VALUE! |  | #DIV/0! |  |  | #DIV/0! |
| Class B | 1.94% | 5.77% |  | 5.15% |  | 2.25% |  | 2.38% |  | 7.78% |  | 98% |  |  | 52% |
| Class C | 1.67% | 8.76% |  | 7.14% |  | 2.35% |  | 4.00% |  | 6.14% |  | 99% |  |  | 52% |
| Class E | 2.05% | 5.07% |  | 3.09% |  | 3.09% |  | 0.67% |  | 0.73% |  | 87% |  |  | 43% |
| Overall | 1.88% | 6.59% |  | 5.29% |  | 2.49% |  | 2.50% |  | 5.47% |  | 95% |  |  | 50% |
| Class D | 1.68% | 8.61% |  | 7.64% |  | 2.11% |  | 0.30% |  | 6.77% |  | 97% |  |  | 50% |
| Class F | 1.19% | 5.27% |  | 1.50% |  | 2.50% |  | 4.69% |  | 4.63% |  | 95% |  |  | 47% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | 1.93% | 7.48% | 8.02% | 1.80% | 9.39% | 9.84% | 100% | 56% |
| Class A2 | 1.39% | 7.55% | 5.10% | 1.98% | 8.84% | 6.33% | 107% | 59% |
| Class B | 1.24% | 7.54% | 7.17% | 1.53% | 10.28% | 10.44% | 107% | 61% |
| Class C | 1.17% | 6.61% | 6.17% | 1.56% | 10.09% | 10.56% | 104% | 62% |
| Class E | 1.90% | 8.98% | 10.11% | 2.80% | 10.45% | 13.08% | 100% | 57% |
| Overall | 1.47% | 7.57% | 7.23% | 1.87% | 9.88% | 10.12% | 104% | 59% |
| Class D | 0.91% | 4.68% | 4.67% | 1.19% | 8.02% | 9.01% | 98% | 65% |
| Class F | 0.88% | 3.78% | 3.02% | 1.25% | 7.71% | 8.03% | 100% | 55% |

Note: Results from InterDigital, crosschecked by OPPO.

### NNVC-9.1 vs NNVC-9.1 LOP2 adaptive content

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 VTM |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -1.38% | 1.12% | -3.34% | -2.73% | 1.77% | 0.33% |  |  |
| Class A2 | 0.14% | -1.93% | -2.95% | 1.39% | 0.59% | -0.76% |  |  |
| Class B | -1.02% | -4.26% | -4.20% | 1.48% | -1.37% | 0.95% |  |  |
| Class C | -0.46% | -1.76% | -3.74% | 0.69% | 0.88% | -0.30% |  |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.71% | -2.05% | -3.65% | 0.41% | 0.25% | 0.15% |  |  |
| Class D | -0.60% | -2.98% | -4.63% | 0.22% | -0.21% | -1.09% |  |  |
| Class F | -0.29% | -2.82% | -4.53% | 0.58% | -0.66% | -2.37% |  |  |

Note: Results from Nokia crosschecked by xxxx.

### NNVC-9.1 RPR 2x vs NNVC-9.1 NNSR

Please note that some rate-distortion curves are not monotonic and cause bad bdrate estimation.

Note: Results from InterDigital crosscheck by xxx,

## Other tools

Other results remain the same as tools were not changed. Please note that VTM anchor and NNVC variants results changed due to MCTF improvement.

### NNVC-9.1-HOP3 vs NNVC-9.1-HOP4

These results show the progress of the HOP model, other aspects being the same.

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 HOP3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.35% | -3.58% | -3.71% | -0.67% | -4.90% | -3.91% | 117% | 125% |
| Class A2 | -0.13% | -5.84% | -16.86% | 0.01% | -6.38% | -17.41% | 121% | 127% |
| Class B | -0.44% | -12.24% | -4.14% | -0.21% | -11.18% | -2.66% | 121% | 129% |
| Class C | -0.48% | -0.42% | -0.07% | -0.22% | -0.96% | -0.48% | 108% | 113% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.37% | -6.07% | -5.51% | -0.26% | -6.24% | -5.28% | 117% | 123% |
| Class D | -0.22% | -1.43% | -0.53% | 0.24% | -2.16% | -0.18% | 108% | 110% |
| Class F | -0.36% | 0.35% | -0.23% | -0.42% | 0.58% | 0.57% | 122% | 126% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 HOP3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | 0.99% | -1.59% | 1.60% | 1.28% | 7.83% | 9.54% | 112% | 114% |
| Class C | 0.32% | 5.48% | 7.87% | 1.35% | 9.49% | 16.90% | 109% | 112% |
| Class E | 1.27% | 4.47% | 4.45% | 2.74% | 3.71% | 9.37% | 121% | 85% |
| Overall | 0.83% | 2.28% | 4.40% | 1.67% | 7.35% | 11.95% | 113% | 105% |
| Class D | 0.45% | 8.10% | 7.40% | 1.89% | 12.59% | 19.33% | 109% | 107% |
| Class F | -0.24% | 2.23% | 1.78% | -0.07% | 6.06% | 9.63% | 117% | 124% |

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 HOP3 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -0.73% | -2.83% | -4.12% | -0.84% | -3.96% | -4.72% | 114% | 135% |
| Class A2 | -0.64% | -7.19% | -13.62% | -0.49% | -8.45% | -12.83% | 110% | 137% |
| Class B | -0.73% | -8.21% | -4.42% | -0.68% | -7.38% | -3.67% | 110% | 141% |
| Class C | -0.61% | -1.69% | -1.55% | -0.71% | -1.46% | -1.49% | 101% | 137% |
| Class E | -0.83% | -5.77% | -5.28% | -0.33% | -6.29% | -5.68% | 108% | 140% |
| Overall | -0.70% | -5.29% | -5.41% | -0.62% | -5.49% | -5.22% | 108% | 138% |
| Class D | -0.30% | -1.80% | -1.72% | -0.27% | -1.97% | -2.97% | 99% | 138% |
| Class F | -0.82% | -1.57% | -2.78% | -0.94% | -1.58% | -2.20% | 103% | 139% |

Note: Results from InterDigital, crosschecked by OPPO.

# Contributions

We have 6 contributions for AhG14.

| JVET-AI0041 | [AHG11] [AHG14] Teleconference on NNVC |  | E. Alshina, F. Galpin |  |
| --- | --- | --- | --- | --- |
| JVET-AI0053 | AHG14: The extension of SADL library |  | N. Fu, W. Bao, J. Zhang, Z. Chen (Wuhan Univ.) |  |
| JVET-AI0057 | AHG14: SADL update |  | F. Galpin, F. Urban, T. Poirier (InterDigital) |  |
| JVET-AI0128 | AHG14: Update of Guidelines for NNVC software development |  | F. Galpin, E. Francois (InterDigital) |  |
| JVET-AI0155 | AhG14: Report of HOP3 training using additional dataset from JVET-AH0120 |  | F.galpin, T. Poirier (InterDigital), |  |
| JVET-AI0230 | AHG14: Fix for mismatch between encoding and decoding in the NNVC with palette on |  | Zhengang Li, Xing Zeng, Mneghu Jia, Zhen Zhang, Yonghua Wang, Jinchi Zhang, Cheng Huang(ZTE) |  |

# Configurations

The following configurations is used to generate the different NNVC results.

The column “tested” is read as follow:

- Y: the configuration has been tested using the new NNVC software

- P: the results are the ones from previous NNVC software basis

- N: not tested.

The column “xcheck” is read as follow:

- Y: the test has been cross-checked

- P: no cross-checked performed but results are consistent with previous version on NNVC

- N: no cross-check available

| Name | Tools | Configuration | Tested | Xcheck |
| --- | --- | --- | --- | --- |
| NNVC VTM mode | none | encoder_xxx_vtm.cfg | Y |  |
| NNVC Anchor/EE1 | Intra Pred + LOP.3 | encoder_xxx_nnvc.cfg | Y |  |
| NNVC. HOP | Intra Pred + HOP.4 | encoder_xxx_nnvc.cfg + nn-based/HOP4.cfg | Y |  |
| HOP only | HOP | encoder_xxx_vtm.cfg + nn-based/HOP4.cfg |  |  |
| Intra lower complexity | NN Intra lower complexity | nn-based/intra2.cfg |  |  |
| Sr | Super-resolution | nn-based/nnsr.cfg + nn-based/nnsr_classAx.cfg |  |  |
| Pf | Adaptive post-filters | nn-based/nnpf/nnpf_xxx.cfg |  |  |
| ALOP | Intra pred+adaptive LOP | encoder_xxx_vtm.cfg +nn-based/intra2.cfg+xxx |  |  |

Deprecated options:

| set0 | Loop filter set #0 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_1.cfg | P |  |
| --- | --- | --- | --- | --- |
| set1 | Loop filter set #1 | encoder_xxx_vtm.cfg + nn-based/NnlfOption_2.cfg | P |  |
| set0+rdo | Loop filter set #0 + Rdo | nn-based/NnlfOption_1.cfg + --EncNnlfOpt=1 | P |  |
| set1+rdo+intra+temporal filter | Loop filter set #1 + Temporal filter | nn-based/NnlfOption_2.cfg + --NnlfSet1Multiframe=1 | P |  |

# Recommendations

The AHG recommends to:

- Continue to develop NNVC software.

- Improve the software documentation.

- Encourage people to report all (potential) bugs that they are finding using GitLab Issues functionality https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/issues

Encourage people to submit merge requests fixing identified bugs.