| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 |  |  |  | Document: JVET-AI0023-v1 |
| --- | --- | --- | --- | --- |
| Title: | EE1: Summary report of exploration experiment on neural network-based video coding |  |  |  |
| Status: | input document to JVET |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | E. Alshina, R. Chang, F. Galpin, Y.Li, D. Rusanovskyy, M. Santamaria, J. Ström, and Z. Xie | Email: | elena.alshina@huawei.com renjiechang@tencent.com franck.galpin@interdigital.com yue.li@bytedance.com dmytror@qti.qualcomm.com maria.santamaria_gomez@nokia.com jacob.strom@ericsson.com xiezhihuang@oppo.com |  |
| Source: | EE Coordinators |  |  |  |

_____________________________

# Abstract

This report summarizes the activities of the Exploration Experiment 1 (EE1) performed between the 34th and 35th JVET meetings to evaluate Neural Network-based Video Coding (NNVC) technologies, analyze their performance, evaluate their complexity aspects, and clarify training procedure. Two teleconferences have been conducted during this meeting circle. With combination of two neural network-based tools gain over VTM  of 5.3%, 7.3% or 14% (depending on complexity of neural network based filter) can be achieved in random access configuration, with roughly 29, 49 and 483 kMAC/pxl computational complexity, 1.2...2.9 increase of encoding run time and 29, 49, 100 increase of decoding run time. Up to 1% additional compression gain can be achieved by adaptive resolution coding with neural-network based re-sampling. EE1 tests were targeting improving the performance complexity trade-off of for all three operation points of neural-network based filter, neural-network based Intra and super-resolution.

# Introduction

- It was decided 1  that the code base for the test should be NNVC-9.1. The anchor for all tests is default configuration of NNVC-9.1 (NN-Intra and LOP-3 filter enabled). Anchor performance results were provided by AhG 14 2 and been used by all proponents.

- For tests competing with technologies in NNVC-9.1 it was agreed to configure the proposed solution targeting close to existing NNVC tool complexity, but not exceeding it:

- kMAC/pxl of EE1 test  kMAC/pxl NNVC (must),

- Number of Parameters EE1 test   Number of Parameters NNVC (if possible).

NN architecture provided in this test description should not be changed, beside minor adjustment for parameters (such as channels number) in order to meet recommendation above.

Exact parameters settings were announced by proponents by 2nd AhG11/14 teleconference on June 18 3.

# List of tests

This round of EE1 tests includes:

- EE1-1: LOP and VLOP in-loop filters

  - EE1-1.1: LOP input adjustment with trainable components

    - JVET-AI0173 (Qualcomm)

  - EE1-1.2: Partial Convolution and Over-Parameterization

    - JVET-AI0068 (UESTC),  (Transsion)

  - EE1-1.3: On Low Complexity Operational Point for In-Loop Filtering

    - JVET-AI0221 (Qualcomm)

  - EE1-1.4: Content-adaptive VLOP-filter

    - JVET-AI0157, (Nokia),   (Qualcomm)

  - EE1-1.5: VLOP using LOP3 architecture

    - JVET-AI0107 (Ericsson),  (Qualcomm)

- EE1-2: HOP in-loop filter

  - EE1-2.1: On the detailed analysis of integer Transformer ILF with luma-chroma component balancing and complexity trade-off

    - JVET-AI0176  (Qualcomm)

  - EE1(AhG 11): On the complexity adjustment of HOP4

    - JVET-AI0172 Qualcomm),  (Interdigital),  (Bytedance)

- EE1-3: NN-inter prediction

  - withdrawn

- EE1-4: NN-super-resolution

  - EE1-4.1: Multiple scaling ratios coding for NNVC with NNSR

    - JVET-AI0074(vivo)

  - EE1-4.2: On simplified super resolution model

    - JVET-AI0093  (Bytedance)

- EE1-5: Neural network based Intra coding

  - EE1-5.1 : combination of the neural network-based intra prediction mode and ISP

    - JVET-AI0130 (Interdigital)

All neural network tools combinations tested are summarized in Table 1. Results in this table reported relatively to NNVC-9.1 configures as VTM.

Gain in MS-SSIM is 1-2% higher than in PSNR for NN-based tools.

Table 1 Summary of all test results vs NNVC (configured as VTM)

| Test | Random Access cfg BD-rate vs VTM |  |  |  |  | All Intra cfg BD-rate vs VTM |  |  |  |  | # tools combi-ned | Total kMAC /pxl | Total Num Param |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Enc | Dec | Y | U | V | Enc | Dec |  |  |  |
| NN-Intra & LOP NN-filter |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-9.1 | -7.3% | -13.1% | -11.3% | 1.2 | 81 | -8.4% | -14.2% | -13.9% | 1.7 | 49 | 2 | 24.8 | 1.72 |
| NNVC-8.0 | -6.9% | -13.2% | -12.1% | 1.2 | 73 | -8.1% | -13.3% | -13.4% | 1.7 | 44 | 2 | 24.9 | 1.55 |
| NNVC-7.1 | -6.9% | -13.2% | -12.1% | 1.3 | 86 | -8.1% | -13.3% | -13.4% | 1.8 | 56 | 2 | 24.9 | 1.55 |
| NN-Intra & HOP NN-filter |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-9.1 | -14.0% | -19.1% | -19.5% | 2.9 | 1446 | -13.3% | -16.2% | -17.7% | 2.6 | 853 | 2 | 483.6 | 3.0 |
| NNVC-8.0 | -13.7% | -13.9% | -14.5% | 2.5 | 1092 | -12.7% | -11.6% | -13.0% | 2.4 | 56 | 2 | 473.8 | 2.9 |
| NN-Intra & VLOP NN-filter |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NNVC-9.1 | -5.3% | -5.4% | -5.2% | 1.2 | 40 | -7.1% | -7.8% | -7.7% | 1.8 | 29 | 2 | 15.0 | 0.4 |
| NN-Intra & LOP NN-filter & Adaptive resolution coding NNVC-8.0 (*) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| SR RPR | -7.5% | -10.9% | -9.7% |  |  | -8.5% | -11.3% | -11.0% |  |  | 3 | 24.9 | 1.55 |
| NNSR | -7.8% | -11.9% | -10.5% |  |  | -8.6% | -11.8% | -11.3% |  |  | 3 | 45.2 | 1.63 |

Tabulated results are visualized ion Figure 1 and Figure 2.  Blue dots show combination of neural network-based filters with neural network based Intra. On Figure 1 we see that compression gain (BD-rate) for filters increases linearly with log of computational complexity.

Figure 1 BD-rate (Y) relatively to VTM anchor in random access (RA) configuration vs kMAC/pxl for different tools combinations.

Figure 2 BD-rate (Y) relatively to VTM anchor in random access (RA) configuration total number of parameters for different tools combinations.

More details about tested tools can be found in presentation attached to this report.

# Improvements for NNVC-LOP filter

| Test | Random Access |  |  | All Intra |  |  |  |  | kMAC/pxl |  |  |  | Param |  |  |  | Source |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V |  | Y |  | U | V |  | Total | Filter | Intra |  | Total | Filter | Intra |  |  |  |
| NNVC9.1 | 0.0% | 0.0% | 0.0% |  | 0.0% |  | 0.0% | 0.0% |  | 24.8 | 16.9 | 7.9 |  | 1.72 | 0.205 | 1.52 |  | JVET-AI0014 |  |
| EE1-1.1 | -0.1% | 0.5% | 0.7% |  | 0.0% |  | 0.0% | 0.5% |  | 26.8 | 16.9 | 7.9 |  | 1.72 | 0.205 | 1.52 |  | JVET-AI0173 |  |
| EE1-1.2.1 |  |  |  |  | 0.0% |  | 2.3% | 2.3% |  | 24.8 | 16.9 | 7.9 |  | 1.5 | 0.205 | 1.52 |  | JVET-AI0068 |  |
| EE1-1.2.2 |  |  |  |  | 0.0% |  | -1.2% | -1.7% |  | 24.8 | 16.9 | 7.9 |  | 1.5 | 0.205 | 1.52 |  | JVET-AI0068 |  |
| EE1-1.2.3 |  |  |  |  | 0.0% |  | -1.6% | -2.6% |  | 24.8 | 16.9 | 7.9 |  | 1.5 | 0.205 | 1.52 |  | JVET-AI0068 |  |
| EE1-1.2.3 (int) |  |  |  |  | 0.1% |  | -1.7% | -2.5% |  | 24.8 | 16.9 | 7.9 |  | 1.5 | 0.205 | 1.52 |  | JVET-AI0068 |  |
| EE1-1.3.a | -0.8% | 1.2% | 0.1% |  | -0.7% |  | 1.4% | 1.3% |  | 24.5 | 16.6 | 7.9 |  | 1.5 | 0.055 | 1.52 |  | JVET-AI0221 |  |

EE1-1.1 – trainable ‘DCT’ for LOP filter

EE1-1.2.1 – over parametrization (training hint)

EE1-1.2.2 – partial convolution (total number of convolutions 94  74)

EE1-1.2.3 – over parametrization and – partial convolution

EE1-1.3.a  – new architecture for LOP filter  (full float32 results made available on 09.07.2024).

# Improvements for NNVC-VLOP filter

| Test | Random Access |  |  | All Intra |  |  | kMAC/pxl |  |  | Param |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Total | Filter | Intra | Total | Filter | Intra |  |
| NNVC9.1 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 13.0 | 5.12 | 7.9 | 1.5 | 0.061 | 1.52 | JVET-AI0014 |
| EE1-1.3b | -0.7% | -1.5% | -1.2% | -0.6% | -1.5% | -1.3% | 13.1 | 5.18 | 7.9 | 1.5 | 0.012 | 1.52 | JVET-AI0221 |
| EE1-1.4 | -1.0% | -1.7% | -1.0% |  |  |  | 13.4 | 5.48 | 7.9 | 1.5 | 0.017 | 1.52 | JVET-AI0157 |
| EE1-1.5 | -0.2% | -1.6% | -0.7% | -0.1% | -1.4% | -0.9% | 13.1 | 5.16 | 7.9 | 1.5 | 0.0607 | 1.52 | JVET-AI0107 |

EE1-1.3.b  – new architecture for VLOP filter  (full float32 results made available on 09.07.2024).

EE1-1.4   – content adaptive VLOP.

EE1-1.5   – VLOP with LOP architecture

# Improvements for NNVC-HOP filter

| Test | Random Access |  |  | All Intra |  |  | kMAC/pxl |  |  | Param |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Total | Filter | Intra | Total | Filter | Intra |  |
| NNVC9.1 | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 0.0% | 484 | 476 | 7.9 | 2.96 | 1.439 | 1.52 | JVET-AI0014 |
| EE1-2.1.1 | -1.2% | 1.6% | 0.7% | -1.1% | 3.4% | 3.7% | 476 | 468 | 8 | 2.89 | 1.39 | 1.5 | JVET-AI0176 |
| EE1-2.1.4 | -0.1% | -2.9% | -3.3% | -0.1% | -2.2% | -1.3% | 473 | 465 | 8 | 2.85 | 1.35 | 1.5 | JVET-AI0176 |
| HOP4+ | -0.1% | -0.2% | -0.4% | -0.2% | 0.3% | 0.7% | 474 | 466 | 7.9 | 1.5 | 0.0607 | 1.52 | JVET-AI0172 |

EE1-2.1.1  – transformers in all BBBs.

EE1-2.1.2  – transformers in two BBBs.

‘HOP4+’ design is recommended during AhG11&14 teleconference

# Improvements for NNSR

| Test | Random Access |  |  | All Intra |  |  | kMAC/pxl |  |  | Param |  |  | Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | Y | U | V | Total | F&I | SR | Total | F&I | SR |  |
| NNVC-9.1 | -7.3% | -13.1% | -11.3% | -8.4% | -14.2% | -13.9% | 24.8 | 24.8 | 0 | 1.72 | 1.72 | 0 | JVET-AI0014 |
| NNVC-8.0 NNSR | -7.8% | -12.8% | -10.5% | -8.6% | -11.8% | -11.3% | 45.2 | 24.8 | 20.3 | 1.79 | 1.72 | 0.083 | JVET-AH0014 |
| NNVC-9.1-NNSR | - | - | - | - | - | - | 45.2 | 24.8 | 20.3 | 1.79 | 1.72 | 0.083 | JVET-AI0014 |
| EE1-4.1.1 (RPR) | -7.9% | -10.9% | -8.7% | -8.8% | -11.2% | -10.9% | 24.8 | 24.8 | 0 | 1.73 | 1.72 | 0 | JVET-AI0074 |
| EE1-4.1.2 (NNSR) | -8.1% | -10.3% | -8.9% | -9.2% | -11.5% | -11.0% | 29.5 | 24.8 | 4.7 | 1.78 | 1.72 | 0.050 | JVET-AI0074 |
| EE1-4.2 | -8.1% | -11.4% | -9.4% | -9.0% | -12.6% | -11.8% | 30.1 | 24.8 | 5.3 | 1.77 | 1.72 | 0.038 | JVET-AI0093 |

EE1-4.1.1 Encoder strategy for adaptive resolution (RPR filter for re-sampling)

EE1-4.1.2 same as EE1-4.1.1 but with NNSR filter (simpler than one in NNVC-9.1)

EE1-4.2 NNSR but simpler than one in NNVC-9.1.

BD-rate gain measured based on MS-SSIM for adaptive resolution coding is 2…2.2% higher compared to PSNR gain (both for RPR and NN-based filters)

# Improvements for NN-Intra

| Test |  |  | Random Access |  |  |  |  |  |  | All Intra |  |  |  |  | kMAC/pxl |  |  |  |  |  |  |  | Param |  |  |  |  |  |  | Source |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y |  |  | U |  | V |  | Y |  |  | U | V |  |  | Total |  | Filter |  | Intra |  | Total |  |  |  | Filter |  | Intra |  |  |  |  |  |
| NNVC9.1 |  | 0.0% |  |  | 0.0% |  | 0.0% |  | 0.0% |  |  | 0.0% | 0.0% |  |  | 24.8 |  | 16.9 |  | 7.9 |  | 1.72 |  |  |  | 0.205 |  | 1.52 |  |  | JVET-AI0014 |  |  |
| NNVC9.1-LCnnIntra |  | 0.1% |  |  | 0.0% |  | 0.0% |  | 0.4% |  |  | 0.3% | 0.3% |  |  | 21.7 |  | 16.9 |  | 4.8 |  | 1.53 |  |  |  | 0.205 |  | 1.33 |  |  | JVET-AI0014 |  |  |
| EE1-5.1 |  | -0.1% |  |  | -0.5% |  | -0.4% |  | -0.1% |  |  | -0.2% | -0.1% |  |  | 21.7 |  | 16.9 |  | 4.8 |  | 1.53 |  |  |  | 0.205 |  | 1.33 |  |  | JVET-AI0130 |  |  |

EE1-5.1 improved NN-INTRA with ISP design and better loss-function in training.

# SW location

https://vcgit.hhi.fraunhofer.de/jvet-ah-ee1/VVCSoftware_VTM/-/tree/EE1-X.X

# References

[1]  E. Alshina, F. Galpin, R. Chang, Y. Li, D. Rusanovskyy, M. Santamaria, J. Ström, Z. Xie  Exploration Experiments on Neural Network-based Video Coding (EE1)   JVET-AH2023.

[2]  F. Galpin (chair), R. Chang, Y. Li, Y. Li, J. N. Shingala, Z. Xie (vice-chairs) JVET AHG report: NNVC software development (AHG14) JVET-AI0014

[3]  E. Alshina, F. Galpin  [AHG11] [AHG14] Teleconference on NNVC JVET-AI0014