| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0050-v1 |
| --- | --- |

| Title: | EE2-4.1: Improvements on Inter LFNST/NSPT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhaolei.0324@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

In EE2 Test-4.1, two aspects are proposed to enhance inter LFNST/NSPT. In Test-4.1a, a multiple LFNST/NSPT kernel set selection method is applied to GPM-coded blocks. In Test-4.1b, inter LFNST/NSPT is enabled for SBT-coded blocks. It is reported that on top of ECM-13.0, simulation results of the proposed method are as below:

Test-4.1a: Multiple kernel set selection for inter LFNST/NSPT

RA: -0.02 % (Y), 0.07 % (U), -0.02 % (V), 100.2% (EncT), 99.7% (DecT)

LB: -0.06 % (Y), -0.01 % (U), -0.12% (V), 100.5% (EncT), 99.1% (DecT)

Test-4.1b: LFNST/NSPT for SBT-coded blocks

RA: -0.xx % (Y), -0.xx% (U), -0.xx % (V), xxx% (EncT), xxx% (DecT)

LB: -0.xx % (Y), 0.xx % (U), 0.xx % (V), xxx% (EncT), xxx% (DecT)

Test-4.1c: Test-4.1a + Test-4.1b

RA: -0.10 % (Y), 0.10 % (U), 0.05 % (V), 101.2% (EncT), 99.5% (DecT)

LB: -0.24 % (Y), 0.17 % (U), 0.05% (V), 101.9% (EncT), 99.1% (DecT)

- Introduction

In ECM-13.0, LFNST/NSPT can be applied to inter-coded blocks, where the transform kernels for intra LFNST/NSPT are reused by inter blocks. Based on the prediction signal of the current block, a Histogram of Gradients (HoG) is built in a similar way as DIMD, and the 1st DIMD intra prediction mode (IPM) corresponding to the highest amplitude is used to determine the LFNST/NSPT kernel set. It is asserted that a single IPM may not sufficiently profile a GPM-coded block. Besides, in ECM-13.0, inter LFNST/NSPT cannot be applied to an SBT-coded block.

- Proposed method

In Test-4.1, two aspects are investigated to enhance inter LFNST/NSPT:

Test-4.1a: Multiple LFNST/NSPT kernel set for GPM mode

For a GPM-coded block, an alternative IPM is derived for inter LFNST/NSPT. In addition to the 1st DIMD IPM, a second DIMD IPM corresponding to the 2nd highest HOG amplitude is used as an additional IPM candidate. A CU-level flag indicating the IPM index is signaled.

Test-4.1b: LFNST/NSPT for SBT-coded blocks

It is proposed to enable LFNST/NSPT for SBT-coded blocks. A CU-level LFNST/NSPT index is signaled for an SBT-coded block to indicate the usage of LFNST/NSPT. When LFNST/NSPT is applied to an SBT-coded CU, the TU with non-zero residual will perform LFNST/NSPT, and the prediction signal within the TU region is used to derive the IPM for transform.

- Experimental results

Three experiments are tested on top of ECM-13.0 under the common test conditions [1], as listed below:

Test-4.1a: Multiple LFNST/NSPT kernel set for GPM mode

Test-4.1b: LFNST/NSPT for SBT-coded blocks

Test-4.1c: Combination of Test-4.1a and Test-4.1b

Table 1: Coding performance of Test-4.1a

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.07% | -0.13% | 100.9% | 99.7% |  |  |  |  |  |
| Class A2 | -0.02% | 0.02% | 0.12% | 101.6% | 99.4% |  |  |  |  |  |
| Class B | 0.00% | 0.18% | -0.09% | 99.6% | 99.8% | -0.08% | -0.12% | -0.01% | 100.6% | 99.5% |
| Class C | -0.03% | -0.04% | 0.06% | 99.4% | 99.8% | -0.14% | -0.10% | -0.52% | 100.3% | 99.1% |
| Class E |  |  |  |  |  | 0.07% | 0.28% | 0.21% | 100.7% | 98.4% |
| Overall | -0.02% | 0.07% | -0.02% | 100.2% | 99.7% | -0.06% | -0.01% | -0.12% | 100.5% | 99.1% |
| Class D | -0.09% | 0.19% | 0.24% | 101.0% | 100.3% | -0.10% | -0.09% | -0.15% | 99.8% | 99.3% |
| Class F | -0.03% | 0.08% | 0.06% | 99.6% | 99.7% | 0.07% | 0.13% | 0.18% | 100.5% | 99.6% |

Table 1: Coding performance of Test-4.1b

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.06% | 0.08% | -0.02% | 101.6% | 99.7% | -0.14% | 0.11% | -0.23% | 101.9% | 100.1% |
| Class C | -0.07% | 0.10% | 0.14% | 99.9% | 99.8% | -0.05% | 0.85% | -0.35% | 101.7% | 98.0% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.08% | 0.14% | 0.10% | 103.8% | 100.0% | -0.16% | 0.27% | 0.19% | 98.6% | 98.8% |
| Class F | -0.02% | 0.01% | 0.04% | 102.1% | 99.8% |  |  |  |  |  |

Table 1: Coding performance of Test-4.1c

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.02% | 0.01% | 101.4% | 99.6% |  |  |  |  |  |
| Class A2 | -0.11% | 0.26% | 0.20% | 101.4% | 99.1% |  |  |  |  |  |
| Class B | -0.10% | 0.11% | -0.02% | 101.3% | 99.4% | -0.28% | 0.05% | 0.12% | 101.4% | 99.3% |
| Class C | -0.12% | 0.06% | 0.06% | 101.0% | 99.7% | -0.26% | 0.33% | 0.00% | 102.6% | 99.2% |
| Class E |  |  |  |  |  | -0.15% | 0.17% | 0.01% | 102.0% | 98.7% |
| Overall | -0.10% | 0.10% | 0.05% | 101.2% | 99.5% | -0.24% | 0.17% | 0.05% | 101.9% | 99.1% |
| Class D | -0.09% | 0.14% | 0.22% | 101.8% | 100.0% | -0.31% | 0.17% | 0.00% | 104.5% | 99.1% |
| Class F | -0.09% | 0.02% | -0.02% | 99.8% | 99.6% | -0.29% | 0.02% | -0.30% | 100.9% | 100.1% |

- Conclusions

This proposal reports the performance of EE2 Test-4.1. Simulation results indicate attractive coding gain in RA/LDB configurations with acceptable computational complexity. It is recommended to adopt this contribution in ECM.

- References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).