| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0056 |
| --- | --- |

| Title: | EE2-6.1 Entropy coding extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Le Leannec F. Lo Bianco C. Salmon-Legagneur M. Balcilar | Tel: Email: | franck.galpin@interdigital.com |
| Source: | InterDigital Inc. |  |  |

# Abstract

In contribution JVET-AH0108 and JVET-AG0085, it was proposed to add more flexibility to the bins coding in current ECM. First, some bins coded in EP are coded with an associated CABAC context. Second, an additional coding type is introduced in between the CABAC and the EP coding where the encoding uses a fixed probability without adaptation. The method is tested on some bins which were coded in EP in ECM-13.0 design, as well as together with the retraining of the original CABAC bins. It is reported that on top of ECM-13.0, the overall coding performance impact for {Y, U, V} is { -0.01% 0.07% -0.04%} {-0.04% 0.10% 0.02%} in AI and RA configurations respectively with original CABAC retraining. Allowing NEP model, and only NEP model for legacy EP, it is reported that on top of ECM-13.0, the overall coding performance impact for {Y, U, V} is {-0.02% -0.02% -0.10%} {-0.10% -0.06% -0.05%} in AI and RA configurations. Finally, by allowing CABAC or NEP for all elements, it is reported that on top of ECM-13.0, the overall coding performance impact for {Y, U, V} is { -0.05% -0.12% -0.17%} { -0.17% -0.09% -0.13%} in AI and RA configurations.

# Introduction

In current ECM, slice data can be coded either using CABAC, where a context is associated to the bin to encode or using EP coding.

In some cases, the adaptation of CABAC or the fixed probability of EP might not be optimal. It is proposed to add a mode NEP (Non Equi-Probable) where the probability depends on the syntax element to encode.

In order to choose when to use this mode, EP coded bins are changed in order to get an associated probability. The change does not impact the encoding and decoding of current ECM-13.0 but allows to choose which bins can use CABAC, EP or NEP.

# Simulations

In the tests, a training is done on the same training set: the dump of all ECM-13.0 CTC sequences for AI, RA, LDB, LDP, adding TGM sequences. The training is done for the 4 models: B, P, I, and L. As in JVET-AG0085, the L slice is used only for low-delay sequences.

Note: Enc/Dec timing are not reliable, but complexity is not expected to increase.

## Test 6.1a – Legacy CABAC training

In this test, the training of all legacy CABAC contexts is performed, using only the CABAC as model.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.11% | 0.02% | 100.8% | 100.5% | 96.9% | 102.0% |
| Class A2 | -0.01% | -0.01% | -0.01% | 100.0% | 100.4% | 100.2% | 101.8% |
| Class B | -0.01% | 0.03% | -0.10% | 101.2% | 100.2% | 100.4% | 100.9% |
| Class C | -0.02% | 0.09% | 0.01% | 100.5% | 100.6% | 101.9% | 100.2% |
| Class E | -0.02% | 0.15% | -0.07% | 100.8% | 101.0% | 100.8% | 100.5% |
| Overall | -0.01% | 0.07% | -0.04% | 100.7% | 100.5% | 100.2% | 101.0% |
| Class D | -0.02% | 0.09% | -0.21% | 101.2% | 101.4% | 100.0% | 100.1% |
| Class F | -0.06% | 0.26% | 0.08% | 100.1% | 100.9% | 101.1% | 100.4% |
| Class TGM | -0.04% | -0.06% | -0.06% | 99.6% | 101.1% | 100.7% | 101.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.04% | -0.12% | 102.2% | 100.5% | 100.2% | 99.8% |
| Class A2 | -0.04% | 0.15% | 0.07% | 101.9% | 100.3% | 100.4% | 99.8% |
| Class B | -0.02% | 0.14% | -0.04% | 101.4% | 100.6% | 100.0% | 99.9% |
| Class C | -0.06% | 0.11% | 0.15% | 100.8% | 100.6% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04% | 0.10% | 0.02% | 101.5% | 100.5% | 100.0% | 99.9% |
| Class D | -0.04% | -0.21% | 0.21% | 101.0% | 98.5% | 100.1% | 100.0% |
| Class F | -0.04% | -0.10% | 0.08% | 101.6% | 100.3% | 100.0% | 100.0% |
| Class TGM | -0.27% | -0.12% | 0.02% | 101.1% | 100.3% | 100.0% | 101.1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.57% | -0.14% | 102.2% | 100.0% | 100.2% | 100.1% |
| Class C | -0.08% | -0.55% | -0.43% | 101.4% | 99.6% | 100.0% | 100.1% |
| Class E | 0.20% | -1.43% | 1.62% | 101.0% | 100.1% | 99.9% | 100.2% |
| Overall | 0.02% | -0.78% | 0.20% | 101.6% | 99.9% | 100.0% | 100.1% |
| Class D | -0.07% | -0.15% | -0.35% | 100.8% | 99.9% | 99.5% | 100.0% |
| Class F | -0.31% | 0.65% | 0.20% | 101.4% | 100.3% | 100.6% | 99.9% |
| Class TGM | -0.23% | 0.16% | -0.05% | 102.0% | 99.5% | 100.2% | 99.2% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.02% | 0.45% | 0.51% | 101.0% | 100.4% | 100.0% | 100.1% |
| Class C | 0.00% | -0.08% | -0.16% | 100.1% | 100.7% | 100.0% | 100.0% |
| Class E | -0.03% | 0.58% | -0.48% | 101.0% | 100.5% | 100.0% | 99.9% |
| Overall | -0.01% | 0.31% | 0.04% | 100.7% | 100.5% | 100.0% | 100.0% |
| Class D | -0.11% | -0.08% | -1.53% | 100.7% | 99.6% | 99.8% | 100.0% |
| Class F | -0.39% | -0.07% | 0.38% | 101.9% | 100.2% | 100.2% | 99.9% |
| Class TGM | 0.03% | 0.00% | -0.03% | 100.7% | 100.6% | 100.4% | 100.0% |

## Test 6.1b – Adding NEP model

In this test, the training of all legacy CABAC contexts is performed, using both the CABAC and NEP models. Legacy EP bins are allowed to use the NEP model only.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.07% | -0.08% | 101.1% | 100.1% | 97.2% | 100.9% |
| Class A2 | -0.02% | -0.08% | -0.11% | 100.1% | 100.1% | 101.7% | 101.7% |
| Class B | 0.00% | -0.01% | -0.26% | 100.2% | 100.8% | 103.4% | 100.8% |
| Class C | -0.01% | 0.03% | 0.02% | 99.8% | 101.2% | 101.4% | 100.2% |
| Class E | -0.04% | 0.02% | 0.00% | 101.1% | 100.8% | 101.8% | 100.5% |
| Overall | -0.02% | -0.02% | -0.10% | 100.4% | 100.7% | 101.4% | 100.8% |
| Class D | -0.06% | 0.03% | -0.09% | 100.8% | 101.4% | 100.1% | 100.0% |
| Class F | -0.06% | 0.06% | -0.04% | 100.5% | 100.0% | 101.1% | 100.3% |
| Class TGM | -0.15% | -0.17% | -0.19% | 100.5% | 99.9% | 101.0% | 100.5% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.14% | -0.12% | -0.03% | 102.0% | 100.4% | 100.3% | 100.2% |
| Class A2 | -0.06% | 0.04% | -0.11% | 101.6% | 100.5% | 99.8% | 100.4% |
| Class B | -0.12% | -0.11% | -0.20% | 101.1% | 100.8% | 100.0% | 100.1% |
| Class C | -0.08% | -0.02% | 0.16% | 100.7% | 99.9% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.10% | -0.06% | -0.05% | 101.3% | 100.4% | 99.9% | 100.1% |
| Class D | -0.10% | -0.10% | 0.00% | 100.2% | 100.0% | 99.9% | 100.0% |
| Class F | -0.13% | 0.12% | -0.05% | 101.4% | 99.7% | 100.1% | 100.1% |
| Class TGM | -0.67% | -0.61% | -0.54% | 100.8% | 100.3% | 99.5% | 100.4% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.03% | -0.27% | 0.05% | 100.9% | 100.4% | 100.1% | 100.0% |
| Class C | -0.06% | -0.51% | -0.87% | 99.8% | 100.4% | 100.0% | 100.0% |
| Class E | 0.15% | -1.86% | 1.53% | 100.9% | 99.1% | 100.4% | 100.2% |
| Overall | 0.00% | -0.75% | 0.11% | 100.5% | 100.1% | 100.2% | 100.0% |
| Class D | -0.16% | 0.42% | 0.56% | 100.1% | 100.2% | 99.9% | 100.0% |
| Class F | -0.08% | 0.26% | 0.50% | 100.8% | 100.3% | 100.5% | 99.9% |
| Class TGM | -0.89% | -0.61% | -0.76% | 101.1% | 100.5% | 100.2% | 99.7% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.23% | 0.13% | 99.8% | 99.9% | 100.0% | 100.1% |
| Class C | 0.00% | -0.27% | -0.24% | 100.3% | 99.0% | 100.0% | 100.0% |
| Class E | -0.05% | -1.09% | -0.47% | 100.9% | 100.4% | 100.0% | 99.8% |
| Overall | 0.00% | -0.27% | -0.14% | 100.2% | 99.7% | 100.0% | 100.0% |
| Class D | 0.06% | 1.46% | -0.05% | 100.4% | 99.8% | 99.8% | 100.0% |
| Class F | -0.31% | -0.30% | 0.29% | 100.6% | 99.9% | 100.1% | 100.1% |
| Class TGM | -0.77% | -0.73% | -0.77% | 101.4% | 100.4% | 99.8% | 100.0% |

## Test 6.1c – Adding NEP model

In this test, the training of all legacy CABAC contexts is performed, using both the CABAC and NEP models. Legacy EP bins are allowed to use both CABAC and NEP model only.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.11% | -0.18% | 101.0% | 100.8% | 96.8% | 100.7% |
| Class A2 | -0.09% | -0.16% | -0.22% | 100.4% | 101.1% | 101.7% | 101.6% |
| Class B | -0.04% | -0.32% | -0.38% | 100.4% | 100.5% | 100.5% | 101.1% |
| Class C | -0.04% | -0.06% | 0.00% | 99.7% | 100.1% | 101.0% | 100.3% |
| Class E | -0.06% | 0.18% | 0.02% | 100.9% | 101.2% | 101.9% | 100.6% |
| Overall | -0.05% | -0.12% | -0.17% | 100.4% | 100.7% | 100.4% | 100.8% |
| Class D | -0.05% | 0.03% | 0.06% | 101.0% | 101.6% | 99.9% | 100.0% |
| Class F | -0.11% | 0.08% | -0.12% | 100.1% | 100.3% | 99.9% | 100.2% |
| Class TGM | -0.35% | -0.38% | -0.39% | 99.4% | 100.4% | 99.9% | 100.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.18% | -0.05% | -0.05% | 101.1% | 100.8% | 99.9% | 99.8% |
| Class A2 | -0.14% | -0.10% | -0.16% | 101.7% | 100.5% | 100.4% | 100.5% |
| Class B | -0.26% | -0.22% | -0.31% | 100.3% | 101.1% | 100.0% | 100.1% |
| Class C | -0.09% | 0.04% | 0.05% | 101.5% | 99.8% | 99.6% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.17% | -0.09% | -0.13% | 101.1% | 100.6% | 100.0% | 100.1% |
| Class D | -0.15% | 0.08% | 0.38% | 100.7% | 99.3% | 99.9% | 100.0% |
| Class F | -0.25% | -0.13% | -0.27% | 101.7% | 99.1% | 99.9% | 100.1% |
| Class TGM | -1.11% | -0.97% | -0.89% | 101.2% | 100.0% | 100.1% | 99.4% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.14% | -0.76% | -0.13% | 101.3% | 100.2% | 100.1% | 100.0% |
| Class C | -0.17% | -0.32% | -0.86% | 100.1% | 99.8% | 100.0% | 100.0% |
| Class E | 0.03% | -1.88% | 0.41% | 100.9% | 99.2% | 100.4% | 100.0% |
| Overall | -0.11% | -0.89% | -0.24% | 100.8% | 99.8% | 100.1% | 100.0% |
| Class D | -0.10% | 0.43% | -0.57% | 99.6% | 100.1% | 99.8% | 100.0% |
| Class F | -0.26% | 0.03% | 0.61% | 100.7% | 100.5% | 100.6% | 99.9% |
| Class TGM | -1.40% | -1.18% | -1.39% | 100.5% | 100.2% | 99.3% | 98.9% |
|  | Low delay P Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.10% | 0.02% | 0.21% | 100.0% | 100.4% | 100.0% | 100.1% |
| Class C | -0.06% | -0.14% | -0.24% | 99.6% | 100.6% | 100.0% | 100.1% |
| Class E | -0.15% | -0.52% | 0.04% | 100.0% | 99.7% | 100.0% | 100.1% |
| Overall | -0.10% | -0.17% | 0.02% | 99.9% | 100.3% | 100.0% | 100.1% |
| Class D | 0.04% | 0.30% | -0.27% | 99.8% | 100.3% | 99.6% | 100.0% |
| Class F | -0.45% | 0.21% | 0.20% | 101.3% | 100.5% | 100.3% | 100.0% |
| Class TGM | -1.27% | -1.20% | -1.32% | 100.2% | 100.6% | 99.8% | 100.1% |

# Contexts distribution analysis

## Models’ distribution

The models are distributed as follow:

|  | CABAC | NEP |
| --- | --- | --- |
| 6.1a | 6396 | 0 |
| 6.1b | 6264 | 1222 |
| 6.1c | 6918 | 703 |

## Models’ usage on all CTC

### Test 6.1a

| Configuration | QP | CABAC bins | NEP bins | EP bin | EP bins |
| --- | --- | --- | --- | --- | --- |
| RA | 22 | 86.1 | 0.0 | 11.9 | 1.9 |
| RA | 27 | 89.6 | 0.0 | 7.8 | 2.6 |
| RA | 32 | 90.5 | 0.0 | 6.5 | 3.0 |
| RA | 37 | 90.9 | 0.0 | 5.5 | 3.6 |
| RA | all | 87.7 | 0.0 | 10.0 | 2.3 |
| AI | 22 | 81.4 | 0.0 | 15.8 | 2.8 |
| AI | 27 | 85.8 | 0.0 | 10.5 | 3.7 |
| AI | 32 | 87.8 | 0.0 | 8.1 | 4.1 |
| AI | 37 | 89.4 | 0.0 | 6.3 | 4.3 |
| AI | all | 83.9 | 0.0 | 12.8 | 3.3 |
| LDB | 22 | 87.0 | 0.0 | 11.3 | 1.6 |
| LDB | 27 | 91.4 | 0.0 | 6.2 | 2.4 |
| LDB | 32 | 92.0 | 0.0 | 5.1 | 2.8 |
| LDB | 37 | 92.1 | 0.0 | 4.6 | 3.2 |
| LDB | all | 89.0 | 0.0 | 9.0 | 2.0 |
| ALL | 22 | 84.1 | 0.0 | 13.6 | 2.3 |
| ALL | 27 | 88.0 | 0.0 | 8.9 | 3.1 |
| ALL | 32 | 89.3 | 0.0 | 7.1 | 3.5 |
| ALL | 37 | 90.2 | 0.0 | 5.8 | 4.0 |
| ALL | all | 86.0 | 0.0 | 11.2 | 2.7 |

From the table, it is reported that about 11% of the bins are single EP bin and 3% are successive EP bins.

### Test 6.1b

| Configuration | QP | CABAC bins | NEP bins | EP bin | EP bins |
| --- | --- | --- | --- | --- | --- |
| RA | 22 | 84.8 | 1.6 | 12.9 | 0.7 |
| RA | 27 | 88.2 | 2.2 | 8.9 | 0.8 |
| RA | 32 | 89.0 | 2.6 | 7.6 | 0.9 |
| RA | 37 | 89.3 | 3.0 | 6.6 | 1.1 |
| RA | all | 86.3 | 1.9 | 11.0 | 0.8 |
| AI | 22 | 80.5 | 1.8 | 16.1 | 1.5 |
| AI | 27 | 84.8 | 2.0 | 11.3 | 2.0 |
| AI | 32 | 86.7 | 2.2 | 9.1 | 2.0 |
| AI | 37 | 88.1 | 2.5 | 7.5 | 1.9 |
| AI | all | 82.9 | 1.9 | 13.4 | 1.7 |
| LDB | 22 | 85.4 | 1.3 | 13.0 | 0.2 |
| LDB | 27 | 89.5 | 2.2 | 8.0 | 0.2 |
| LDB | 32 | 90.0 | 3.0 | 6.8 | 0.2 |
| LDB | 37 | 89.9 | 3.5 | 6.4 | 0.2 |
| LDB | all | 87.2 | 1.9 | 10.7 | 0.2 |
| ALL | 22 | 82.9 | 1.6 | 14.4 | 1.0 |
| ALL | 27 | 86.7 | 2.1 | 9.9 | 1.3 |
| ALL | 32 | 88.0 | 2.4 | 8.3 | 1.4 |
| ALL | 37 | 88.7 | 2.8 | 7.1 | 1.4 |
| ALL | all | 84.8 | 1.9 | 12.1 | 1.2 |

From the table, it is reported that about 2% are NEP coded, 12% of the bins are single EP bin and 1% are successive EP bins. The ratio of CABAC bins is slightly decreased to 85%.

### Test 6.1c

| Configuration | QP | CABAC bins | NEP bins | EP bin | EP bins |
| --- | --- | --- | --- | --- | --- |
| RA | 22 | 88.1 | 0.4 | 10.8 | 0.6 |
| RA | 27 | 92.0 | 0.6 | 6.7 | 0.8 |
| RA | 32 | 92.8 | 0.6 | 5.7 | 0.9 |
| RA | 37 | 93.5 | 0.7 | 4.6 | 1.1 |
| RA | all | 89.9 | 0.5 | 8.8 | 0.7 |
| AI | 22 | 83.4 | 0.2 | 14.9 | 1.5 |
| AI | 27 | 87.8 | 0.2 | 10.0 | 1.9 |
| AI | 32 | 90.0 | 0.1 | 7.9 | 2.0 |
| AI | 37 | 91.8 | 0.1 | 6.2 | 1.9 |
| AI | all | 86.0 | 0.2 | 12.1 | 1.7 |
| LDB | 22 | 88.5 | 0.3 | 10.9 | 0.3 |
| LDB | 27 | 93.2 | 0.4 | 6.1 | 0.2 |
| LDB | 32 | 95.2 | 0.4 | 4.2 | 0.2 |
| LDB | 37 | 96.2 | 0.6 | 3.0 | 0.2 |
| LDB | all | 91.5 | 0.4 | 7.9 | 0.3 |
| ALL | 22 | 85.7 | 0.3 | 12.9 | 1.1 |
| ALL | 27 | 89.8 | 0.4 | 8.4 | 1.4 |
| ALL | 32 | 91.6 | 0.4 | 6.7 | 1.4 |
| ALL | 37 | 92.9 | 0.4 | 5.3 | 1.4 |
| ALL | all | 88.0 | 0.3 | 10.5 | 1.2 |

From the table, it is reported that about 0.3% are NEP coded, 10% of the bins are single EP bin and 1% are successive EP bins. The ratio of CABAC bins is slightly increased to 88%.

# Conclusion

This contribution proposes to add more flexibility to the current bin coding in ECM. The overall coding performance impact for {Y, U, V} is { -0.05% -0.12% -0.17%} { -0.17% -0.09% -0.13%} in AI and RA configurations, with minor impact on CABAC/EP bins distribution. It is proposed to adopt the tool in next version of ECM.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).