| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0058-v1 |
| --- | --- |

| Title: | EE2-5.1: Modifications in ALF syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guillaume Boisson Franck Galpin Philippe Bordes | Email: | guillaume.boisson@interdigital.com franck.galpin@interdigital.com philippe.bordes@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Adaptive Loop Filter (ALF) syntax is twofold: ALF filters are signalled in Adaptation Parameter Sets (APSs) while ALF control syntax elements are signalled at CTU level. In this contribution, the results of EE2 Test 5.1 are reported, which investigates the impact of modifying ALF syntax both at APS and CTU levels, and the impact of an encoder-only change in ALF RDO. In test 5.1a, the Lagrangian multiplier that balances rate and distortion in ALF RDO is modified. In test 5.1b, ALF APS syntax is modified with the introduction of exponential-Golomb order Look-up Tables (LUTs) and predictive coding of Luma filter indices. In test 5.1c, most probable candidates for ALF CTU-level control syntax elements are created based on neighbouring CTUs. In test 5.1d, the combination of tests 5.1b and 5.1c is tested. In test 5.1e, the combination of tests 5.1a and 5.1d is tested.

On top of ECM-13.0 with the common test conditions, simulation results of EE2-5.1 are reported as follows.

Test EE2-5.1a:

|  | Y-BDR | U-BDR | V-BDR | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Random access |  |  |  |  |  |  |  |
| Low-delay B | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |

Test EE2-5.1b:

|  | Y-BDR | U-BDR | V-BDR | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0,01% | -0,01% | -0,01% | 99,9% | 99,8% | 100,5% | 100,0% |
| Random access | -0,02% | -0,01% | 0,01% | 99,9% | 100,0% | 99,9% | 100,1% |
| Low-delay B | 0,00% | -0,22% | -0,26% | 100,0% | 99,9% | 100,2% | 100,1% |

Test EE2-5.1c:

|  | Y-BDR | U-BDR | V-BDR | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | 0,00% | 0,01% | 0,01% | 99,8% | 99,7% | 100,2% | 100,0% |
| Random access |  |  |  |  |  |  |  |
| Low-delay B |  |  |  |  |  |  |  |

Test EE2-5.1d:

|  | Y-BDR | U-BDR | V-BDR | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0,01% | 0,01% | 0,00% | 99,9% | 100,0% | 100,4% | 100,0% |
| Random access |  |  |  |  |  |  |  |
| Low-delay B |  |  |  |  |  |  |  |

Test EE2-5.1e:

|  | Y-BDR | U-BDR | V-BDR | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0,01% | 0,01% | 0,00% | 99,9% | 100,0% | 100,4% | 100,0% |
| Random access |  |  |  |  |  |  |  |
| Low-delay B |  |  |  |  |  |  |  |

# Introduction

ALF Luma and Chroma filters are signalled in ALF Adaptation Parameter Sets (APSs). ALF filter coefficients are encoded with a sign bit and exponential-Golomb coding of their absolute value. In ECM-13.0, two exp-Golomb orders are determined at encoder side and signalled prior to the filter. The first exp-Golomb order is used to encode first filter coefficients, and the second exp-Golomb order is used to encode last filter coefficients.

Signalled ALF Luma filters, in addition, rely on a 25-class classification. Signalled ALF Luma filters are therefore gathered in Luma filter sets, a Luma filter set consisting of up to 25 filters together with a mapping list that specifies which class uses which filter. In ECM-13.0, the corresponding Luma filter indices are encoded independently.

An ALF APS can contain up to 4 Luma filter sets and up to 8 Chroma filters. Up to eight ALF APSs can be referenced at the same time.

At CTU level, flags signal whether signalled ALF Luma and Chroma filters are used or not. If applicable, further indices signal which APS, which Luma filter set, and which Chroma filter are used. In ECM-13.0, APS indices, Luma filter set indices, and Chroma filter indices are coded independently from a CTU to another.

At encoder side, the decision to transmit a new ALF APS is taken at slice level, based on a rate-distortion criterion that balances corresponding gains and costs. In ECM-13.0, ALF RDO uses current slice’s regular lambda values.

In JVET-AH0163, ALF syntax is modified both at APS and CTU levels, and ALF RDO criterion is relaxed to let more filters be signalled in the bitstream.

# Proposed method

EE2 Test 5.1 investigates the impact of modifying ALF syntax at APS and CTU levels, and the impact of modifying ALF RDO criterion.

In Test 5.1a, ALF RDO criterion (D+λR) is relaxed using a decreased lambda value for Luma component. That modification is limited to frames with temporal ID higher than zero. Test 5.1a has therefore no impact with respect to anchors in All intra and Low-delay configurations.

In Test 5.1b, ALF APS syntax is modified regarding the coding of filter coefficients and the coding of Luma filter indices, as follows.

Regarding filter coefficients, the two exp-Golomb orders signalled are replaced by the index of a Look-up Table (LUT) that specifies an exp-Golomb order for each coefficient. Exp-Golomb order LUTs depend on filter coefficients precision.

Regarding Luma filter indices, a filter index predictor is introduced that consists in the value of the previous filter index in the mapping list. The predictor for the first index in the list is set to zero.

In Test 5.1c, ALF CTU-level syntax is modified as follows: APS indices and filter indices are signalled from a list of indices from the neighbouring CTUs.

In Test 5.1d, a combination of tests 5.1b and 5.1c is tested.

In Test 5.1e, a combination of tests 5.1a and 5.1d is tested.

# Simulation results

The proposed method was implemented on top of ECM-13.0 and tested following the common test conditions described in JVET-AF2017. Following tables 1 to 5 report the simulation results of EE2 tests 5.1a to 5.1e, respectively.

Table 1 – Results of Test 5.1a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class A2 | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class B | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class C | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class E | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Overall | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class D | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class F | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class TGM | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | 0,00% | 0,23% | 0,20% | 100,1% | 100,1% | 100,0% | 99,9% |
| Class C | -0,09% | 0,19% | 0,17% | 99,9% | 100,1% | 99,5% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,14% | 0,08% | 0,14% | 100,2% | 100,6% | 100,1% | 100,0% |
| Class F | -0,02% | 0,19% | 0,10% | 99,9% | 100,2% | 100,2% | 100,0% |
| Class TGM | 0,04% | 0,18% | 0,20% | 99,6% | 100,3% | 99,5% | 98,6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class C | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class E | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Overall | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class D | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class F | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |
| Class TGM | 0,00% | 0,00% | 0,00% | 100,0% | 100,0% | 100,0% | 100,0% |

Table 2 – Results of Test 5.1b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,01% | -0,01% | -0,01% | 100,1% | 99,9% | 101,8% | 100,1% |
| Class A2 | 0,00% | -0,02% | 0,00% | 100,2% | 99,0% | 100,4% | 100,1% |
| Class B | -0,01% | 0,00% | -0,01% | 100,1% | 99,8% | 99,8% | 99,9% |
| Class C | -0,01% | 0,00% | 0,00% | 99,8% | 99,7% | 100,0% | 100,0% |
| Class E | -0,01% | 0,00% | -0,02% | 99,4% | 100,5% | 101,2% | 100,0% |
| Overall | -0,01% | -0,01% | -0,01% | 99,9% | 99,8% | 100,5% | 100,0% |
| Class D | -0,01% | -0,05% | -0,09% | 100,1% | 100,5% | 99,9% | 100,0% |
| Class F | 0,00% | 0,00% | -0,01% | 100,3% | 99,8% | 100,8% | 99,9% |
| Class TGM | 0,00% | 0,00% | 0,00% | 99,2% | 100,0% | 100,0% | 99,9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,04% | -0,01% | 0,01% | 99,7% | 99,9% | 100,1% | 100,0% |
| Class A2 | -0,04% | -0,16% | 0,02% | 99,1% | 100,0% | 99,9% | 100,2% |
| Class B | -0,02% | 0,01% | -0,02% | 100,1% | 100,1% | 100,0% | 100,0% |
| Class C | 0,00% | 0,06% | 0,03% | 100,5% | 100,0% | 99,6% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,02% | -0,01% | 0,01% | 99,9% | 100,0% | 99,9% | 100,1% |
| Class D | -0,04% | 0,04% | 0,15% | 100,2% | 99,8% | 99,9% | 100,0% |
| Class F | 0,02% | 0,08% | 0,06% | 99,3% | 100,1% | 100,1% | 100,0% |
| Class TGM | -0,03% | 0,11% | 0,03% | 100,7% | 100,5% | 99,7% | 99,5% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,03% | -0,52% | -0,38% | 99,8% | 100,0% | 100,1% | 100,1% |
| Class C | -0,04% | -0,03% | -0,38% | 100,6% | 99,9% | 100,0% | 100,1% |
| Class E | 0,08% | 0,04% | 0,11% | 99,4% | 99,9% | 100,6% | 100,1% |
| Overall | 0,00% | -0,22% | -0,26% | 100,0% | 99,9% | 100,2% | 100,1% |
| Class D | -0,02% | -0,85% | -0,74% | 99,5% | 100,0% | 99,8% | 100,0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

Table 3 – Results of Test 5.1c

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,01% | 0,01% | -0,02% | 99,5% | 99,9% | 101,1% | 100,0% |
| Class A2 | -0,01% | 0,04% | 0,03% | 99,4% | 99,7% | 100,7% | 100,1% |
| Class B | 0,00% | 0,01% | 0,02% | 99,8% | 99,7% | 99,6% | 99,9% |
| Class C | 0,00% | 0,00% | 0,00% | 100,1% | 99,3% | 100,0% | 100,0% |
| Class E | 0,00% | 0,00% | 0,00% | 100,0% | 99,9% | 100,0% | 100,0% |
| Overall | 0,00% | 0,01% | 0,01% | 99,8% | 99,7% | 100,2% | 100,0% |
| Class D | 0,00% | 0,00% | 0,00% | 100,5% | 99,8% | 99,9% | 100,0% |
| Class F | 0,00% | 0,00% | 0,00% | 100,3% | 99,9% | 100,0% | 99,8% |
| Class TGM | -0,04% | -0,02% | 0,01% | 100,8% | 99,9% | 102,5% | 100,0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,05% | -0,09% | -0,08% | 100,1% | 100,3% | 100,0% | 99,9% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0,05% | 0,02% | -0,08% | 101,0% | 99,8% | 99,9% | 100,1% |
| Class C | 0,01% | -0,02% | -0,06% | 99,3% | 99,4% | 99,6% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,04% | 0,07% | 0,05% | 100,2% | 99,8% | 99,9% | 100,0% |
| Class F | 0,03% | 0,06% | -0,02% | 100,2% | 100,0% | 100,1% | 100,0% |
| Class TGM | -0,13% | -0,06% | -0,04% | 100,8% | 100,3% | 99,8% | 98,6% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0,05% | -0,15% | -0,40% | 100,1% | 99,6% | 100,0% | 100,0% |
| Class E | 0,10% | -0,22% | -0,31% | 100,0% | 100,2% | 99,9% | 100,0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,07% | 0,20% | -0,89% | 100,0% | 99,8% | 99,8% | 100,0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

Table 4 – Results of Test 5.1d

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,01% | 0,01% | -0,02% | 99,8% | 99,8% | 100,9% | 100,0% |
| Class A2 | -0,01% | 0,03% | 0,01% | 100,0% | 100,3% | 101,6% | 100,1% |
| Class B | -0,01% | 0,01% | 0,02% | 99,8% | 99,9% | 99,9% | 100,0% |
| Class C | -0,01% | 0,00% | 0,00% | 99,8% | 100,2% | 100,0% | 100,0% |
| Class E | -0,01% | 0,00% | -0,02% | 100,3% | 99,9% | 100,0% | 100,0% |
| Overall | -0,01% | 0,01% | 0,00% | 99,9% | 100,0% | 100,4% | 100,0% |
| Class D | -0,01% | -0,05% | -0,10% | 100,1% | 100,3% | 99,9% | 100,0% |
| Class F | -0,01% | -0,01% | -0,02% | 100,0% | 100,5% | 100,0% | 99,9% |
| Class TGM | -0,04% | -0,02% | 0,01% | 100,0% | 100,1% | 102,6% | 99,9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0,08% | -0,03% | -0,13% | 100,3% | 100,0% | 99,9% | 100,1% |
| Class C | -0,01% | 0,01% | 0,01% | 100,2% | 100,0% | 99,6% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,07% | 0,08% | 0,18% | 100,3% | 100,2% | 99,9% | 100,0% |
| Class F | 0,00% | 0,01% | -0,02% | 100,1% | 100,4% | 100,0% | 99,9% |
| Class TGM | -0,13% | -0,04% | 0,01% | 99,4% | 99,8% | 99,3% | 99,2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0,06% | 0,01% | -0,35% | 100,3% | 100,0% | 100,0% | 99,9% |
| Class E | 0,11% | 0,21% | 0,19% | 99,7% | 100,0% | 100,0% | 100,0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,14% | -0,72% | 0,08% | 100,2% | 99,9% | 99,5% | 100,0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

Table 5 – Results of Test 5.1e

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,01% | 0,01% | -0,02% | 99,8% | 99,8% | 100,9% | 100,0% |
| Class A2 | -0,01% | 0,03% | 0,01% | 100,0% | 100,3% | 101,6% | 100,1% |
| Class B | -0,01% | 0,01% | 0,02% | 99,8% | 99,9% | 99,9% | 100,0% |
| Class C | -0,01% | 0,00% | 0,00% | 99,8% | 100,2% | 100,0% | 100,0% |
| Class E | -0,01% | 0,00% | -0,02% | 100,3% | 99,9% | 100,0% | 100,0% |
| Overall | -0,01% | 0,01% | 0,00% | 99,9% | 100,0% | 100,4% | 100,0% |
| Class D | -0,01% | -0,05% | -0,10% | 100,1% | 100,3% | 99,9% | 100,0% |
| Class F | -0,01% | -0,01% | -0,02% | 100,0% | 100,5% | 100,0% | 99,9% |
| Class TGM | -0,04% | -0,02% | 0,01% | 100,0% | 100,1% | 102,6% | 99,9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0,09% | 0,25% | 0,19% | 100,0% | 100,1% | 100,0% | 100,0% |
| Class C | -0,08% | 0,18% | 0,07% | 100,2% | 100,0% | 100,0% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,17% | 0,09% | 0,16% | 99,9% | 99,9% | 99,9% | 100,0% |
| Class F | 0,02% | 0,19% | 0,08% | 99,5% | 100,1% | 99,9% | 100,1% |
| Class TGM | -0,10% | 0,13% | 0,12% | 99,6% | 100,0% | 99,0% | 99,1% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | -0,06% | 0,01% | -0,35% | 100,3% | 100,0% | 100,0% | 99,9% |
| Class E | 0,11% | 0,21% | 0,19% | 99,7% | 100,0% | 100,0% | 100,0% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,14% | -0,72% | 0,08% | 100,2% | 99,9% | 99,5% | 100,0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

# Conclusion

This contribution reports the results of EE2 Test 5.1. It is recommended to adopt Test 5.1e.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).