| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0059-v1 |
| --- | --- |

| Title: | AHG9: Comments on Text Description Information SEI Message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed related to the Text Description Information SEI message.

## It is proposed to define the persistence scope for the Text Description Information SEI to be purpose specific. For this, it is proposed to move the signaling of txt_descr_purpose syntax element to an earlier position. Also, it is proposed to modify the semantics of txt_cancel_flag to apply to txt_descr_purpose, to move txt_descr_id to a later position and to include a new txt_id_cancel_flag.

# Introduction

The VSEI V4 draft 2 [1] includes a text description information SEI.

Following is proposed related to the Text Description Information SEI message.

## It is proposed to define the persistence scope for the Text Description Information SEI to be purpose specific. For this, it is proposed to move the signaling of txt_descr_purpose syntax element to an earlier position. Also, it is proposed to modify the semantics of txt_cancel_flag to apply to txt_descr_purpose, to move txt_descr_id to a later position and to include a new txt_id_cancel_flag.

# Proposal

It is proposed to define the persistence scope for the Text Description Information SEI to be purpose specific.

## It is proposed to move the signaling of txt_descr_purpose syntax element to an earlier position. It is asserted that this allows entities interested in only certain text_descr_purpose values (e.g. copyright or AI marking) to easily identify those SEIs (e.g. by parsing the first syntax element) without having to do conditional parsing.

## It is proposed to have a cancel flag which cancels persistence of all previous SEIs which have the same txt_descr_purpose as the current value of the txt_descr_purpose in the SEI message. It is asserted that this helps the splicing use case, where a new SEI can be inserted after the splice point and cancel the persistence of all previous text description information SEIs having the same txt_descr_purpose, irrespective of the txt_descr_id value of those previous SEIs. For this, semantics of txt_cancel_flag are modified to apply to txt_descr_purpose, txt_descr_id is moved to a later position and a new txt_id_cancel_flag is proposed.

## The proposed modifications are shown on top of JVET-AH2006 [1].

  - Text description information SEI

    - Text description information SEI message syntax

| text_description( payloadSize ) { | Descriptor |
| --- | --- |
| txt_descr_purpose | u(8) |
| txt_descr_id | u(14) |
| txt_cancel_flag | u(1) |
| if( !txt_cancel_flag ) { |  |
| txt_descr_id | u(14)u(13) |
| txt_id_cancel_flag | u(1) |
| if( !txt_id_cancel_flag ) { |  |
| txt_persistence_flag | u(1) |
| txt_descr_purpose | u(8) |
| txt_num_strings_minus1 | u(8) |
| for( i = 0; i <= txt_num_strings_minus1; i++ ) { |  |
| txt_descr_string_lang[ i ] | st(v) |
| txt_descr_string[ i ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |

    - Text description information SEI message semantics

The text description information SEI message provides text description about one or more pictures.

txt_descr_purpose indicates the purpose of the text description SEI as specified in Table xx.

Table xx – Definition of txt_descr_purpose

| Value | Interpretation |
| --- | --- |
| 0 | Application defined |
| 1 | Copyright information |
| 2 | AI marking information |
| 3 | General comment information |
| 4 | Content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) |
| 5 | Tag URI for identifying the bitstream |
| 6-255 | Reserved |

txt_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_id txt_descr_purpose in output order that applies to the current layer. txt_cancel_flag equal to 0 indicates that text description information follows.

txt_descr_id indicates the identifier value of this text description information SEI message. The value of txt_descr_id shall be in the range of 1 to 16383, inclusive. Value 0 is reserved.

txt_id_cancel_flag equal to 1 indicates that the text description information SEI message cancels the persistence of any previous text description information SEI message with the same txt_descr_id value as the current SEI in output order that applies to the current layer. txt_id_cancel_flag equal to 0 indicates that text description information syntax elements (txt_persistence_flag, txt_num_strings_minus1, txt_descr_string_lang[ i ] , txt_descr_string[ i ]  ) follow.

txt_persistence_flag specifies the persistence of the text information description SEI message for the current layer.

txt_persistence_flag equal to 0 specifies that the text description information applies to the current decoded picture only.

txt_persistence_flag equal to 1 specifies that the text description information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a text description information SEI message with the same txt_descr_id is output that follows the current picture in output order.

txt_num_strings_minus1 plus 1 indicates the number of entries for txt_descr_string_lang[ i ] and txt_descr_string[ i ] that follow.

txt_descr_string_lang[ i ] specifies the language of the txt_descr_string[ i ]. The language of the txt_descr_string[ i ]  shall be given by a language tag as defined by IETF RFC 5646 [2]. The length of txt_descr_string_lang[ i ]  shall be in the range of 0 to 49, inclusive.

txt_descr_string[ i ] specifies i-th text description information string whose value is interpreted as specified by the txt_descr_purpose.

When txt_descr_purpose is equal to 0, the interpretation of what information is conveyed in the txt_descr_string is application-defined.

When txt_descr_purpose is equal to 1, the txt_descr_string[ i ] specifies copyright information that pertains to the picture(s) in the persistence scope of this SEI message defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 2, the txt_descr_string[ i ] specifies, when not a null string, AI marking information that pertains to the picture(s) within the persistence scope of this SEI message.

NOTE: When txt_descr_purpose is equal to 2 the string can contain information about the machine-learning-based processing, intended use of the decoded pictures, or other aspects relevant to the associated pictures.

When txt_descr_purpose is equal to 3, the txt_descr_string[ i ] specifies a general text label description that pertains to the picture(s) in the persistence scope of this SEI message defined by txt_cancel_flag and txt_persistence_flag.

When txt_descr_purpose is equal to 4, the txt_descr_string[ i ] specifies content advisory rating information conforming to US. And Canadian Rating Region Tables (RRT) [3] that pertains to the picture(s) in the persistence scope of this SEI message defined by txt_cancel_flag and txt_persistence_flag

When txt_descr_purpose is equal to 5, txt_descr_string[ i ] contains a tag URI with syntax and semantics as specified in IETF RFC 4151 identifying the CLVS.

# Conclusion

It is proposed to adopt the proposed changes to the next draft of VSEI version 4.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AH2006, J. Boyce, S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang, SEI messages for VSEI version 4 (Draft 2), May 2024

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.