| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0061-v1 |
| --- | --- |

| Title: | AHG9: On Neural Network Post Filter for Spatial Extrapolation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed related to the Neural-Network Post-Filter Characteristics SEI.

- Proposal 1: When the NNPFC purpose indicates spatial extrapolation, it is proposed to optionally signal a text prompt for the spatial extrapolation image generation.

- Proposal 2: It is asserted that modifications are needed to allow correct operation of NNPF in patch-wise manner for spatial extrapolation purpose. Two alternative options are proposed for this.

# Introduction

The VSEI V4 draft 2 [1] includes a Neural-Network Post-Filter Characteristics SEI.

The Neural network post filter characteristics (NNPFC) SEI [1] includes a spatial extrapolation purpose which allows generating content outside of the spatial area of the input pictures.

Following is proposed related to spatial extrapolation NNPF:

- Proposal 1: It is asserted that the currently defined spatial extrapolation purpose only allows specifying syntax elements which define the spatially extended output picture dimensions. When the NNPFC purpose indicates spatial extrapolation, it is proposed to optionally specify a text prompt for the spatial extrapolation image generation. If the text prompt is specified it is proposed to be passed as an auxiliary information to the NNPF.

- Proposal 2: It is asserted that NNPF operates in a patch-wise manner which is manifested in equation 98. In that equation, input image is processed in patches of size (inpPatchHeight x inpPatchWidth), such that for each input patch inputTensor is created using DeriveInputTensors( ) function, followed by PostProcessingFilter( ) which takes as input the inputTensor to generate the filtered and/or interpolated picture(s) in a patch-wise manner returning outputTensor, which is in turn stored in Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, (as indicated by nnpfc_out_order_idc) of output patch size (outPatchHeight x outPatchWidth). It is asserted that as opposed to resolution resampling where each input patch may increase or decrease in size based on the resampling ratio, the spatial extrapolation only expands area outside the border of the spatial area of the input pictures. Thus, it is asserted that the patch-wise operation currently utilizing above equations for spatial extrapolation would not work correctly if image is broken in smaller patches, as each patch will get expanded by a factor which is dependent on how much spatial area is added by spatial extrapolation. Since the patch-wise operation is already defined in VSEI version 3, minimum change is proposed to make sure spatial extrapolation works correctly such that the spatial extrapolation NNPF output filtered pixels are only added outside the spatial area of the input pictures. Thus it is proposed to do this by setting input patch to be equal to the spatial area of the input picture and by defining inferences/ constraints. Two alternative options are proposed for this.

# Proposals

## Proposal 1

It is asserted that the currently defined spatial extrapolation purpose only allows specifying syntax elements which define the spatially extended output picture dimensions. When the NNPFC purpose indicates spatial extrapolation, it is proposed to optionally specify a text prompt for the spatial extrapolation image generation. If the text prompt is specified it is proposed to be passed as an auxiliary information to the NNPF.

A few example uses of the proposed text prompt for spatial extrapolation purpose are shown for illustration.

In figure 1 the original input image is shown.

Figure 1: Original input image

Figures 2 (a)-(c) respectively show spatial extrapolation of the original input image (from figure 1) where spatial extrapolation area is added to the right side of the image. In 2 (a) no text prompt is provided and the spatial extrapolation data is filled by NNPF to be generally consistent with the image contents. In 2 (b) spatial extrapolation is done where the text prompt used is “Cactus trees”. In contrast image 2 (c) shows the spatial extrapolation using the text prompt “Road”. As can be observed the extrapolated output images in figure 2 (a), 2 (b) and 2 (c) are different based on the provided text prompt (or no text prompt).

Figure 2 (a) No text prompt

Figure 2 (b) Text Prompt “Cactus trees”

Figure 2 (c) Text Prompt “Road”

Another illustration is shown in Figures 3 (a)- (c). Figures 3 (a)-(C) respectively show spatial extrapolation of the original input image (from figure 1) where spatial extrapolation area is added to the bottom of the image. The results shown in figures 3 (a), 3 (b), 3 (c) respectively correspond to no text prompt, text prompt of “River” and text prompt of “Road” respectively. Again, as can be observed the extrapolated output images in figure 3 (a), 3 (b) and 3 (c) are different based on the provided text prompt (or no text prompt).

Figure 3 (a) No text prompt

Figure 3 (b) Text Prompt “River”

Figure 3 (c) Text Prompt “Road”

The above examples illustrate the importance of how text prompt can vary the content of the spatially extrapolated region. It is thus proposed to optionally signal a text prompt to support this flexibility for spatial extrapolation.

## The proposed approach is shown below with modifications on top of JVET-AH2006 [1].

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| nnpfc_base_flag | u(1) |
| nnpfc_mode_idc | ue(v) |
| if( nnpfc_mode_idc = = 1 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_a | u(1) |
| nnpfc_tag_uri | st(v) |
| nnpfc_uri | st(v) |
| } |  |
| nnpfc_property_present_flag | u(1) |
| if( nnpfc_property_present_flag ) { |  |
| /* input and output formatting */ |  |
| nnpfc_num_input_pics_minus1 | ue(v) |
| if( nnpfc_num_input_pics_minus1 > 0 ) { |  |
| for( i = 0; i <= nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_input_pic_filtering_flag[ i ] | u(1) |
| nnpfc_absent_input_pic_zero_flag | u(1) |
| } |  |
| if( ChromaUpsamplingFlag ) |  |
| nnpfc_out_sub_c_flag | u(1) |
| if( ColourizationFlag ) |  |
| nnpfc_out_colour_format_idc | u(2) |
| if( ResolutionResamplingFlag ) { |  |
| nnpfc_pic_width_num_minus1 | ue(v) |
| nnpfc_pic_width_denom_minus1 | ue(v) |
| nnpfc_pic_height_num_minus1 | ue(v) |
| nnpfc_pic_height_denom_minus1 | ue(v) |
| } |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| if( SpatialExtrapolationFlag ) { |  |
| nnpfc_spatial_extrapolation_left_offset | ue(v) |
| nnpfc_spatial_extrapolation_right_offset | ue(v) |
| nnpfc_spatial_extrapolation_top_offset | ue(v) |
| nnpfc_spatial_extrapolation_bottom_offset | ue(v) |
| nnpfc_spatial_extrapolation_prompt_present_flag | u(1) |
| if (nnpfc_spatial_extrapolation_prompt_present_flag) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_c | u(1) |
| nnpfc_prompt | st(v) |
| } |  |
| } |  |
| nnpfc_component_last_flag | u(1) |
| nnpfc_inp_format_idc | ue(v) |
| nnpfc_auxiliary_inp_idc | ue(v) |
| nnpfc_inp_order_idc | ue(v) |
| … |  |
| /* ISO/IEC 15938-17 bitstream */ |  |
| if( nnpfc_mode_idc = = 0 ) { |  |
| while( !byte_aligned( ) ) |  |
| nnpfc_alignment_zero_bit_b | u(1) |
| for( i = 0; more_data_in_payload( ); i++ ) |  |
| nnpfc_payload_byte[ i ] | b(8) |
| } |  |
| } |  |

nnpfc_spatial_extrapolation_prompt_present_flag equal to 1 specifies that nnpfc_prompt syntax element is present and nnpfc_alignment_zero_bit_c syntax element may be present. nnpfc_spatial_extrapolation_prompt_present_flag equal to 0 specifies that nnpfc_prompt syntax element and nnpfc_alignment_zero_bit_c syntax element are not present.

nnpfc_alignment_zero_bit_c shall be equal to 0.

nnpfc_prompt specifies the text string prompt used for generating the contents of the spatial extrapolation image area. When nnpfc_spatial_extrapolation_prompt_present_flag is equal to 1, nnpfc_prompt shall not be a null string.

…

nnpfc_auxiliary_inp_idc greater than 0 indicates that auxiliary input data is present in the input tensor of the NNPF. nnpfc_auxiliary_inp_idc equal to 0 indicates that auxiliary input data is not present in the input tensor. nnpfc_auxiliary_inp_idc equal to 1, 2 or 3 specifies that auxiliary input data is derived as specified in Equation 95.

When nnpfc_auxiliary_inp_idc is equal to 2 or 3, nnpfc_spatial_extrapolation_prompt_present_flag shall be equal to 1.

The value of nnpfc_auxiliary_inp_idc shall be in the range of 0 to 255, inclusive. Values of 4 to 255, inclusive, for nnpfc_auxiliary_inp_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_auxiliary_inp_idc in the range of 24 to 255, inclusive.

When nnpfc_auxiliary_inp_idc is equal to 1 the auxiliary input data consists of strengthControlScaledVal[ i ].

When nnpfc_auxiliary_inp_idc is equal to 2 the auxiliary input data consists of nnpfc_prompt character values.

When nnpfc_auxiliary_inp_idc is equal to 3, the auxiliary input data consists of strengthControlScaledVal[ i ] and nnpfc_prompt character values.

nnpfc_inp_order_idc indicates the method of ordering the sample arrays of an input picture to form an input tensor to the NNPF.

The value of nnpfc_inp_order_idc shall be in the range of 0 to 255, inclusive. Values of 4 to 255, inclusive, for nnpfc_inp_order_idc are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall ignore NNPFC SEI messages with nnpfc_inp_order_idc in the range of 4 to 255, inclusive.

When ChromaFormatIdc is not equal to 1, nnpfc_inp_order_idc shall not be equal to 3.

When ChromaFormatIdc is equal to 0, nnpfc_inp_order_idc shall be equal to 0.

When ChromaUpsamplingFlag is equal to 1, nnpfc_inp_order_idc shall not be equal to 0.

Table 21 contains an informative description of nnpfc_inp_order_idc values.

Table 21 – Description of nnpfc_inp_order_idc values

| nnpfc_inp_ order_idc | Description |
| --- | --- |
| 0 | If nnpfc_auxiliary_inp_idc is equal to 0, one luma matrix is present in the input tensor for each input picture, and the number of channels is 1. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 1 or 2, one luma matrix and one auxiliary input matrix are present, and the number of channels is 2. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 3, one luma matrix and two auxiliary input matrices are present, and the number of channels is 3. |
| 1 | If nnpfc_auxiliary_inp_idc is equal to 0, two chroma matrices are present in the input tensor, and the number of channels is 2. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 1 or 2, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 3. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 3, two chroma matrices and two auxiliary input matrices are present, and the number of channels is 4. |
| 2 | If nnpfc_auxiliary_inp_idc is equal to 0, one luma and two chroma matrices are present in the input tensor, and the number of channels is 3. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 1 or 2, one luma matrix, two chroma matrices and one auxiliary input matrix are present, and the number of channels is 4. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 3, one luma matrix, two chroma matrices and two auxiliary input matrices are present, and the number of channels is 5. |
| 3 | If nnpfc_auxiliary_inp_idc is equal to 0, four luma matrices and two chroma matrices are present in the input tensor, and the number of channels is 6. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 1 or 2, four luma matrices, two chroma matrices, and one auxiliary input matrix are present in the input tensor, and the number of channels is 7. Otherwise, when nnpfc_auxiliary_inp_idc is equal to 3, four luma matrices, two chroma matrices, and two auxiliary input matrices are present in the input tensor, and the number of channels is 8. The luma channels are derived in an interleaved manner as illustrated in Figure 12. This nnpfc_inp_order_idc can only be used when the input chroma format is 4:2:0. |
| 4..255 | Reserved |

…

The process DeriveInputTensors( ), for deriving the input tensor inputTensor for a given vertical sample coordinate cTop and a horizontal sample coordinate cLeft specifying the top-left sample location for the patch of samples included in the input tensor, is specified as follows:

for( i = 0; i < numInputPics; i++ ) {
	if( nnpfc_inp_order_idc  = =  0 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpVal = InpY( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				promptCharVal = utf8ToUInt(nnpfc_prompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag )
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpVal
				else
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpVal
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = strengthControlScaledVal[ i ]
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc − 1 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc − 1 ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  1 )				(96)
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				inpCbVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( cTop + yP, cLeft + xP, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) ) 
				promptCharVal = utf8ToUInt(nnpfc_prompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][  nnpfc_auxiliary_inp_idc ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  2 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yY = cTop + yP
				xY = cLeft + xP
				yC = yY / SubHeightC
				xC = xY / SubWidthC
				inpYVal = InpY( InpSampleVal( yY, xY, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / SubHeightC,
						CroppedWidth / SubWidthC, CroppedCrPic[ i ], 2 ) ) 
				promptCharVal = utf8ToUInt(nnpfc_prompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpYVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpYVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc + 1 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc + 1 ] = promptCharVal
			}
	else if( nnpfc_inp_order_idc  = =  3 )
		for( yP = −nnpfc_overlap; yP < inpPatchHeight + nnpfc_overlap; yP++)
			for( xP = −nnpfc_overlap; xP < inpPatchWidth + nnpfc_overlap; xP++ ) {
				yTL = cTop + yP * 2
				xTL = cLeft + xP * 2
				yBR = yTL + 1
				xBR = xTL + 1
				yC = cTop / 2 + yP
				xC = cLeft / 2 + xP
				inpTLVal = InpY( InpSampleVal( yTL, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpTRVal = InpY( InpSampleVal( yTL, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBLVal = InpY( InpSampleVal( yBR, xTL, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpBRVal = InpY( InpSampleVal( yBR, xBR, CroppedHeight,
						CroppedWidth, CroppedYPic[ i ], 0 ) )
				inpCbVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCbPic[ i ], 1 ) )
				inpCrVal = InpC( InpSampleVal( yC, xC, CroppedHeight / 2,
						CroppedWidth / 2, CroppedCrPic[ i ], 2 ) ) 
				promptCharVal = utf8ToUInt(nnpfc_prompt) 
				yPovlp = yP + nnpfc_overlap
				xPovlp = xP + nnpfc_overlap
				if( !nnpfc_component_last_flag ) {
					inputTensor[ 0 ][ i ][ 0 ][ yPovlp ][ xPovlp ] = inpTLVal
					inputTensor[ 0 ][ i ][ 1 ][ yPovlp ][ xPovlp ] = inpTRVal
					inputTensor[ 0 ][ i ][ 2 ][ yPovlp ][ xPovlp ] = inpBLVal
					inputTensor[ 0 ][ i ][ 3 ][ yPovlp ][ xPovlp ] = inpBRVal
					inputTensor[ 0 ][ i ][ 4 ][ yPovlp ][ xPovlp ] = inpCbVal
					inputTensor[ 0 ][ i ][ 5 ][ yPovlp ][ xPovlp ] = inpCrVal
				} else {
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 0 ] = inpTLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 1 ] = inpTRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 2 ] = inpBLVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 3 ] = inpBRVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 4 ] = inpCbVal
					inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 5 ] = inpCrVal
				}
				if( nnpfc_auxiliary_inp_idc  = =  1 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ 6 ][ yPovlp ][ xPovlp ] = strengthControlScaledVal[ i ]
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ 6 ] = strengthControlScaledVal[ i ] 
				if( nnpfc_auxiliary_inp_idc  = =  2 | | nnpfc_auxiliary_inp_idc  = =  3)
					if( !nnpfc_component_last_flag )
						inputTensor[ 0 ][ i ][ nnpfc_auxiliary_inp_idc + 4 ][ yPovlp ][ xPovlp ] = promptCharVal
					else
						inputTensor[ 0 ][ i ][ yPovlp ][ xPovlp ][ nnpfc_auxiliary_inp_idc + 4 ] = promptCharVal
			}
}

utf8ToUInt(x) {
    result = 0 
    len = 0 
    // Check end of text prompt string
    if(x==null)
        return 0 // (or return -1)
    // Determine the number of bytes in the UTF-8 character
    if ((x[0] & 0x80) == 0) len = 1  // 1-byte character (ASCII) 
    else if ((x[0] & 0xE0) == 0xC0) len = 2  // 2-byte character
    else if ((x[0] & 0xF0) == 0xE0) len = 3  // 3-byte character
    else if ((x[0] & 0xF8) == 0xF0) len = 4  // 4-byte character
    else len=0 // Invalid UTF-8 character
    for (i = 0; i < len; i++) // Construct the integer from the bytes
        result = (result << 8) | (unsigned char)x[i]
    x=x+len
    return result
}

## Proposal 2

It is asserted that NNPF operates in a patch-wise manner which is manifested in equation 98. In that equation, input image is processed in patches of size (inpPatchHeight x inpPatchWidth), such that for each input patch inputTensor is created using DeriveInputTensors( ) function, followed by PostProcessingFilter( ) which takes as input the inputTensor to generate the filtered and/or interpolated picture(s) in a patch-wise manner returning outputTensor, which is in turn stored in Y, Cb, and Cr sample arrays FilteredYPic, FilteredCbPic, and FilteredCrPic, respectively, (as indicated by nnpfc_out_order_idc) of output patch size (outPatchHeight x outPatchWidth).

It is asserted that as opposed to resolution resampling where each input patch may increase or decrease in size based on the resampling ratio, the spatial extrapolation only expands area outside the border of the spatial area of the input pictures. Thus, it is asserted that the patch-wise operation currently utilizing above equations for spatial extrapolation would not work correctly if image is broken in smaller patches, as each patch will get expanded by a factor which is dependent on how much spatial area is added by spatial extrapolation. Since the patch-wise operation is already defined in VSEI version 3, minimum change is proposed to make sure spatial extrapolation works correctly such that the spatial extrapolation NNPF output filtered pixels are only added outside the spatial area of the input pictures, by setting input patch to be equal to the spatial area of the input pictures and by defining inferences/ constraints.

Two alternative options are proposed for setting the input patch to be equal to the spatial area of the input pictures.

## Option 1: When NNPF purpose is spatial extrapolation, nnpfc_overlap, and patch size related syntax (nnpfc_constant_patch_size_flag, nnpfc_patch_width_minus1, nnpfc_patch_height_minus1, nnpfc_extended_patch_width_cd_delta_minus1, nnpfc_extended_patch_height_cd_delta_minus1) is not signalled and instead the required values are inferred.

## Option 2: When NNPF purpose is spatial extrapolation, nnpfc_overlap is constrained to be equal to 0, nnpfc_constant_patch_size_flag is constrained to be equal to 1 and nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 are respectively constrained to be equal to CroppedWidth – 1 and CroppedHeight – 1.

## Option 1:

## When NNPF purpose is spatial extrapolation, nnpfc_overlap, and patch size related syntax (nnpfc_constant_patch_size_flag, nnpfc_patch_width_minus1, nnpfc_patch_height_minus1, nnpfc_extended_patch_width_cd_delta_minus1, nnpfc_extended_patch_height_cd_delta_minus1) is not signalled and instead the required values are inferred.

## The proposed approach is shown below with modifications on top of JVET-AH2006 [1].

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| … |  |
| if( nnpfc_out_order_idc > 0 ) |  |
| nnpfc_chroma_loc_info_present_flag | u(1) |
| if( nnpfc_chroma_loc_info_present_flag ) |  |
| nnpfc_chroma_sample_loc_type_frame | ue(v) |
| if( !SpatialExtrapolationFlag ) { |  |
| nnpfc_overlap | ue(v) |
| nnpfc_constant_patch_size_flag | u(1) |
| if( nnpfc_constant_patch_size_flag ) { |  |
| nnpfc_patch_width_minus1 | ue(v) |
| nnpfc_patch_height_minus1 | ue(v) |
| } else { |  |
| nnpfc_extended_patch_width_cd_delta_minus1 | ue(v) |
| nnpfc_extended_patch_height_cd_delta_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

nnpfc_overlap indicates the overlapping horizontal and vertical sample counts of adjacent input tensors of the NNPF. The value of nnpfc_overlap shall be in the range of 0 to 16 383, inclusive. When SpatialExtrapolationFlag is equal to 1, nnpfc_overlap is inferred to be equal to 0.

nnpfc_constant_patch_size_flag equal to 1 indicates that the NNPF accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 0 indicates that the NNPF accepts as input any patch size with width inpPatchWidth and height inpPatchHeight such that the width of an extended patch (i.e., a patch plus the overlapping area), which is equal to inpPatchWidth + 2 * nnpfc_overlap, is a positive integer multiple of nnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfc_overlap, and the height of the extended patch, which is equal to inpPatchHeight + 2 * nnpfc_overlap, is a positive integer multiple of nnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfc_overlap. When SpatialExtrapolationFlag is equal to 1, nnpfc_constant_patch_size_flag is inferred to be equal to 1.

nnpfc_patch_width_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the horizontal sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32 766, CroppedWidth − 1 ), inclusive. When SpatialExtrapolationFlag is equal to 1, nnpfc_patch_width_minus1 is inferred to be equal to CroppedWidth − 1.

nnpfc_patch_height_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the vertical sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32 766, CroppedHeight − 1 ), inclusive. When SpatialExtrapolationFlag is equal to 1, nnpfc_patch_height_minus1 is inferred to be equal to CroppedHeight − 1.

## Option 2:

## When NNPF purpose is spatial extrapolation, nnpfc_overlap is constrained to be equal to 0, nnpfc_constant_patch_size_flag is constrained to be equal to 1 and nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 are respectively constrained to be equal to CroppedWidth – 1 and CroppedHeight – 1.

## The proposed approach is shown below with modifications on top of JVET-AH2006 [1].

nnpfc_overlap indicates the overlapping horizontal and vertical sample counts of adjacent input tensors of the NNPF. The value of nnpfc_overlap shall be in the range of 0 to 16 383, inclusive. When SpatialExtrapolationFlag is equal to 1, nnpfc_overlap shall be equal to 0.

nnpfc_constant_patch_size_flag equal to 1 indicates that the NNPF accepts exactly the patch size indicated by nnpfc_patch_width_minus1 and nnpfc_patch_height_minus1 as input. nnpfc_constant_patch_size_flag equal to 0 indicates that the NNPF accepts as input any patch size with width inpPatchWidth and height inpPatchHeight such that the width of an extended patch (i.e., a patch plus the overlapping area), which is equal to inpPatchWidth + 2 * nnpfc_overlap, is a positive integer multiple of nnpfc_extended_patch_width_cd_delta_minus1 + 1 + 2 * nnpfc_overlap, and the height of the extended patch, which is equal to inpPatchHeight + 2 * nnpfc_overlap, is a positive integer multiple of nnpfc_extended_patch_height_cd_delta_minus1 + 1 + 2 * nnpfc_overlap. When SpatialExtrapolationFlag is equal to 1, nnpfc_constant_patch_size_flag shall be equal to 1.

nnpfc_patch_width_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the horizontal sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_width_minus1 shall be in the range of 0 to Min( 32 766, CroppedWidth − 1 ), inclusive. When SpatialExtrapolationFlag is equal to 1, nnpfc_patch_width_minus1 shall be equal to CroppedWidth − 1.

nnpfc_patch_height_minus1 plus 1, when nnpfc_constant_patch_size_flag equal to 1, indicates the vertical sample counts of the patch size required for the input to the NNPF. The value of nnpfc_patch_height_minus1 shall be in the range of 0 to Min( 32 766, CroppedHeight − 1 ), inclusive. When SpatialExtrapolationFlag is equal to 1, nnpfc_patch_height_minus1 shall be equal to CroppedHeight − 1.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- JVET-AH2006, J. Boyce, S. McCarthy, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullivan, Y.-K. Wang,  SEI messages for VSEI version 4 (Draft 2), May 2024

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.