| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0063-v1 |
| --- | --- |

| Title: | AHG12: Improvements on GPM partitioning splits |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kevin Reuzé Ya Chen Philippe Bordes Mikaël Le Pendu | Tel: Email: | firstname.lastname@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

This contribution proposes Geometric Partition Mode (GPM), SGPM and IBC-GPM where the list of partition candidates is block-shape dependent. In particular, a unit using GPM always gets access to its diagonal and inverse-diagonal splits. On top of this, the number of partitions allowed for regular GPM is increased from 64 to 80. The average results for the All Intra (AI) and Random Access (RA) configurations are presented as follows (Y, U, V, Enc time, Dec time):

# AI: -0.01%, 0.01%, -0.01%, 100.2%, 100.0%

# RA: -0.07%, -0.13%, -0.09%, 99.7%, 100.3%

# Introduction

Geometric Partition Mode (GPM), introduced in VVC, is a mode that splits a CU into two partitions by a straight splitting line. Each partition uses its own motion compensation(s) or intra prediction, and the final prediction for the CU is a sample-based blending of the two predictions. In ECM-13.0, GPM allows 64 partitions using 20 different angles, which are fixed for all block sizes. IBC-GPM and SGPM use subsets of the GPM split modes, respectively with IBC+intra predictions and intra+intra predictions.

# Proposed Solution

In this contribution, it is proposed to adapt the partitions used by GPM, SGPM and IBC-GPM to be dependent on the block shape, similar to wide angle intra prediction. In particular, it is asserted that diagonal splits are favored for a given block shape, therefore diagonal and inverted diagonal splits are included for each block shape, as well as splits with angles close to those. This means that the range of angles available GPM, SGPM and IBC-GPM is increased from 20 to 32.

For each width/height ratio from width/height=1/8 to width/height=8 a list of GPM partitions is proposed so that each block shape uses the most relevant partitions. The partitions are ordered so that the first 8 partitions constitute the IBC-GPM first set, the next 40 are the ones to be used for IBC-GPM second set, and the first 26 are the ones to be used for SGPM.

For regular GPM, the total number of partitions is also increased from 64 to 80, allowing for more split diversity. Since this change only adds SAD checks, it is asserted that this change does not impact the runtime.

# Experimental results

The proposed method is implemented on top of ECM-13.0. The simulations were performed following the CTCs [1] for the all-intra (AI) and random-access (RA) configurations. Reported runtimes are not reliable but since no additional RD checks are performed it is asserted that the runtimes at encoder and decoder is not impacted.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.04% | 0.12% | 99.9% | 101.2% | 99.9% | 100.1% |
| Class A2 | -0.01% | -0.03% | 0.11% | 100.8% | 99.6% | 103.7% | 101.0% |
| Class B | 0.00% | -0.03% | -0.09% | 100.0% | 99.9% | 101.8% | 100.0% |
| Class C | -0.01% | 0.08% | -0.03% | 99.4% | 99.4% | 104.4% | 100.1% |
| Class E | -0.04% | -0.03% | -0.11% | 101.3% | 100.2% | 105.0% | 100.1% |
| Overall | -0.01% | 0.01% | -0.01% | 100.2% | 100.0% | 102.9% | 100.2% |
| Class D | 0.02% | 0.03% | -0.04% | 99.9% | 100.0% | 104.5% | 100.1% |
| Class F | 0.02% | 0.12% | 0.05% | 100.0% | 99.6% | 104.2% | 99.9% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.22% | -0.15% | 99.1% | 100.2% | 100.6% | 99.8% |
| Class A2 | -0.15% | -0.23% | -0.21% | 99.6% | 99.4% | 100.9% | 100.1% |
| Class B | -0.05% | -0.14% | -0.16% | 99.6% | 101.3% | 101.2% | 100.0% |
| Class C | -0.07% | 0.03% | 0.13% | 100.4% | 99.7% | 103.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.13% | -0.09% | 99.7% | 100.3% | 101.5% | 100.0% |
| Class D | -0.01% | -0.09% | 0.17% | 99.8% | 101.5% | 103.9% | 100.1% |
| Class F | -0.06% | 0.07% | -0.08% | 99.3% | 99.5% | 101.9% | 100.2% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

# Conclusion

We recommend further study of the proposed method in the next round of EE2.

# References

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

# Patent rights declaration(s)

InterDigital Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).