| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JT, 12–19 July 2024 | Document: JVET-AI0065_v3 |
| --- | --- |

| Title: | EE2-4.3 16 States TCQ |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | M. Balcilar, K. Naser Y. Chen F. Galpin F. Le Léannec | Tel: Email: | muhammet.balcilar@interdigital.com |
| Source: | InterDigital Inc. |  |  |

# Abstract

This encoder-side non-normative contribution proposes to increase the number of trellis coded quantization (TCQ) states in dependent quantization (DQ). The number of states is increased from 8 to 16, where the 8 new states use similar transition as the original ones. With the proposed method, encoder can select when default 8 states or proposed 16 states DQ is used. To control the extra encoding complexity, encoder selects proposed 16 states for the last 50 coefficients in each transform unit in reading order. To reduce the encoder complexity further, one single state out of 8 extra states is updated. The updated state is the one whose RD cost is minimum among extra states.

The following results {Y, U, V; EncT, DecT, EncVmPeak} are obtained on top of ECM-13.0:

- AI:    { -0.02%, -0.04%, -0.02%; 101.8%, 99.8%, 101.7%}.

- RA:   { -0.03%, 0.01%, -0.06%	;  100.8%, 100.4%, 100.4%}.

- LDB:{ -0.04%, -0.51%, -0.12%; 100.8%, 99.5%, 100.7%}.

# Introduction

TCQ maps transform coefficients to reconstruction points to two predefined scalar quantizers while taking RD cost into account. The selection of the scalar quantizer used for current transform coefficient depends on the current state. The next coefficient’s state depends on the current state and the parity of the current coefficient’s quantization index. Quantization always starts with state 0 and state to state transitions are carefully designed to optimize the RD cost. Currently 8 states TCQ is used.

It is known that using higher number of states increases the search capability of TCQ, thus, it results in better RD cost theoretically. Under the conditions that the source is i.i.d and has known distribution, designing new transition table with higher number of states may be a trivial task. However, it is not the case for realistic task such as improving the performance of state-of-the-art video codecs. In this proposal, we introduce 16-states TCQ with a new state transition table.

# Proposed Method

We propose to use 16 states and following state to state transition table where default scalar quantizers are 
Q0,Q1. The transition depends on the current state and the parity of the quantization index to be either even or odd shown by pn=0 or pn=1 respectively.

| state sn | quantizer used | next state sn+1 |  |
| --- | --- | --- | --- |
|  |  | pn=0 | pn=1 |
| 0 | Q0 | 0 | 2 |
| 1 | Q1 | 5 | 7 |
| 2 | Q0 | 1 | 3 |
| 3 | Q1 | 6 | 4 |
| 4 | Q0 | 10 | 8 |
| 5 | Q1 | 12 | 14 |
| 6 | Q0 | 11 | 9 |
| 7 | Q1 | 15 | 13 |
| 8 | Q0 | 8 | 10 |
| 9 | Q1 | 13 | 15 |
| 10 | Q0 | 9 | 11 |
| 11 | Q1 | 14 | 12 |
| 12 | Q0 | 2 | 0 |
| 13 | Q1 | 4 | 6 |
| 14 | Q0 | 3 | 1 |
| 15 | Q1 | 7 | 5 |

We call the first half of the states original states, while the second half of the states are called dummy states. Each original state has its own dummy state. Thus, (i+8)th state is dummy state of ith original state where i<8. After cost calculation of each coefficient, we exchange the value of dummy state and its original one if the dummy state’s RD cost is better. We call this exchange process the state exchange mechanism.

To control the encoding complexity of the 16 states TCQ, the proposed method was implemented to emulate default 8 states TCQ if it is needed. Since the main source of complexity is related to the number of coefficients, the best trade off was found to apply 16 states TCQ for the last 50 coefficients in quantization order (top 50 lowest frequency component) and default 8 states for the rest.

To decrease the encoder complexity further, the first 8 states (original states) are always updated according to given transition table, but just the best single state (in terms of RD cost) of the last 8 states (dummy states) is updated. Thus, the implemented transition table can be seen as 9 active states where the first 8 states are fixed, but last state can be one of the dummy states. Since proposed method is encoder only, we can activate/deactivate method for desired configurations.

# Simulations

Proposed method is implemented on top of ECM-13.0 and is tested under CTC with all intra, random access and low delay B configurations.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.06% | 0.01% | 102.1% | 100.5% | 99.5% | 102.0% |
| Class A2 | -0.02% | -0.03% | 0.01% | 101.6% | 100.1% | 102.1% | 102.0% |
| Class B | -0.03% | -0.04% | -0.10% | 101.7% | 100.0% | 103.3% | 103.5% |
| Class C | -0.01% | 0.05% | 0.05% | 101.2% | 98.8% | 101.6% | 104.6% |
| Class E | -0.02% | -0.16% | -0.09% | 102.7% | 99.6% | 100.9% | 104.4% |
| Overall | -0.02% | -0.04% | -0.02% | 101.8% | 99.8% | 101.7% | 103.4% |
| Class D | 0.00% | 0.12% | -0.01% | 103.1% | 100.5% | 101.3% | 105.0% |
| Class F | -0.03% | 0.04% | -0.03% | 101.5% | 99.7% | 102.9% | 104.0% |
| Class TGM | 0.01% | 0.03% | -0.01% | 101.2% | 98.2% | 101.8% | 103.4% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1* | -0.03% | -0.13% | -0.12% | 100.8% | 100.0% | 100.2% | 101.3% |
| Class A2 | -0.01% | -0.01% | -0.14% | 101.5% | 100.1% | 100.3% | 101.7% |
| Class B | -0.05% | -0.09% | -0.06% | 100.7% | 100.8% | 100.2% | 102.7% |
| Class C | -0.01% | 0.26% | 0.04% | 100.6% | 100.3% | 100.8% | 104.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | 0.01% | -0.06% | 100.8% | 100.3% | 100.4% | 102.6% |
| Class D | -0.03% | -0.26% | 0.00% | 100.4% | 100.3% | 101.0% | 104.8% |
| Class F | 0.06% | 0.05% | -0.07% | 100.5% | 99.7% | 100.6% | 103.5% |
| Class TGM | -0.08% | -0.02% | 0.04% | 101.4% | 99.9% | 99.7% | 101.0% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B* | -0.07% | -0.56% | 0.21% | 101.0% | 99.5% | 100.4% | 102.6% |
| Class C | -0.15% | -0.35% | -0.48% | 100.8% | 99.9% | 101.0% | 104.2% |
| Class E | 0.13% | -0.63% | -0.19% | 100.6% | 99.2% | 100.7% | 103.8% |
| Overall | -0.04% | -0.51% | -0.12% | 100.8% | 99.5% | 100.7% | 103.4% |
| Class D | -0.19% | 0.18% | -0.12% | 100.4% | 99.3% | 101.0% | 104.9% |
| Class F | 0.05% | 0.24% | 0.37% | 102.0% | 100.7% | 100.8% | 103.4% |
| Class TGM* | -0.13% | 0.05% | -0.12% | 100.8% | 99.6% | 100.2% | 100.7% |

* One test point (qp=22) is replaced with anchor data.

# Conclusion

Considering the coding efficiency improvement, we recommend integrating the tool in next version of the ECM. It is proposed to adopt the tool in next version of ECM.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).