| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0067 |
| --- | --- |

| Title: | AHG12: GPM mode extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yongjo Ahn Junghak Nam Naeri Park Jaehyun Lim 19 Yangjaedaero-11gil, Soecho-gu, Seoul 06772, South Korea Seunghwan Kim 10680 Treena St, San Diego, CA 92131, USA | Tel: Email: | +82-2-6912-6561 yongjo.ahn@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to extend the number of GPM partitioning mode. In ECM-13.0, GPM supports 64 partitioning modes combined with 20 slope based angles and 4 distances. Specifically, the proposed method extends the number of partitioning mode from 64 to 112 considering additional angles and distances. The proposed GPM mode extension was implemented on top of ECM-13.0 and the summarized test results are shown as follows:

RA: -0.07%/-0.02%/-0.05% (Y/U/V), 101.4% (EncT), 100.0% (DecT)

LDB: 0.03%/-0.41%/-0.15% (Y/U/V), 102.3% (EncT), 100.1% (DecT)

Introduction

In the VVC and ECM, GPM supports 64 partitioning modes from the combination between 20 angles and 4 distances. In the initial contribution of GPM, 32 angles with 11.25 degrees spacing were proposed [1]. During CE discussion, it was simplified *64 GPM modes with 20 slope based angles and 4 distances to get a better trade-off between signaling overhead and coding gain [2]. (*64 GPM modes = 8 angles × 4 distances + 8 angles × 3 distances + 4 angles × 2 distances)

Figure 1. An example of 64 GPM partitioning modes in ECM-13.0

In addition, template matching (TM) based reordering and signaling modifications for GPM partitioning modes were proposed and adopted in ECM to reduce mode signaling overhead [3]. In ECM-13.0, 64 GPM partitioning modes are reordered based on TM cost and one of the best 32 modes is signaled. With the TM based reordering and signaling modifications, signaling overhead from the additional GPM partitioning modes can be reduced. Deriving from this motivation, GPM mode extension is proposed to improve coding efficiency without signaling overhead.

Proposed methods

This contribution proposes to extend the number of GPM partitioning mode from 64 to 112. Technical details of the proposed method are described in the following subsections:

Additional angles

In the proposed GPM mode extension, the number of GPM angles are extended to 32. In the current ECM, GPM supports 20 slope based angles and indices with un-equal spacing. It was proposed that the angle index near vertical angle direction (horizontal split directions) are removed since most nature objects or motions are vertically [2]. The proposed extension supports 12 additional slope based angles with equal spacing. The additional angles can be combined with 4 distances same as the conventional angles, and total of *42 GPM modes are newly added. (*42 GPM modes = 6 angles × 4 distances + 6 angles × 3 distances)

Figure 2. The proposed 32 angles for GPM partitioning mode

Additional distances

In the current GPM, six distance indices for horizontal and vertical angles (0, 8, 16, 24) are removed because these six partitioning modes are likely to overplay with binary tree and ternary tree split [2]. However, blending process can be applied to the boundary samples of GPM partitions, it makes different coding results compared with binary tree and ternary tree split on CU partitioning level. In addition, the current ECM supports TM based reordering process and it helps to reduce signaling overhead for additional GPM modes.

This contribution proposes to add total 6 GPM modes for 6 horizontal and vertical distances which were removed in 64 GPM modes as shown in Figure 3.

Figure 3. 6 additional vertical and horizontal distances for GPM mode extension

Experimental results

The proposed GPM mode extension has been implemented in ECM-13.0 [4] and evaluated under random access (RA) and low-delay B (LDB) common test configurations [5].

As shown in the below tables, the proposed method provides 0.07% BD-rate saving over ECM-13.0 under the RA configurations. Encoding time increment is 101.4% and no meaningful decoding time increment is observed.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.07% | -0.20% | 101.2% | 99.7% | 101.3% | 99.9% |
| Class A2 | -0.09% | 0.06% | -0.09% | 101.6% | 99.9% | 101.8% | 100.1% |
| Class B | -0.05% | 0.02% | -0.18% | 101.4% | 100.1% | 103.7% | 100.1% |
| Class C | -0.06% | -0.08% | 0.25% | 101.6% | 100.0% | 109.5% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.02% | -0.05% | 101.4% | 100.0% | 104.3% | 100.1% |
| Class D | 0.02% | 0.21% | 0.51% | 101.4% | 100.4% | 112.6% | 100.1% |
| Class F | -0.02% | -0.19% | -0.33% | 101.2% | 100.3% | 105.9% | 100.0% |
| Class TGM | -0.20% | -0.12% | -0.07% | 101.4% | 99.7% | 101.7% | 99.9% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.06% | -0.44% | 0.23% | 102.4% | 99.7% | 104.7% | 100.0% |
| Class C | -0.02% | -0.08% | -0.62% | 102.5% | 100.4% | 110.1% | 100.2% |
| Class E | 0.23% | -0.81% | -0.15% | 101.8% | 100.2% | 107.6% | 100.1% |
| Overall | 0.03% | -0.41% | -0.15% | 102.3% | 100.1% | 107.2% | 100.1% |
| Class D | -0.01% | -0.03% | -0.49% | 102.3% | 100.8% | 112.9% | 100.1% |
| Class F | -0.13% | 0.16% | 0.67% | 101.2% | 100.5% | 106.1% | 100.1% |
| Class TGM | -0.28% | 0.10% | -0.05% | 101.9% | 101.3% | 103.0% | 101.2% |

Conclusion

This contribution proposes to extend GPM partitioning mode from 64 to 112. The proposed extension achieves 0.07% BD-rate saving with 101.4% encoding time and 100.0% decoding time under RA configuration. It is recommended to include the proposed method in the next round of EE2.

Reference(s)

- S. Esenlik, H. Geo, A. Filippov, V. Rufitskiy, A. M. Korta, B. Wang, E. Alshina, M. Blaser, J. Sauer, “Non-CE4: Geometrical partitioning for inter blocks,” JVET-O0489, 15th JVET meeting, Gothenburg, SE, 3-12 July 2019.

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, R.-L. Liao, J. Chen, Y. Ye, J. Luo, K. Reuzé, C.-C. Chen, H. Huang, W.-J. Chien, V. Seregin, M. Blaser, J. Sauer, “CE4-1: Geometric Inter Prediction with 64 Modes,” JVET-Q0059, 17th JVET meeting, Brussels, BE, 7-15 January 2020.

- C.-C. Chen, H. Huang, Y. Zhang, Z. Zhang, Y.-J. Chang, V. Seregin, M. Karczewicz, “EE2-2.4: Template matching based reordering for GPM split modes,” JVET-Z0056, 26th JVET meeting, by teleconference, 20-29 April 2022.

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, 32nd JVET Meeting, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).