| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0071-v1 |
| --- | --- |

| Title: | AHG8/AHG9: Indication of NNPFs and processing chains for human viewing and/or machine consumption |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Honglei Zhang Jill Boyce | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com honglei.1.zhang@nokia.com jill.boyce@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

It is asserted that post-filters or post-filter chains may be trained differently for optimized human viewing than for optimized machine analysis. Since different post-filters or post-filter chains may be made available in the same bitstream, signalling of the intended usage of the post-filters or post-filter chains would be helpful for the decoding system to select the post-filters or post-filter chains depending on the current usage for human viewing or machine analysis.

This contribution responds to the comments received on JVET-AH0125 and proposes both of the following items:

- Indication of the user viewing and/or machine analysis usage in the NNPFC SEI message with 2-bit indicators nnpfc_for_human_viewing_idc and nnpfc_for_machine_analysis_idc, which are specified like the respective indicators in the EOI SEI message but apply to the output of the NNPF process.

- Indication of the user viewing and/or machine analysis usage in the SPO SEI message with 2-bit indicators po_for_human_viewing_idc and po_for_machine_analysis_idc, which are specified like the respective indicators in the EOI SEI message but apply to the output of the processing chain.

# Introduction

To indicate the target usage (human viewing and/or machine analysis), JVET-AH0125 proposed both of the following items:

- To indicate the user viewing and/or machine analysis usage of a neural-network post-filter (NNPF), it is proposed to add the target usage syntax element as a gated ue(v)-coded bit field in the metadata extension part of the NNPF characteristics (NNPFC) SEI message. Bit 0 is defined to indicate human viewing and bit 1 for machine analysis.

- To indicate the user viewing and/or machine analysis usage of a processing chain, it is proposed to allow the usage of the encoder optimization information (EOI) SEI message in an SEI processing order (SPO) SEI message. In this case:

  - The semantics are interpreted to apply to the video that results from the processing chain up to the processing order value of the EOI SEI message.

  - It is required to include the EOI SEI message(s) in processing order nesting (PON) SEI message(s).

JVET meeting notes on JVET-AH0125 are copied below in italics:

About aspect 1: It was commented that it might be not a single filter but the entire chain that is relevant. On the other hand, it is usually only a neural network postprocessing that could convert decoder output that is useful for machine viewing into something that is human consumable (or vice versa).

About aspect 2: It was commented that changing the semantics of EOI is undesirable, and resusing it different from its original purpose is desirable. EOI is intended for signalling something that was done in preprocessing.

The following alternatives are suggested rather than redefining EOI, or extend NNPF:

- Define a new SEI message signalling the properties of PP output (H/M)

- Include such information in SPO

Generally, it is agreed to be useful, also in context of TR. Further study was requested on identifying a more consistent solution.

This contribution is a follow-up of JVET-AH0125.

# Discussion

It is asserted that the intended usage for user viewing and/or machine analysis is beneficial in the NNPFC SEI message, because:

- An NNPF may be activated outside of any processing chain defined by an SPO SEI message.

- An NNPF is trained for a particular target usage. Thus, the target usage is an NNPF property and conceptually belongs to an NNPFC SEI message.

The option to define a new SEI message signalling the properties of post-processing output seems heavy for carrying just the user viewing and/or machine analysis indications. Furthermore, such an SEI message would need to be included in SPO and PON SEI messages, because otherwise its association with a particular post-processing operation would be unclear. It seems cleaner to add user viewing and machine analysis indications directly into the SPO SEI message.

As an option to item 1, it was considered to add nnpfc_purpose bits to indicate human viewing and machine analysis. However, this option is not recommended, since decoding systems conforming to VSEI v3 discard NNPFC SEI messages with either or both of these bits equal to 1, even though they would be capable of performing the NNPF inference. Moreover, in the proponents' opinion, nnpfc_purpose indicates a filtering purpose as a format transfer, whereas the intended usage of the filtered pictures for human viewing or machine analysis is another dimension.

The 2-bit human viewing and machine analysis indications of the EOI SEI message are asserted to provide a sufficient granularity of information to the decoding system. Hence, this contribution uses the same design choice in the proposed indications, which also strengthens the consistency of the design across the EOI, NNPFC, and SPO SEI messages.

# Overview of the proposal

This contribution proposes both of the following items:

- Indication of the user viewing and/or machine analysis usage in the NNPFC SEI message with 2-bit indicators nnpfc_for_human_viewing_idc and nnpfc_for_machine_analysis_idc, which are specified like the respective indicators in the EOI SEI message but apply to the output of the NNPF process.

- Indication of the user viewing and/or machine analysis usage in the SPO SEI message with 2-bit indicators po_for_human_viewing_idc and po_for_machine_analysis_idc, which are specified like the respective indicators in the EOI SEI message but apply to the output of the processing chain.

# Specification text for item 1: indication of the user viewing and/or machine analysis usage in the NNPFC SEI message

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| nnpfc_num_metadata_extension_bits | ue(v) |
| if( nnpfc_num_metadata_extension_bits > 0 ) { |  |
| if( nnpfc_purpose = = 0 ) { |  |
| nnpfc_application_purpose_tag_uri_present_flag | u(1) |
| if( nnpfc_application_purpose_tag_uri_present_flag ) |  |
| nnpfc_application_purpose_tag_uri | st(v) |
| } |  |
| if( SpatialExtrapolationFlag ) |  |
| nnpfc_scan_type_idc | u(2) |
| nnpfc_for_human_viewing_idc | u(2) |
| nnpfc_for_machine_analysis_idc | u(2) |
| nnpfc_reserved_metadata_extension | u(v) |
| } |  |
| … |  |

nnpfc_for_human_viewing_idc equal to 3 specifies that the intended optimal usage of the video resulting from the NNPF process includes human viewing. nnpfc_for_human_viewing_idc equal to 2 specifies that the video resulting from the NNPF process is suitable but not specifically optimized for human viewing. nnpfc_for_human_viewing_idc equal to 1 specifies that the video resulting by the NNPF process is unsuitable for human viewing. nnpfc_for_human_viewing_idc equal to 0 specifies that it is unknown if the video resulting by the NNPF process is suitable for human viewing. When not present, nnpfc_for_human_viewing_idc is inferred to be equal to 0.

nnpfc_for_machine_analysis_idc equal to 3 specifies that the intended optimal usage of the video resulting from the NNPF process includes machine analysis. nnpfc_for_machine_analysis_idc equal to 2 specifies that the video resulting from the NNPF process is suitable but not specifically optimized for machine analysis. nnpfc_for_machine_analysis_idc equal to 1 specifies that the video resulting from the NNPF process is unsuitable for machine analysis. nnpfc_for_machine_analysis_idc equal to 0 specifies that it is unknown if the video resulting from the NNPF process is suitable for machine analysis. When not present, nnpfc_for_machine_analysis_idc is inferred to be equal to 0.

NOTE – When a decoding system displays the video for human viewing, any NNPF that has nnpfc_for_human_viewing_idc equal to 1 is suggested to be omitted. When a decoding system performs machine analysis, any NNPF that has nnpfc_for_machine_analysis_idc equal to 1 is suggested to be omitted.

# Specification text for item 2: indication of the user viewing and/or machine analysis usage in the SPO SEI message

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_for_human_viewing_idc | u(2) |
| po_for_machine_analysis_idc | u(2) |
| po_reserved_zero_4bits | u(4) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

po_for_human_viewing_idc equal to 3 specifies that the intended optimal usage of the video resulting from the processing chain specified by this SPO SEI message includes for human viewing. po_for_human_viewing_idc equal to 2 specifies that that the video resulting from the processing chain specified by this SPO SEI message is suitable but not specifically optimized for human viewing. po_for_human_viewing_idc equal to 1 specifies that the video resulting from the processing chain specified by this SPO SEI message is unsuitable for human viewing. po_for_human_viewing_idc equal to 0 specifies that it is unknown if the video resulting from the processing chain specified by this SPO SEI message is suitable for human viewing.

po_for_machine_analysis_idc equal to 3 specifies that the intended optimal usage of the video resulting from the processing chain specified by this SPO SEI message includes machine analysis. po_for_machine_analysis_idc equal to 2 specifies that the video resulting from the processing chain specified by this SPO SEI message is suitable but not specifically optimized for machine analysis. po_for_machine_analysis_idc equal to 1 specifies that the video resulting from the processing chain specified by this SPO SEI message is unsuitable for machine analysis. po_for_machine_analysis_idc equal to 0 specifies that it is unknown if the video resulting from the processing chain specified by this SPO SEI message is suitable for machine analysis.

po_reserved_zero_4bits shall be equal to 0. Values greater than 0 for po_reserved_zero_4bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall allow any value of po_reserved_zero_4bits in the range of 0 to 15, inclusive.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).