| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0072-v2 |
| --- | --- |

| Title: | AHG9: Activating an NNPF in a processing chain for temporally interpolated or extrapolated pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela ¦ francesco.cricri}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

JVET-AH0123 proposed to add indications in the processing order nesting (PON) SEI message to indicate which ones of the candidate pictures, consisting of the current picture and the interpolated and extrapolated pictures generated in any previous processing stages, the semantics of the PON-nested SEI messages apply. Thus, for example, a PON-nested neural-network post-filter activation (NNPFA) SEI message can be indicated to apply to only the interpolated pictures but not the current picture. The JVET-AH meeting report concludes the recommendation for JVET-AH0123 as follows: "Further study was encouraged with the suggestion to find a way to avoid putting this syntax in the PON when it is only needed for NNPF."

This input document is a follow-up of JVET-AH0123 and proposes an NNPF-specific approach for the asserted problem. The proposal includes the following aspects:

- Two options are proposed for NNPF activation with controlled selection of input pictures:

  - Extension of the NNPFA SEI message by adding new syntax elements for controlled selection of input pictures at the end of the syntax structure.

    - In order to maintain compatibility with v3 decoders, the extended syntax is allowed only when the NNPFA SEI message is included in a PON SEI message.

  - NNPF extended activation (NNPFEA) SEI message, which includes syntax elements that correspond to those in the NNPFA SEI message and the syntax elements for controlled selection of input pictures.

    - It is disallowed to use both the NNPFA and NNPFEA SEI messages to activate the same NNPF for the same picture.

- The input pictures for an NNPF are selected in two phases:

First, the candidate input pictures are selected the same way as input pictures for an NNPF are selected in VVC v3.

Second, the input pictures are selected from the candidate input pictures based on the NNPFA (option 1.a) or NNPFEA (option 1.b) syntax elements for controlled selection of input pictures.

# Problem Statement

When a processing chain includes picture rate upsampling, it is asserted that it is unclear which pictures the subsequent processing stages in the same processing chain apply. Use cases where the subsequent processing stages do not apply to both the current picture and all the pictures that were generated through picture rate upsampling include the following:

- An interpolated picture is enhanced since the picture rate upsampling NNPF (also called the interpolator NNPF) has low complexity and produces relatively poor picture quality. Thus, a visual enhancement filter is to be applied only on the interpolated pictures, but not the cropped decoded pictures. See the following figure.

Additionally, it is asserted that the current design does not allow an encoder to make a decision between the base NNPF or an updated NNPF for interpolated pictures, but rather the choice between the base NNPF or an updated NNPF that is indicated for a cropped decoded picture also applies to the interpolated pictures. An example:

- An encoder finetunes a quality-enhancement NNPF based on interpolated pictures of a random access segment and sends an NNPF update through an NNPFC SEI message. Ideally, an encoder would indicate the use of the base quality-enhancement NNPF for the cropped decoded pictures and the updated NNPF for the interpolated pictures, but this is not possible with the current design. See the following figure.

Furthermore, it is asserted that when a processing chain includes a picture rate upsampling NNPF, the current design does not allow an encoder to indicate to which pictures in a list of generated pictures resulting from a single NNPF activation the subsequent processing stages in the same processing chain apply. An example:

- An interpolator NNPF generates three pictures based on two input pictures, e.g., generates pictures 1, 2, 3 based on pictures 0 and 4. It is likely that the inner-most generated picture, e.g., generated pictures 2, is of lower quality than the other generated pictures in the list of generated pictures, e.g., than generated pictures 1 and 3. In order to enhance only the inner-most generated picture, e.g., generated picture 2, an encoder would need to indicate the use of a quality-enhancement NNPF for that inner-most generated picture only. See the following figure.

# Earlier works

## JVET-AH0123

JVET-AH0123 proposed to add indications in the PON SEI message to indicate which ones of the candidate pictures, consisting of the current picture and the interpolated and extrapolated pictures generated in any previous processing stages, the semantics of the PON-nested SEI messages apply. Thus, for example, a PON-nested NNPFA SEI message can be indicated to apply to only the interpolated pictures but not the current picture. Two options were proposed in JVET-AH0123:

- pon_curr_pic_association_flag and pon_gen_pic_association_all_flag indicating, when equal to 1, that the PON-nested SEI messages apply to the current picture and all the generated pictures, respectively.

With this option, examples 1 and 2 can be realized.

- In addition to the functionality of option 1, optional additions in the PON SEI message are proposed to indicate per each generated picture whether the PON-nested SEI messages apply.

With this option, all examples 1 to 3 can be realized.

The JVET-AH meeting report concludes the recommendation for JVET-AH0123 as follows: "Further study was encouraged with the suggestion to find a way to avoid putting this syntax in the PON when it is only needed for NNPF."

## JVET-AE0052

JVET-AE0052 proposed to enable cascades and other groups of NNPFs with the following new SEI messages:

- NNPF group characteristics (NNPFGC) SEI message, which can be used to define either a cascade of NNPFs or indicate a set of alternative NNPFs or alternative NNPF cascades.

- NNPF group activation (NNPFGA) SEI message, which is used for activating an NNPF cascade.

Among other things, the NNPFGA SEI message design included the following aspects for each NNPF in a group:

- An indication of whether to apply the base NNPF or an updated NNPF.

- An indication if the input pictures for the i-th NNPF in the NNPF group are formed from the list of candidate input pictures without skipping. If not, additional indications of which candidate pictures are used as input pictures to the NNPF.

JVET-AE0052 was integrated into JVET-AE0298, which was included in the VSEI TuC.

# Overview of the proposal

The proposal assumes that an available input picture list for a processing stage is derived like described in VVC subclause D.11.3 (Handling of a processing chain) included in the VSEI TuC (JVET-AH2032). In version 2 of this document, the specification text of aspect 2 was updated to integrate the use of PoPicList variable derived in VVC subclause D.11.3.

The proposal includes the following aspects:

- Two options are proposed for NNPF activation with controlled selection of input pictures:

  - Extension of the NNPFA SEI message by adding new syntax elements for controlled selection of input pictures at the end of the syntax structure.

    - In order to maintain compatibility with v3 decoders, the extended syntax is allowed only when the NNPFA SEI message is included in a PON SEI message.

  - NNPF extended activation (NNPFEA) SEI message, which includes syntax elements that correspond to those in the NNPFA SEI message and the syntax elements for controlled selection of input pictures.

    - It is disallowed to use both the NNPFA and NNPFEA SEI messages to activate the same NNPF for the same picture.

- The input pictures for an NNPF are selected in two phases:

First, the candidate input pictures are selected the same way as input pictures for an NNPF are selected in VVC v3.

Second, the input pictures are selected from the candidate input pictures based on the NNPFA (option 1.a) or NNPFEA (option 1.b) syntax elements for controlled selection of input pictures.

# Proposed specification text

## NNPF activation with controlled selection of input pictures

### Extension of the NNPFA SEI message (option 1.a)

| nn_post_filter_activation( payloadSize ) { | Descriptor |  |
| --- | --- | --- |
| nnpfa_target_id | ue(v) |  |
| nnpfa_cancel_flag | u(1) |  |
| if( !nnpfa_cancel_flag ) { |  |  |
| nnpfa_persistence_flag | u(1) |  |
| nnpfa_target_base_flag | u(1) |  |
| nnpfa_no_prev_clvs_flag | u(1) |  |
| if( nnpfa_persistence_flag ) |  |  |
| nnpfa_no_foll_clvs_flag | u(1) |  |
| nnpfa_num_output_entries | ue(v) |  |
| for( i = 0; i < nnpfa_num_output_entries; i++ ) |  |  |
| nnpfa_output_flag[ i ] | u(1) |  |
| if( more_data_in_payload( ) ) { |  |  |
| nnpfa_input_selected_pics_flag | u(1) |
| if( nnpfa_input_selected_pics_flag ) { |  |
| nnpfa_num_input_pics_minus1 | ue(v) |
| for( i = 0; i <= nnpfa_num_input_pics_minus1; i++ ) |  |
| nnpfa_input_pic_skip_count[ i ] | ue(v) |
| } |  |
| } |  |  |
| } |  |  |
| } |  |  |

nnpfa_input_selected_pics_flag equal to 1 specifies that the input pictures to the NNPF are selected from the list of candidate input pictures in a manner that some candidate input pictures are skipped. nnpfa_input_selected_pics_flag equal to 0 specifies that the input pictures to the NNPF are selected from the list of candidate input pictures without skipping. When not present, nnpfa_input_selected_pics_flag is inferred to be equal to 0.

nnpfa_num_input_pics_minus1 plus 1 specifies the number of input pictures for the NNPF. When present, nnpfa_num_input_pics_minus1 shall be equal to nnpfc_num_input_pics_minus1 for an NNPF with nnpfc_id equal to nnpfa_target_id. When not present, nnpfa_num_input_pics_minus1 is inferred to be equal to nnpfc_num_input_pics_minus1 for an NNPF with nnpfc_id equal to nnpfa_target_id.

nnpfa_input_pic_skip_count[ i ] specifies the i-th picture count that is skipped in the list of candidate input pictures when selecting input pictures for the NNPF. When nnpfa_input_pic_skip_count[ i ] is not present, it is inferred to be equal to 0 for all values of i in the range of 0 to nnpfa_num_input_pics_minus1, inclusive.

### NNPF extended activation (NNPFEA) SEI message (option 1.b)

| nn_post_filter_extended_activation( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfea_target_id | ue(v) |
| nnpfea_cancel_flag | u(1) |
| if( !nnpfea_cancel_flag ) { |  |
| nnpfea_persistence_flag | u(1) |
| nnpfea_target_base_flag | u(1) |
| nnpfea_no_prev_clvs_flag | u(1) |
| if( nnpfea_persistence_flag ) |  |
| nnpfea_no_foll_clvs_flag | u(1) |
| nnpfea_num_output_entries | ue(v) |
| for( i = 0; i < nnpfea_num_output_entries; i++ ) |  |
| nnpfea_output_flag[ i ] | u(1) |
| nnpfea_input_selected_pics_flag | u(1) |
| if( nnpfea_input_selected_pics_flag ) { |  |
| nnpfea_num_input_pics_minus1 | ue(v) |
| for( i = 0; i <= nnpfea_num_input_pics_minus1; i++ ) |  |
| nnpfea_input_pic_skip_count[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

nnpfea_target_id, nnpfea_cancel_flag, nnpfea_persistence_flag, nnpfea_target_base_flag, nnpfea_no_pref_clvs_flag, nnpfea_no_foll_clvs_flag, nnpfea_num_output_entries, nnpfea_output_flag[ i ], nnpfea_input_selected_pics_flag, nnpfea_num_input_pics_minus1, and nnpfea_input_pic_skip_count[ i ] have the same semantics as nnpfa_target_id, nnpfa_cancel_flag, nnpfa_persistence_flag, nnpfa_target_base_flag, nnpfa_no_pref_clvs_flag, nnpfa_no_foll_clvs_flag, nnpfa_num_output_entries, nnpfa_output_flag[ i ], nnpfa_input_selected_pics_flag, nnpfa_num_input_pics_minus1, and nnpfa_input_pic_skip_count[ i ], respectively.

### Constraint to VVC use of NNPFC and NNPFA SEI messages (aspect 1.a.i)

It is a requirement of bitstream conformance that when an NNPFA SEI message is not included in a PON SEI message, nnpfa_input_selected_pics_flag, nnpfa_num_input_pics_minus1, and nnpfa_input_pic_skip_count[ i ] shall not be present in the NNPFA SEI message.

### Constraints to VVC use of NNPFC and NNPFA SEI messages (aspect 1.b.i)

It is a requirement of bitstream conformance that the same NNPF is not activated for any picture with both an NNPFA SEI message and an NNPFEA SEI message that are not included in a processing order nesting SEI message.

It is a requirement of bitstream conformance that the same NNPF is not activated for any picture with both a PON-nested NNPFA SEI message and a PON-nested NNPFEA SEI message that are associated with the same value of po_id value and the same processing order value.

## Selection of input pictures in two phases (aspect 2)

The yellow-highlighted changes are proposed to VVC D.12.11 (Use of the neural network post-filter characteristics SEI message and the neural network post-filter activation SEI message)

[Ed. the changes are presented using the proposed new NNPFA syntax elements (option 1.a) but would be likewise realized with the respective NNPFEA syntax elements for option 1.b.]

The variable numCandInputPics, which indicates the number of candidate input pictures to the NNPF, is derived as follows:

numCandInputPics = 0
for( i = 0; i  <=  nnpfa_num_input_pics_minus1; i++ )
	numCandInputPics += 1 + nnpfa_input_pic_skip_count[ i ]	(xx)

The list availPicList, which is a list of pictures from which the candidate input pictures to the NNPF process are selected, is derived as follows:

–	If the NNPFA SEI message is not applied as part of a processing chain indicated by an SEI processing order SEI message or is applied as the first processing step of the processing chain indicated by an SEI processing order SEI message, availPicList is the list of those cropped decoded pictures of the bitstream in output order that belong to the current layer.

–	Otherwise (the NNPFA SEI message is applied as the second or a later processing step of the processing chain indicated by an SEI processing order SEI message), availPicList is set to be the same as PoPicList derived for currPic when the processing chain is handled as specified in clause D.11.3 up to but excluding the processing stage corresponding to the NNPFA SEI message.

Let currCandPic be the picture in availPicList that corresponds to currPic, i.e., has the same output time as currPic.

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] candInputPic[ i ] and inputCandPresentFlag[ i ] for i in the range of 0 to numCandInputPics − 1, inclusive, representing all the candidate input pictures and the presence of candidate input pictures, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] candInputPic[ k ] is set to be currCandPic and inputCandPresentFlag[ k ] is set equal to 0.

–	The j-th candidate input picture, inputPic[ j ] candInputPic[ j ], is set to be currCandPic and inputCandPresentFlag[ j ] is set equal to 1.

–	When numCandInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numCandInputPics − 1, inclusive, in increasing order of i:

–	If both of the following conditions are true, inputPic[ i ] candInputPic[ i ] is set to be prevPic and inputPresentFlag[ i ] is set equal to 1:

–	Either of the following conditions is true:

–	pictureRateUpsamplingFlag is equal to 1 and currPic is associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and a particular value of fp_current_frame_is_frame0_flag, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that are present in availPicList have nuh_layer_id equal to currLayerId, precede inputPic[ i − 1 ] candInputPic[ i − 1 ] in output order, and are associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

–	pictureRateUpsamplingFlag is equal to 0 or currPic is not associated with a frame packing arrangement SEI message with frame_packing_arrangement_type equal to 5, and there is a cropped decoded output picture prevPic that is the last picture in output order among all cropped decoded output pictures that are present in availPicList have nuh_layer_id equal to currLayerId and precede inputPic[ i − 1 ] candInputPic[ i − 1 ] in output order.

–	nnpfa_no_prev_clvs_flag is equal to 0 or the coded picture corresponding to prevPic and the current picture are present in the same CLVS.

–	Otherwise, the following applies:

–	inputPic[ i ] candInputPic[ i ] is set to be the same picture as inputPic[ i − 1 ] candInputPic[ i − 1 ] and inputCandPresentFlag[ i ] is set equal to 0.

–	It is a requirement of bitstream conformance that, when pictureRateUpsamplingFlag is equal to 1, nnpfc_interpolated_pics[ i − 1 ] shall be equal to 0.

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures, respectively, are specified as follows:

for( i = 0, candIdx = 0; i <= nnpfa_num_input_pics_minus1; i++, candIdx++ ) {
	candIdx += nnpfa_input_pic_skip_count[ i ]
	inputPic[ i ] = candInputPic[ candIdx ]		(xx)
	inputPresentFlag[ i ] = inputCandPresentFlag[ candIdx ]
}

–	For each value of i in the range 0 to numInputPics − 1, inclusive, it It is a requirement of bitstream conformance that when inputPresentFlag[ i ] is equal to 0 and nnpfc_input_pic_output_flag[ i ] is equal to 1, the value of nnpfa_output_flag[ idx ] shall be equal to 0 for the value of idx such that InpIdx[ idx ] is equal to i.

# Examples

## Example 1

Description of example 1: An interpolated picture is enhanced since the picture rate upsampling NNPF (also called the interpolator NNPF) has low complexity and produces relatively poor picture quality. Thus, a visual enhancement filter is to be applied only on the interpolated pictures, but not the cropped decoded pictures. The picture rate upsampling NNPF inputs two pictures and outputs one interpolated picture. The visual enhancement NNPF inputs one picture and outputs the resulting filtered picture.

Realization with option a:

- An SPO SEI message is encoded, including:

  - NNPFC and/or NNPFA SEI message types for the picture rate upsampling NNPF

  - NNPFA SEI message type and optionally NNPFC SEI message type for the visual enhancement NNPF

- NNPFC and NNPFA SEI messages for the picture rate upsampling NNPF are encoded.

- NNPFC SEI message(s) for the visual enhancement NNPF are encoded.

- PON SEI messages, each including an NNPFA SEI message for the visual enhancement NNPF are encoded. For each pair of consecutive pictures in output order that are used as input to the picture rate upsampling NNPF, the PON SEI message is present in the latter picture in output order. The PON-nested NNPFA SEI message has:

  - nnpfa_input_selected_pics_flag equal to 1

  - nnpfa_num_input_pics_minus1 equal to 0

  - nnpfa_input_pic_skip_count[ 0 ] equal to 1; thus, the NNPFA SEI message does not apply to the picture corresponding to the current picture where the NNPFA SEI message is present but to the previous picture in output order, which is the interpolated picture.

Realization with option b:

- An SPO SEI message is encoded, including:

  - NNPFC and/or NNPFA SEI message types for the picture rate upsampling NNPF

  - NNPFEA SEI message type and optionally NNPFC SEI message type for the visual enhancement NNPF

- NNPFC and NNPFA SEI messages for the picture rate upsampling NNPF are encoded.

- NNPFC SEI message(s) for the visual enhancement NNPF are encoded.

- PON SEI messages, each including an NNPFEA SEI message for the visual enhancement NNPF are encoded. For each pair of consecutive pictures in output order that are used as input to the picture rate upsampling NNPF, the PON SEI message is present in the latter picture in output order. The PON-nested NNPFEA SEI message has:

  - nnpfea_input_selected_pics_flag equal to 1

  - nnpfea_num_input_pics_minus1 equal to 0

  - nnpfea_input_pic_skip_count[ 0 ] equal to 1; thus, the NNPFEA SEI message does not apply to the picture corresponding to the current picture where the NNPFEA SEI message is present but to the previous picture in output order, which is the interpolated picture.

## Example 2

Description of example 2: An encoder finetunes a quality-enhancement NNPF based on interpolated pictures of a random access segment and sends an NNPF update through an NNPFC SEI message. En encoder indicates the use of the base quality-enhancement NNPF for the cropped decoded pictures and the updated NNPF for the interpolated pictures. The processing order is visual quality NNPF for the cropped decoded pictures, followed by the picture rate upsampling, followed by the visual quality NNPF for the interpolated pictures.

Realization with option a:

- An SPO SEI message is encoded, including:

  - NNPF 1: NNPFC and/or NNPFA SEI message types for the visual enhancement NNPF

  - NNPF 2: NNPFA SEI message type and optionally NNPFC SEI message type for the picture rate upsampling NNPF

  - NNPF 3: NNPFA SEI message type and optionally NNPFC SEI message type for the visual enhancement NNPF.

- An NNPFC SEI message specifying the base visual enhancement NNPF (NNPF 1) is encoded.

- An NNPFC SEI message specifying the picture rate upsampling NNPF (NNPF 2) is encoded.

- NNPFC SEI message(s) defining an update (NNPF 3) to the base visual enhancement NNPF is encoded.

- NNPFA SEI message(s) for the base visual enhancement NNPF are encoded. These NNPFA SEI messages are not included in a PON SEI message and thus apply to the cropped decoded pictures and have nnpfa_target_base_flag equal to 1.

- PON SEI messages, each including an NNPFA SEI message for the picture rate upsampling are encoded. nnpfa_input_selected_pics_flag, nnpfa_num_input_pics_minus1, and nnpfa_input_pic_skip_count[ 0 ] are not present in the PON-nested NNPFA SEI message. Thus, the NNPFA SEI message applies the picture rate upsampling with quality-enhanced cropped decoded pictures as input.

- PON SEI messages, each including an NNPFA SEI message for the visual enhancement NNPF are encoded. For each pair of consecutive pictures in output order that are used as input to the picture rate upsampling NNPF, the PON SEI message is present in the latter picture in output order. The PON-nested NNPFA SEI message has:

  - nnpfa_target_base_flag equal to 0.

  - nnpfa_input_selected_pics_flag equal to 1

  - nnpfa_num_input_pics_minus1 equal to 0

  - nnpfa_input_pic_skip_count[ 0 ] equal to 1; thus, the NNPFA SEI message does not apply to the picture corresponding to the current picture where the NNPFA SEI message is present but to the previous picture in output order, which is the interpolated picture.

Realization with option b: similar to the realization with option a but NNPFEA SEI messages are used for NNPF 3.

## Example 3

Description of example 3: An interpolator NNPF generates three pictures based on two input pictures, e.g., generates pictures 1, 2, 3 based on pictures 0 and 4. It is likely that the inner-most generated picture, e.g., generated picture 2, is of lower quality than the other generated pictures in the list of generated pictures, e.g., than generated pictures 1 and 3. In order to enhance only the inner-most generated picture, e.g., generated picture 2, an encoder would need to indicate the use of a quality-enhancement NNPF for that inner-most generated picture only.

Realization with option a:

- An SPO SEI message is encoded, including:

  - NNPF 1: NNPFC and/or NNPFA SEI message types for the picture rate upsampling NNPF

  - NNPF 2: NNPFA SEI message type and optionally NNPFC SEI message type for the visual enhancement NNPF

- NNPFC and NNPFA SEI messages for the picture rate upsampling NNPF are encoded.

- NNPFC SEI message(s) for the visual enhancement NNPF are encoded.

- For NNPF 2, PON SEI messages, each including an NNPFA SEI message for the visual enhancement NNPF are encoded. For each pair of consecutive pictures in output order that are used as input to the picture rate upsampling NNPF, the PON SEI message is present in the latter picture in output order. The PON-nested NNPFA SEI message has:

  - nnpfa_input_selected_pics_flag equal to 1

  - nnpfa_num_input_pics_minus1 equal to 0

  - nnpfa_input_pic_skip_count[ 0 ] equal to 2; thus, the NNPFA SEI message applies to the interpolated picture that appears 2 pictures before the current picture, in output order (so, skipping both the current picture and the previous picture that is an interpolated picture).

Realization with option b: similar to the realization with option a but NNPFEA SEI messages are used for NNPF 2.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).