| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0074-v1 |
| --- | --- |

| Title: | EE1-4.1: Multiple scaling ratios coding for NNVC with NNSR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhuoyi Lv Chuan Zhou | Email: | zhuoyi.lv@vivo.com chuan.zhou@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of EE1-4.1 on top of NNVC9.0. Two sub-tests were conducted as follows:

- EE1-4.1.1: Adaptive resolution strategy to support multi-scaling ratios (s=2 && s=1.5) with RPR.

- EE1-4.1.2: Unified NNSR for adaptive resolution strategy (EE1-4.1.1) to support multi-scaling ratios (s=2 && s=1.5).

The overall BD-rate savings in terms of PSNR are:

- EE1-4.1.1: AI {-0.45%, 3.15%, 3.20%} and RA {-0.53%, 2.34%, 2.72%};

- EE1-4.1.2: AI {-0.80%, 2.85%, 3.04%} and RA {-0.80%, 3.06%, 2.57%}.

The overall BD-rate savings in terms of MS-SSIM are:

- EE1-4.1.1: AI {-2.15%, 2.90%, 2.74%} and RA {-1.97%, 2.15%, 2.04%};

- EE1-4.1.2: AI {-2.27%, 2.21%, 2.23%} and RA {-2.05%, 1.46%, 1.42%}.

Introduction

GOP-based RPR encoder control has been available for VTM since VTM-19.0 and for ECM since ECM-12.0, respectively. It can be used to adapt to transmission capability and to improve coding efficiency at low bitrates, which can be configured to switch between the source resolution and one of the following reduced resolutions: 1/2, 2/3, and 4/5, whilst at the same time controlling a QP offset [2].

NNVC9.0 allows the encoder to adaptively decide whether i-th GOP will be down-scaled with a ratio of 2 by comparing the down-up scaled PSNR for the first picture of i-th GOP with a threshold. However, current NNVC does not have such functionality to support switching within multiple scaling ratios. It was asserted that both subjective quality (MS-SSIM) and objective quality (PNSR) can be improved by the previous contributions on this topic, JVET-AC0078 [3] and JVET-AB0102 [4].

This contribution proposes to add such functionality to NNVC. Compared to previous contributions, three scale factors {x2.0, x1.5, x1.0} are supported. In addition, the adaptive resolution strategy and QP offsets are designed for NNVC in order to achieve bit-rate alignment.

Proposed method

EE1-4.1.1: Adaptive resolution strategy to support multi-scaling ratios (s=2 && s=1.5) with RPR

The PSNR is calculated between original and down-up scaled picture using down-sampling filters in RPR with a ratio of 2.0, where the picture is the first picture of i-th GOP.

Figure 1 Flowchart of encoder strategy

- For RA, if PSNRU>thrUV and PSNRV>thrUV, and PSNRY>thrY, scaling ratio is set equal to 2.0.

For AI, if PSNRU>thrUV and PSNRV>thrUV, scaling ratio is set equal to 2.0.

- else

  - if PSNRY>(basePSNR)-QPi-BaseQP*0.5, scaling ratio is set equal to 2.0.

  - else if PSNRY>(basePSNR-offsetPSNR)-QPi-BaseQP*0.5, scaling ratio is set equal to 1.5.

  - else, scaling ratio is set equal to 1.0.

In this contribution, basePSNR is set equal to 47 and 44 for RA and AI, respectively. baseQP is set equal to 32. OffsetPSNR is set equal to 2 and 3 for RA and AI, respectively.

According to selected scaling ratio for i-th GOP, QP offset is set equal to -5 and -4 for 2.0x and x1.5x respectively.

EE1-4.1.2: Unified NNSR for adaptive resolution strategy (EE1-4.1.1) to support multi-scaling ratios (s=2 && s=1.5).

The architecture of the proposed NNSR with low complexity is shown in Figure 2.

Scaling ratio (Sr) is set as an input to the model:

- For scaling ratio is 2.0x, Sr = 1;

- For scaling ratio is 1.5x, Sr = 2;

- For scaling ratio is 1.0x, Sr = 0.

Figure 2 Network structure

Training details

Table 1 Network information for models in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | A100 |
|  | Framework: | PyTorch v2.1.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 60 |
|  | Batch size: | 128 |
|  | Training time: | ~7 days |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn on NNLF-LOP2 and NNIntra |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128 |
|  | Learning rate: | 4e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Other information: |  |

Inference

The network information in the inference stage is provided in Table 2.

Table 2 Network information for the proposed models in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 0.05M |
|  | Total Parameter Number | 0.05M |
|  | Parameter Precision (Bits) | 32 (F) |
|  | Memory Parameter (kB) | 19.6 kB |
|  | Multiply Accumulate (MAC) | 4.7kMAC/pixel |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers | 0 |
|  | Batch size: | 1 |
|  | Patch size | 256 |

Experimental results

The proposed method is tested on top of NNVC9.0, according to the common test condition [5].

Table 3 EE1-4.1-1

|  | AI Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| Class A1 | -1.42% | 4.92% | 5.32% | -4.49% | 5.59% | 5.74% |
| Class A2 | -1.29% | 14.00% | 13.90% | -8.43% | 11.81% | 10.70% |
| Overall | -0.45% | 3.15% | 3.20% | -2.15% | 2.90% | 2.74% |
|  | RA Main10 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| Class A1 | -2.64% | 1.88% | 4.10% | -5.82% | 0.91% | 1.98% |
| Class A2 | -0.01% | 9.80% | 9.50% | -4.02% | 9.85% | 8.22% |
| Overall | -0.53% | 2.34% | 2.72% | -1.97% | 2.15% | 2.04% |

Table 4 EE1-4.1-2

|  | AI Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| Class A1 | -1.59% | 6.97% | 7.77% | -4.52% | 4.83% | 4.89% |
| Class A2 | -3.22% | 10.10% | 10.49% | -9.13% | 8.44% | 8.49% |
| Overall | -0.80% | 2.85% | 3.04% | -2.27% | 2.21% | 2.23% |
|  | RA Main10 |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y msssim | U msssim | V msssim |
| Class A1 | -2.97% | 7.68% | 6.23% | -5.93% | -0.64% | 0.46% |
| Class A2 | -1.03% | 7.63% | 6.63% | -4.29% | 7.92% | 6.62% |
| Overall | -0.80% | 3.06% | 2.57% | -2.05% | 1.46% | 1.42% |

Conclusion

This contribution proposes to add the multi-scaling ratio selection functionality to NNVC, to enable further improvement of coding efficiency and flexibility of NN-based super-resolution.

It is shown that both subjective and objective qualities can be improved by the proposal. So it is recommended to adopt it into NNVC.

References

- C. Zhou, Z. Lv, “EE1-related: Multiple scaling ratios coding for NNVC with NNSR”, JVET-AH0100.

- Adrian Browne, Yan Ye, Seung Hwan Kim, “Algorithm description for Versatile Video Coding and Test Model 19 (VTM19)”, JVET-AC2002.

- J. Nam, S. Yoo, J. Lim, S. Kim, “EE1-2.1: Updates on RPR encoder and post-filter”, JVET-AC0078.

- J. Nam, S. Yoo, J. Lim, S. Kim, “AHG11/EE1-related: Updates on RPR encoder and filters”, JVET-AB0102.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).