| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0082 |
| --- | --- |

| Title: | EE2-3.3: GPM with inter prediction and IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang, Na Zhang, Kai Zhang, Li Zhang (Bytedance) Luhang Xu, Yue Yu, Haoping Yu, Dong Wang (OPPO) | Tel: Email: | wangyang.cs@bytedance.com xuluhang@oppo.com |
| Source: | Bytedance Inc., OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results for EE2 Test 3.3. In Test 3.3a, GPM with inter prediction and intra block copy (GPM-inter-IBC) is proposed. Test 3.3b is the combination of Test 3.2 and Test 3.3a. On top of ECM-13.0, the simulation results are reported as below:

Test 3.3a:

RA: Class F {-0.02%, 0.00%, -0.15%; 100.4%, 100.8%}, Class TGM {-0.51%, -0.35%, -0.39%; 98.8%, 100.5%};

LDB: Class F{-0.02%, 0.29%, 0.27%; 100.1%, 100.2%}, Class TGM {-0.71%, -0.51%, -0.62%; 100.0%, 99.9%}.

Test 3.3b (Test 3.2 + Test 3.3a):

RA: Class F {-0.03%, 0.05%, -0.04%; 99.6%, 99.4%}, Class TGM {-1.07%, -0.92%, -0.98%; 100.5%, 100.8%};

LDB: Class F{-0.27%, -0.24%, -0.31%; 99.9%, 99.7%}, Class TGM {-1.50%, -1.44%, -1.64%; 99.4%, 100.7%}.

Introduction

GPM with inter prediction and intra block copy (GPM-inter-IBC) was proposed in JVET-AH0091 [1], which was established as Test 3.3a in JVET-AH EE2 [2]. In GPM-inter-IBC, the two geometric sub-partitions are predicted using inter prediction and IBC, individually. To generate the prediction signal of IBC, an IBC candidate list is constructed, and the index of the selected block vector is signalled. The prediction signal of IBC for one sub-partition can be blended with regular GPM, GPM-MMVD, and GPM-affine for the other sub-partition.

In Test 3.2 [3], temporal BV candidates are added to the IBC merge/AMVP list after the non-adjacent BV candidates. The temporal BVs are derived from collocated positions and shifted collocated positions in the collocated pictures. The IBC merge/AMVP list size, the number of processes applied for template matching and the number of full RDO processes are kept unchanged in the proposed method.

Test 3.3b is the combination of Test 3.2 and Test 3.3a.

Simulation results

EE2 Test 3.3a and 3.3b are implemented on top of ECM-13.0 [4]. Simulations are performed following the common test conditions of EE2 [5]. The simulation results compared to ECM-13.0 are shown in the following tables.

Table 1: Test results of Test 3.3a

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.02% | 0.00% | -0.15% | 100.4% | 100.8% | 100.2% | 100.3% |
| Class TGM | -0.51% | -0.35% | -0.39% | 99.8% | 100.5% | 100.6% | 100.4% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.02% | 0.29% | 0.27% | 100.1% | 100.2% | 100.7% | 99.7% |
| Class TGM | -0.71% | -0.51% | -0.62% | 100.0% | 99.9% | 100.5% | 101.0% |

Table 2: Test results of Test 3.3b

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.03% | 0.05% | -0.04% | 99.6% | 99.4% | 100.2% | 99.9% |
| Class TGM | -1.07% | -0.92% | -0.98% | 100.5% | 100.8% | 100.3% | 99.3% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.27% | -0.24% | -0.31% | 99.9% | 99.7% | 100.2% | 100.0% |
| Class TGM | -1.50% | -1.44% | -1.64% | 99.4% | 100.7% | 100.7% | 97.4% |

Conclusions

In this contribution, the test results of EE2 Test 3.3a on GPM with inter prediction and intra block copy (GPM-inter-IBC), and Test 3.3b which is the combination of Test 3.2 and Test 3.3a are reported. It is recommended to adopt Test 3.3b into ECM.

Reference

- Y. Wang, K. Zhang, and L. Zhang, “Non-EE2: Geometry partitioning mode with inter prediction and intra block copy,” Document of Joint Video Experts Team, JVET-AH0091, Rennes, FR, 17–24 April 2024.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-AH2024, Rennes, FR, 17–24 April 2024.

- L. Xu, Y. Yu, H. Yu, D. Wang, N. Zhang, K. Zhang and L. Zhang, “AHG12: Temporal BV for IBC merge list construction,” Document of Joint Video Experts Team, JVET-AH0059, Rennes, FR, 17–24 April 2024.

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).