| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0083 |
| --- | --- |

| Title: | EE2-3.8: GPM-affine with MMVD and TM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yang Wang1, Kai Zhang2, Li Zhang2 1Fangheng Building, No.27, 3rd Ring West Rd, Haidian District, 100098, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | wangyang.cs@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution reports the test results for EE2 Test 3.8 on GPM-affine harmonized with GPM-MMVD and GPM-TM. In Test 3.8a, GPM-affine with GPM-MMVD is proposed. In Test 3.8b, GPM-affine with GPM-TM is proposed. Test 3.8c is the combination test of Test 3.8a and Test 3.8b. On top of ECM-13.0, the simulation results are reported as below:

Test 3.8a:

RA: {};

LDB: {}.

Test 3.8b

RA: {};

LDB: {}.

Test 3.8c:

RA: {};

LDB: {}.

Introduction

GPM-affine with GPM-MMVD (GPM-affine-MMVD) and GPM-affine with GPM-TM (GPM-affine-TM) were proposed in JVET-AH0222 [1], which were established as Test 3.8a and Test 3.8b in JVET-AH EE2 [2]. In Test 3.8a, an MV offset is added to all CPMVs of an affine candidate and the modified CPMVs are used for predicting a GPM sub-partition. In test 3.8b, an affine candidate could be refined by template matching using the same method as in affine mode. A GPM sub-partition can be predicted by GPM-affine-TM or GPM-TM. Test 3.8c is the combination test of Test 3.8a and Test 3.8b.

Simulation results

EE2 Test 3.8a, Test 3.8b, and Test 3.8c are implemented on top of ECM-13.0 [3]. Simulations are performed following the common test conditions of EE2 [4]. The simulation results compared to ECM-13.0 are shown in the following tables.

Table 1: Test results of Test 3.8a

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 2: Test results of Test 3.8b

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Table 3: Test results of Test 3.8c

|  | RA |  |  |  |  | LDB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.04% | -0.04% | -0.13% |  |  |  |  |  |  |  |
| Class C | -0.07% | -0.02% | -0.01% |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  | -0.09% | 0.15% | 0.13% |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, the test results of EE2 Test 3.8a on GPM-affine with GPM-MMVD (GPM-affine-MMVD), Test 3.8b on GPM-affine with GPM-TM (GPM-affine-TM), and Test 3.8c which is the combination test of Test 3.8a and Test 3.8b are reported. It is recommended to adopt Test 3.8c into ECM.

Reference

- Y. Wang, K. Zhang, and L. Zhang, “Non-EE2: GPM-Affine with MMVD and TM,” Document of Joint Video Experts Team, JVET-AH0222, Rennes, FR, 17–24 April 2024.

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Strom, F. Wang, M. Winken, X. Xiu, and K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” Document of Joint Video Experts Team, JVET-AH2024, Rennes, FR, 17–24 April 2024.

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0.

- M. Karczewicz and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-AF2017, Hannover, DE, 13–20 October 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).