| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0085-v2 |
| --- | --- |

| Title: | Non-EE2: Matrix-based position dependent intra prediction for TIMD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Kidani Haruhisa Kato Kei Kawamura (KDDI) 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | yo-kidani@kddi.com |
| Source: | KDDI Corporation (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes a matrix-based position-dependent intra prediction (PDP) for template-based intra-mode derivation (TIMD). PDP was a newly adopted intra prediction in JVET-AH meeting. PDP partially replaces the conventional intra prediction modes (IPMs) of the regular intra prediction with alternative IPMs using matrix of weights, defined for a block shape and IPMs. However, the conventional IPMs derived from TIMD are not replaced by PDP in the current ECM. This contribution proposes partially replacing the conventional IPMs with PDP in TIMD and employing PDP in template cost calculation for TIMD. Furthermore, the proposed method is leveraged to the inter prediction tools using TIMD, such as CIIP and GPM intra.

On top of the ECM-13.0, simulation results {Y, U, V, EncT, DecT} for overall test sequences in CTC are reported below:

AI: {-0.07%	, -0.07%, -0.12%, 100.6%, 104.8%};

RA: {-0.06%, -0.04%, -0.02%, 100.2%	, 100.7%};

Introduction

ECM-13.0 [1][2] has newly adopted matrix-based position-dependent intra prediction (PDP) proposed by JVET-AH0209 [2]. PDP partially replaces a part of the conventional intra prediction modes (IPMs) of the regular intra prediction with alternative IPMs using a matrix of weights. Specifically, PDP is only used for mode 0, 1, and (2+2*k) angular mode for coding blocks with width and height up to 16 samples; otherwise, PDP is only used for mode 0, 1, and (2+4*k). The matrix weights are defined for a block shape and IPMs. The derived matrix weights are applied to L-shaped causal reference template to generate final prediction samples.

However, in ECM-13.0, the conventional IPMs derived from template-based intra mode derivation (TIMD) [2] are not replaced by PDP, which implies room to enhance TIMD.

Proposed method

This contribution proposes a PDP for TIMD, i.e., partially replacing the conventional IPMs with PDP in TIMD and employing PDP in template cost calculation for TIMD. Furthermore, the proposed method is leveraged to the inter prediction tools using TIMD, such as CIIP and GPM intra. Details of the proposed method are as follows:

- Signalling is kept as the conventional TIMD.

- Angular modes replaced by PDP in TIMD are different from those in the regular intra prediction since the number of IPMs in TIMD is double of regular intra prediction. Specifically, PDP is only used for mode 0, 1, and (2+4*k) for coding blocks with width and height up to 16 samples; otherwise, PDP is only used for mode 0, 1, and (2+8*k).

- Template cost for TIMD is calculated by applying PDP to reference samples of the template, as shown in Figure 1.

- PDPC is not applied to the PDP samples in TIMD, just as PDPC is not applied to the PDP samples in regular intra prediction.

Figure 1 PDP (left) and PDP for template cost calculation in TIMD (right)

Experimental results

The proposed method was implemented on top of ECM-13.0 [1] and tested according to the common test condition of all intra and random access configurations as specified in the JVET-AF2017 [4] and JVET-AH2024 [5]. Experimental results are summarized below. More detailed results can be found in the attached spreadsheets.

Table 2 Experimental results of the proposed method over ECM-13.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.10% | -0.07% | -0.02% | 100.3% | 103.1% | 100.1% | 100.0% |
| Class A2 | -0.08% | -0.07% | -0.02% | 100.7% | 102.9% | 97.5% | 99.9% |
| Class B | -0.06% | -0.12% | -0.22% | 100.7% | 105.1% | 98.9% | 100.0% |
| Class C | -0.03% | -0.11% | -0.06% | 100.5% | 106.4% | 100.0% | 100.0% |
| Class E | -0.08% | 0.04% | -0.21% | 101.0% | 105.8% | 102.0% | 100.0% |
| Overall | -0.07% | -0.07% | -0.12% | 100.6% | 104.8% | 99.6% | 100.0% |
| Class D | -0.02% | 0.10% | -0.10% | 100.7% | 107.0% | 100.1% | 100.0% |
| Class F | -0.02% | -0.04% | 0.02% | 100.5% | 102.7% | 103.3% | 99.9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.13% | -0.15% | 100.2% | 100.9% | 99.9% | 100.0% |
| Class A2 | -0.04% | -0.19% | -0.10% | 100.4% | 100.6% | 100.1% | 99.6% |
| Class B | -0.06% | 0.08% | -0.08% | 100.2% | 100.6% | 99.9% | 99.9% |
| Class C | -0.06% | -0.02% | 0.21% | 100.1% | 100.7% | 99.6% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.06% | -0.04% | -0.02% | 100.2% | 100.7% | 99.9% | 99.9% |
| Class D | 0.01% | -0.34% | -0.08% | 100.3% | 100.7% | 99.9% | 100.0% |
| Class F | 0.00% | 0.11% | 0.02% | 100.2% | 100.3% | 100.1% | 100.1% |

Conclusion

This contribution proposes a matrix-based position-dependent prediction (PDP) for template-based intra-mode derivation (TIMD). The proposed method brings some coding gains compared to ECM-13.0. The decoding runtime increase observed in all intra configuration can be reduced by simplifying the mode derivation process of TIMD using PDP. It is recommended that the proposed method be adopted into the next EE.

Reference

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-13.0

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 13 (ECM 13),” JVET-AH2025, April 2024.

- B. Ray, H. Wang, P. Garus, V. Seregin, M. Karczewicz, S. Eadie, G. Verba, “EE2-2.13: Matrix based intra prediction replacing conventional intra modes,” JVET-AH0209, April 2024.

- M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October. 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AH2024, April 2024.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).