| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0095-v1 |
| --- | --- |

| Title: | EE1-related: Improved gain for NN in-loop filters using early cropping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Mitra Damghanian, Du Liu and Per Wennersten | Email: | {jacob.strom, mitra.damghanian, du.liu, per.wennersten}@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution is a follow-up to JVET-AH0195 that proposes to gradually reduce the patch size of the LOP3 neural network-based in-loop filter. Whereas JVET-AH0195 reduces the patch size in the last few layers of the network, this contribution instead proposes to do it earlier. It is claimed that cropping earlier reduces complexity further in terms of kMAC/pixel. In this proposal, the saved complexity is then spent by widening and deepening the LOP3 models from N Y=14 to N Y=15 backbone blocks and from C1Y=144 to C1Y=176 channels. The result is a model with a claimed complexity of 16.83 kMAC/pixel, which is similar to (but smaller than) that of LOP3 (16.90 kMAC/pixel). The deepening and widening of the network mean that the number of parameters goes up from 0.20M (LOP3) to 0.25M (proposed), and that the network needs to be retrained. The reported int16 results over NNVC-9.1 are

RA-BDR 	-0.26% (Y)	-1.33%* (U)	-1.00% (V)
AI-BDR	-0.26% (Y)	-1.21% (U)	-1.08% (V)		
LD-BDR	-0.23% (Y)	-0.85% (U)	-0.59%. (V)

It is proposed to adopt the method into the next round of EE1.

# Introduction

The current neural network-based in-loop filters (HOP, LOP and VLOP) currently operate as illustrated in Figure 1. To improve a 128x128 patch of the picture, a larger patch of 144x144 samples (8 samples on each side) is inputted to the network.

Figure 1. The current operation of the neural network-based in-loop filters in JVET. A patch of 144x144 samples is processed and results in an NN output of 144x144 samples, which is then cropped before adding with the input to produce the final output.

An output of the same size (144x144) is produced, and this is then cropped to 128x128 to modify the block. This process is wasteful since the neural network calculates a lot of values that are not used: Only values in the middle 128x128 area (marked with the dashed rectangle) of the NN are used for final output, the ones outside are simply discarded in the subsequent cropping step. In JVET-AH0195 it was suggested to not perform the unnecessary calculations needed to obtain the margin samples that are thrown away. That approach would lower the complexity in terms of kMAC/pixel by about 0.8% and produce identical results with LOP3. The method in this contribution is inspired by JVET-AH0195 but goes further by cropping earlier and thus saves more complexity.

# Proposed method

Calculating the margin samples is expensive. As an example, the first 1x1 convolution layer in the backbone block in LOP3 uses Kx*Ky*W*H*Cin*Cout = 1*1*36*36*144*32 = 5 971 968 multiplications. Here, 36x36 samples are needed to cover the 144x144 samples of the output. If the spatial size could instead be brought down to 32x32 samples (that would cover the necessary 128x128 final output samples), this could be lowered to 1*1*32*32*144*32 = 4 718 592 multiplications, a reduction of 21%. The main idea with this proposal is to show that that complexity is better spent elsewhere, namely by widening the number of channels and adding more backbone blocks.

Fig. 2. The architecture of the LOP3 filter, including patch sizes.

The current architecture of the LOP3 filter is shown in Figure 2. A patch of size 144x144 (a) is inputted to the network. After DCT it has size 72x72 (b), and after convolution layers with stride 2 it has size 36x36 (e). This size is now kept throughout the backbone layers (f through j) and the tail (k) until the PixelShuffle layer, which increases the resolution to 72x72 (l). After IDCT the patch size is 144x144 and now cropping is performed to obtain a size of 128x128 that is added to the input which was also cropped to this size (o and p).

Fig. 3. The proposed architecture of the proposed method. Changes highlighted in blue.

The proposed architecture is shown in Figure 3. It is the same up to the point (f), which is the input to the first backbone layer, and where the size is 36x36. Instead of a regular backbone block (which we will refer to as backbone block B), a modified backbone block “backbone block A” is used. This uses pad=(0,0) in the sCONV layers (both the 1x3 and 3x1) which has the effect of shrinking the size by two samples in both dimensions. Hence the output of the first backboneA block at (g) is 34x34 samples. This is followed by another backbone block of type A, shrinking the size to 32x32 (h). We have now reached the correct size, and no further shrinking is necessary. Therefore, the remaining backbone blocks are of the regular type B, which does not change the size of the input. After pixelshuffle the size is 64x64 (l), and after IDCT the size is 128x128 (m), which means that the last crop layer between (m) and (n) is no longer necessary.

# Saving complexity and then spending it

By reducing the size from 36x36 to 32x32 after the first two backbone blocks, complexity in terms of kMAC/pixel is considerably lowered. This is due to the fact that every convolutional layer after these two backbone layers operate at the smaller size, and every convolutional layer will therefore use (32*32)/(36*36)-1 = 21% fewer MACs. Counted on the entire model, the complexity goes down from 16.90 kMAC/pixel to 14.11 kMAC/pixel, a change of -16.5%. This can be compared to the -0.8% change that was obtained in JVET-AH0195, where the patch size was also lowered, but much later in the network.

To evaluate the performance of this method at equal complexity to LOP3, we then increase the depth (the number of backbone blocks) as well as the width (the CY1 parameter) of the network. In summary we go from

NY=14 (LOP3)  NY=15 (current proposal), and

CY1=144 (LOP3)  CY1=176 (current proposal).

Out of the NY=15 backbone blocks, the first two are of type A (that makes the patch size smaller) and the last 13 are of type B (that keep the patch size the same). The chroma branch of the network keeps the values

NUV=4 and

CUV1=128,

but the architecture of the chroma branch is changed compared to LOP3 due to the fact that the first two of the backbone blocks are also of type A.

With these increases of the luma branch, the complexity goes back to 16.83 kMAC/pixel which is slightly smaller than the 16.90 kMAC/pixel of LOP3. See the accompanying excel file for a detailed calculation of the kMAC/pixel. The thop library has been used to verify the kMAC/pixel and parameter count results from the excel file.

# Training and inference information

Since the network architecture has changed, it is necessary to retrain the network. This is done the same way as for LOP3, except that a loss function weighting of Y=6, UV=2 is used instead of Y=12, UV=2 that was used for LOP3.

The inference software is a modified version of NNVC9.0rc1, where the output of the neural network is assumed to be the correct size (e.g., 128x128) and it is hence not cropped.

| Table 1. Network Information for the proposed model in training. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v2.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage3: 60 |
|  | Batch size: | Stage3: 64 |
|  | Loss function: | L1 and MSE |
|  | Training time: | Stage3: 5 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | The LOP stage 3 dataset |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 (input), 128x128 (output) |
|  | Learning rate: | Stage3: 4e-4, milestones [41, 51, 55] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

| Table 2. Network Information for the model in inference. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 248037 |
|  | Parameter Precision (Bits) | fp32, int16 |
|  | Memory Parameter (MB) | fp32: 0.95MB, int16: 0.47MB |
|  | Multiplay Accumulate (MAC) | 16.83 kMAC/pixel |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

# Results

Proposed int16 model including NNintra over the anchor NNVC9.1 CTC (including int16 LOP3 and NNintra):

## Random access:

(EncT times are not reliable)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 Intra+LOP3 int16 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.12% | -1.84% | -1.18% | 88% |  |
| Class A2 | -0.23% | -1.93% | -0.75% | 95% |  |
| Class B | -0.31% | -1.40% | -1.22% | 107% |  |
| Class C | -0.33% | -0.43% | -0.79% | 122% |  |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -1.33% | -1.00% | 104% |  |
| Class D | -0.56% | -0.99% | -0.98% | 132% |  |
| Class F | -0.22% | -0.45% | -0.02% | 135% |  |

## Low delay B

(EncT times are not reliable)

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 Intra+LOP3 int16 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -1.63% | -1.58% | 68% |  |
| Class C | -0.42% | 0.23% | 0.60% | 80% |  |
| Class E | -0.06% | -0.97% | -0.54% | 100% |  |
| Overall | -0.23% | -0.85% | -0.59% | 79% |  |
| Class D | -0.33% | 0.99% | 0.92% | 90% |  |
| Class F | -0.23% | 1.13% | -1.83% | 77% |  |

## All Intra

(EncT times are not reliable)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1 Intra+LOP3 int16 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.24% | -1.45% | -0.45% | 93% |  |
| Class A2 | -0.21% | -1.45% | -0.76% | 92% |  |
| Class B | -0.23% | -1.24% | -1.25% | 92% |  |
| Class C | -0.26% | -0.82% | -0.85% | 105% |  |
| Class E | -0.37% | -1.17% | -2.01% | 101% |  |
| Overall | -0.26% | -1.21% | -1.08% | 97% |  |
| Class D | -0.38% | -0.65% | -0.48% | 107% |  |
| Class F | -0.21% | -0.22% | -0.11% | 103% |  |

# Conclusion

This contribution describes a method to increase the gain of the LOP3 filter. The method works by reducing the 8-pixel margin on each side of the patch early in the network. This reduced patch size saves complexity, and this is instead spent on deepening and widening the network to match the complexity of the LOP3 filter. The resulting BDR figures for RA are -0.26% (Y), -1.33% (U) -1.00% (V), for LDB they are -0.23% (Y), -0.85% (U), -0.59% (V) and for AI they are -0.26% (Y), -1.21% (U) and -1.08% (V). It is proposed to adopt this contribution into the next round of EE1.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).