| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0099-v1 |
| --- | --- |

| Title: | AHG9: Showcases on the implementation of applying separate film grain models in different regions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Shaowei Xie, Ping Wu, Ying Gao, Yaxian Bai, Menghu Jia, Weihong Niu, Cheng Huang | Email: | xie.shaowei@zte.com.cn, ping.wu@zte.com.cn, huang.cheng5@zte.com.cn |
| Source: | ZTE |  |  |

_____________________________

- Abstract

This contribution provides some showcase examples on the implementation of applying separate film grain models in different regions, where the model adaptation proposed in JVET-AH0166 was adopted into the VSEI TuC at the April meeting, as recorded in the meeting report [1]. In the showcase examples, we apply two film grain models (i.e., Frequency filtering based method and Auto-regression based method) simultaneously in two separate regions of each video.

Introduction

At the 33rd JVET meeting, the Film Grain Regions Characteristics (FGR) SEI message was documented into the VESI TuC [2], where a list of film grain regions are described and each region is associated with a set of intensity intervals. In order to improve the adaptive ability of this SEI message, the film grain model adaptation allowing separate “fgr_model_id” to be applied in different regions, proposed in JVET-AH0166 [3], was adopted into the FGR SEI message. To confirm the benefit of this improvement, we provide some showcase examples in this proposal.

Test platform

While the VTM implementation of film grain currently only supports the Frequency filtering based method and not the Auto-regression based method, we implemented the Auto-regression model based on a third-party platform, i.e., SVT-AV1. Thus, our testing platform consists of the following two aspects:

- Frequency filtering film grain synthesis method in VTM-23.1 [4], where a configuration file is parsed to insert the film grain related SEI message into the bitstream, and the SEI message is parsed on the decoder side to carry out the film grain synthesis to output the resulted processed decoded video, corresponding to the normal FGC behavior.

- Auto-regression film grain synthesis method in SVT-AV1-v2.1.0 [5], where the film grain model parameters are estimated at the encoder side by considering the difference between the input picture and its denoised version, and the model parameters are included in the bitstream for the decoder to reproduce the film grain and add it in the reconstructed pictures. These steps are similar as to the normal FGC behavior, without parsing configuration file. The denoising of the input picture is performed using a Wiener filter. An Auto-regressive model driven by a unit variance Gaussian noise is used to model the film grain. In our test, to ensure the consistency of film grain sources as much as possible, the input video of SVT-AV1 is the output video with film grain through VTM-23.1.

A block diagram showing the main components of the testing platform is included in Figure 1 below. As shown here, once we achieve the two output videos (yuv420p, 10bits) with film grain through VTM-23.1 and SVT-AV1 respectively, FFmpeg is used to merge them to reach the visual outcome of applying separate “fgr_model_id” in different regions of the same video.

- Figure 1. Test platform of showcasing film grain model adaptation

Case illustrations

We have run some configurations illustrating the initial scenarios.

Case 1: Picture in picture

Video: EuroTruckSimulator2_1920x1080_60Hz_8bit_P420.yuv,

Source: jvet-ftp/ahg/verificationtests/candidates/sdr/hd

To display video contents more comprehensively, the picture-in-picture technology is usually used to present images from different angles at the same time. A computer-generated video indicating the picture-in-picture scenario is illustrated below.

Source video:

Output video with separate film grain models:

In this showcase, a region covering the partial picture in the upper left corner (as shown in the blue box) of the entire picture has Frequency filtering based film grain model enabled, and the rest region has Auto-regression based film grain model enabled. Similar as the observation reported in JVET-AC0199 [6], the synthesized grain with Frequency filtering based model is coarse-grained, while the synthesized grain with Auto-regression based model is fine-grained.

Case 2: Windowed movie viewing

Video: screen_capture_1920x1080_10b_fr460to959_QP32.mp4,

Source: jvet-ftp/ahg/showcases/JVET-AH0112_FilmGrainRegion [7]

In the scenario of cloud-based rendering of virtual desktops, the entire screen in sent to a lightweight client as a single video stream. A simulated case of a windowed view of a movie is illustrated below.

Source video:

Output video with separate film grain models:

Comparing source video and output video:

In this showcase, a region covering the windowed movie (as shown in the blue box) has Frequency filtering based film grain model enabled, and the rest region has Auto-regression based film grain model enabled. As observed, the synthesized grain with Frequency filtering based model is coarse-grained, while the synthesized grain with Auto-regression based model is fine-grained.

Test Configuration

  - 4.1 VTM

FGC Configuration file

The configuration file is the same as the normal FGC SEI message.

Command line examples

EncoderApp -c lowdelayP.cfg -c sequence.cfg -c fgCharacteristics.cfg -i input.yuv --QP=27 -b str.bin

DecoderApp -b str.bin -o dec.yuv --SEIFGSFilename=dec_model0FG.yuv

  - 4.2 SVT-AV1

Command line examples

SvtAv1EncApp -i input.yuv -w 1920 -h 1080 --input-depth 10 --fps 24 --crf 22 --preset 0 --film-grain 25 --film-grain-denoise 1 -b str.ivf

SvtAv1DecApp -i str.ivf -o dec_av1FG25.yuv

Summary

Along with the development of the Showcase, internal people's viewings can be summarized: two different film grain models, which have been existed for many year with H.26x standards, showed distinguishable effects from time to time, there are split opinions on which model’s effects people will like to have, therefore, it is recommended to keep these two models in Film Grain Regions Characteristics (FGR ) SEI message and allow them to be applied into different regions in a picture.

Reference

- Jens-Rainer Ohm, Meeting Report of the 34th JVET Meeting, JVET-AH1000, Apr. 2024.

- Sean McCarthy, Jie Chen, Sachin Deshpande, Miska M. Hannuksela Hendry, Technologies under consideration for future extensions of VSEI (version 3), JVET-AG2032, Mar. 2024.

- Shaowei Xie, Ping Wu, Ying Gao, Yaxian Bai, Menghu Jia, Weihong Niu, Cheng Huang (ZTE), AHG9: Improvements to the film grain regions characteristics SEI message, JVET-AH0166, Apr. 2024.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tree/VTM-23.1?ref_type=tags.

- https://gitlab.com/AOMediaCodec/SVT-AV1/-/tree/v2.1.0?ref_type=tags.

- Xuewei Meng, Wenhao Zhang, Scott Labrozzi, AHG13: On Film Grain Synthesis Subjective Evaluation, JVET-AC0199, Jan. 2023.

- Gilles Teniou (Tencent), Philippe De Lagrange, Edouard François (InterDigital), [AHG9/AHG13] Updates on the implementation of film grain regions characteristics SEI message, JVET-AH0212, Apr. 2024.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).