| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0100-v1 |
| --- | --- |

| Title: | [AHG9] On byte alignment design in SEI processing order SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Gao Ping Wu Yaxian Bai Shaowei Xie Menghu Jia Weihong Niu Cheng Huang | Email: | {gao.ying, ping.wu, bai.yaxian, xie.shaowei, jia.menghu, niu.weihong, huang.cheng5} @zte.com.cn |
| Source: | ZTE |  |  |

_____________________________

- Abstract

This contribution is a follow-up work of last JVET meeting input document JVET-AH0134. On top of the original technical contributions in JVET-AG0105 and JVET-AH0134, further byte alignment arrangement in syntax table is discussed and more design options are given for consideration.

Introduction

In 33rd JVET meeting, it was proposed in JVET-AG0105 [1] to add a conditional (if the po_sei_importance_flag[i] is equal to 1) flag (po_sei_processing_degree_flag[i]) to specify whether or not the decoder by following the intend (sending from encoder) should ignore the entire SPO (SEI Processing Order) SEI (Supplemental Enhancement Information) message if the decoder cannot interpret or does not support the functionality indicated by any indicated SEI messages.

In 34th JVET meeting, there is a follow-up technical proposal in JVET-AH0134 [2] on the same design with introducing more supporting use cases. During the on-site discussions, some experts commented that it would be a better design if the original byte alignment nature in the original syntax table can be maintained. In this document, alternative designs are considered and discussed.

It is noted that this document is proposed on top of JVET document in JVET-AG2027 [3] “SEI processing order and processing order nesting SEI messages in VVC (draft 7)”.

Previously proposed syntax and semantics in JVET-AH0134

Proposed change of syntax of the SEI processing order SEI message

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| if(po_sei_importance_flag[ i ]) |  |
| po_sei_processing_degree_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| ...... |  |
| } |  |

Proposed semantics of the SPO SEI message in JVET-AH1034

po_sei_importance_flag[ i ] indicates the degree of importance determined by the encoder for the type of SEI message with index i.

If the decoding system cannot interpret or does not support the functionality indicated by any indicated SEI message that has po_sei_importance_flag[ i ] equal to 1, it should ignore the entire SPO SEI message process the SPO SEI message based on the value of po_sei_processing_degree_flag[i].

po_sei_processing_degree_flag[i] (when present) indicates the degree of processing for SPO SEI message by the decoder.

If the functionality indicated by the SEI message that has po_sei_importance_flag[ i ] equal to 1 cannot be interpreted or supported by the decoding system, the following applies:

- po_sei_processing_degree_flag[i] equal to 1 indicates that the decoder should ignore the entire SPO SEI message.

po_sei_processing_degree_flag[i] equal to 0 indicates that the decoder should process those SEI messages which has po_sei_processing_order[j] less than po_sei_processing_order[i] based on the preferred order in the SPO SEI message.

The issue raised in last (34th) JVET meeting is: when adding the proposed single conditional flag in above syntax table, an already well considered byte alignment structure will be changed and this may not be beneficial for bitstream parsing.

The further consideration on byte alignment in syntax table

In VVC (H.266) standard specification, for the byte alignment, the following can be found:

Supplemental enhancement information RBSP syntax

| sei_rbsp( ) { | Descriptor |
| --- | --- |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| rbsp_trailing_bits( ) |  |
| } |  |

RBSP trailing bits syntax

| rbsp_trailing_bits( ) { | Descriptor |
| --- | --- |
| rbsp_stop_one_bit /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| rbsp_alignment_zero_bit /* equal to 0 */ | f(1) |
| } |  |

It is clear that for sei_message() data bitstream, the operation will be: (1) adding rbsp_stop_one_bit /* equal to 1 */ first, and (2) if byte alignment is not fulfilled then filling a necessary number of bits (if any) rbsp_alignment_zero_bit /* equal to 0 */.

It is noted for other ISO/IEC specifications, such as H.264 and H.265, the byte alignment operations for sei_rbsp() are specified in the exact same way. In this document, the H.266 specification (syntax table) is only mentioned as the example.

One alternative design can also be considered as that in the following Table 1:

Table 1: Alternative Arrangement 1

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| if (po_sei_importance_flag[ i ]) |  |
| po_sei_processing_degree_flag[ i ] | u(1) |
| } |  |
| ...... |  |
| } |  |

It can be seen that the proposed conditional flag can be moved down in this syntax table, if this is the arrangement, the bitstream parsing process (to this sei message) before this conditional flag can be the same as before (original byte alignment nature will then be kept unchanged for [i]; once po_sei_processing_degree_flag[ j ] is in use (in [j]), in above “for loop”, for any sei with k>j parts, they will no longer be byte aligned as before).

One other consideration can be: making a separate “for loop” (second “for loop”, particularly for the conditional flag(s)) as that in the following Table 2:

Table 2: Alternative Arrangement 2

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| if (po_sei_importance_flag[ i ]) |  |
| po_sei_processing_degree_flag[ i ] | u(1) |
| } |  |
| ...... |  |
| } |  |

For a conditional flag, such as po_sei_processing_degree_flag[ i ], it is not desirable to allow it always occupy 1 bit. If such a flag occasionally appeared in the bitstream, there is always the byte alignment issues on possible syntax table structures.

Conclusion

In this contribution, the alternative syntax table arrangements are considered. There are at least two options: (1) the conditional flag po_sei_processing_degree_flag[ i ] will be indicated just after po_sei_importance_flag[ i ] (as that proposed in JVET-AH0134); (2) the conditional flag po_sei_processing_degree_flag[ i ] will be indicated just below a group of flags, including po_sei_importance_flag[ i ] in syntax table.

Since this conditional flag appeared inside a “for loop”, once this flag is indicated (in use), the byte alignment for the following SEI candidates in the same “for loop” (in a SPO SEI message processing order chain) will be affected. In general, it can be seen that the byte alignment is not very critical for parsing this syntax table, the total number of bits involved in parsing is very limited. Perhaps the original design in JVET-H0134 [2], which is to have this conditional flag po_sei_processing_degree_flag[ i ] located just after its gating flag po_sei_importance_flag[ i ] in syntax table, is a better arrangement (assuming this conditional flag is adopted).

References

- Y. Gao, Y.-X. Bai, S.-W. Xie, M.-H. Jia, C. Huang, P. Wu (ZTE), "AHG9: On the SEI processing order SEI message", JVET-AG0105.

- Y. Gao, P. Wu, Y.-X. Bai, S.-W. Xie, M.-H. Jia, W.-H. Niu, C. Huang (ZTE), "[AHG9] Improvement on the SEI processing order SEI message ", JVET-AH0134.

- G. J. Sullivan, M. M. Hannuksela, Y.-K. Wang “SEI processing order and processing order nesting SEI messages in VVC (draft 7)” JVET-AG2027

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).