| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0101-v1 |
| --- | --- |

| Title: | Non-EE2: High-level control of intra prediction methods for screen contents |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kyungyong (Kyle) Kim Ju-Hyung (John) Son Jin Sam Kwak | Email: | kyungyong.kim@wilusgroup.com |
| Source: | WILUS Inc. |  |  |

_____________________________

- Abstract

This contribution proposes to disable several methods in intra prediction for screen contents, including reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion. The proposed method is implemented on top of ECM-13.0 software, and the performance is reported as follows:

Test 1. High-level control of reference sample filtering, PDPC and intra prediction fusion

AI: { -0.12% Y, -0.03% U, 0.08% V, XXX% EncT, XXX% DecT } for class F

{ -0.33% Y, -0.28% U, -0.29% V, XXX% EncT, XXX% DecT} for class TGM

RA: { XXX% Y, XXX% U, XXX% V, XXX% EncT, XXX% DecT } for class F

{ XXX% Y, XXX% U, XXX% V, XXX% EncT, XXX% DecT } for class TGM

Test 2. High-level control of PDPC and intra prediction fusion

AI: { XXX% Y, XXX% U, XXX% V, XXX% EncT, XXX% DecT } for class F

{ -0.29% Y, -0.09% U, -0.32% V, XXX% EncT, XXX% DecT} for class TGM

RA: { -0.01% Y, -0.09% U, -0.32% V, XXX% EncT, XXX% DecT } for class F

{ XXX% Y, XXX% U, XXX% V, XXX% EncT, XXX% DecT } for class TGM

Introduction

Reference sample filtering reduces artifacts caused by edges in the reference sample. However, in screen contents, the edge can be important for generating the prediction block. Position-dependent prediction combination (PDPC) reduces prediction errors along the block boundary. Intra prediction fusion method derives predicted samples as a weighted combination of multiple predictors generated from different reference lines. While these methods provide gains in natural contents, reference sample filtering, PDPC and intra prediction fusion may not be effective for prediction of screen contents.

Proposal

In this contribution, it is proposed to disable several methods (reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion) in intra prediction for screen contents.

The application of methods for screen content is controlled by individual SPS flags, as illustrated in Fig. 1. Each SPS flag is set based on the hash block hit percentage at encoder, which is already available in ECM and used in other tools. If it is larger than a threshold, the video sequence is interpreted as screen content and several methods (reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion) are not applied in the sequence.

Figure 1. SPS flags for controlling reference sample filtering, PDPC and intra prediction fusion

Test Result

The proposed methods were implemented on top of the ECM-13.0 [1]. Simulations were performed by following the common test conditions [2]. More detailed results could be found in the attached excel sheets. The test results are summarized below.

Test result for Test 1. High-level control of reference sample filtering, PDPC and intra prediction fusion

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.12% | -0.03% | 0.08% |  |  |  |
| Class TGM | -0.33% | -0.28% | -0.29% |  |  |  |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |

Test result for Test 2. High-level control of PDPC and intra prediction fusion

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F |  |  |  |  |  |  |
| Class TGM | -0.29% | -0.31% | -0.27% |  |  |  |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class F | -0.01% | -0.09% | -0.32% |  |  |  |
| Class TGM |  |  |  |  |  |  |

Conclusion

This contribution proposes to disable several methods (reference sample filtering, PDPC (and gradient PDPC), and intra prediction fusion) in intra prediction for screen contents. It is recommended to adopt the proposed methods into the next EE2.

Reference

[1] M. Coban, and et al., “Algorithm description of Enhanced Compression Model 13 (ECM 13),” JVET-AH2025, April 2024.

[2] M. Karczewicz and Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017, October 2023.

Patent rights declaration(s)

WILUS Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).