| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0111-v1 |
| --- | --- |

| Title: | AhG11: Content-adaptive LOP3 filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maria Santamaria Ruiying Yang Francesco Cricri Miska M. Hannuksela Döne Bugdayci Sansli Antti Hallapuro Jani Lainema Honglei Zhang Nokia Technologies, Tampere, Finland | Tel: Email: | {maria.santamaria_gomez, ruiying.yang, francesco.cricri, miska.hannuksela, done.bugdayci_sansli, antti.hallapuro, jani.lainema, honglei.1.zhang}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The content-adaptive LOP2 loop-filter was adopted into NNVC-9 software. Since a new version of LOP loop-filter, namely the LOP3, was also adopted into the NNVC-9 software, this contribution reports the content adaptation per random-access segment for LOP3 loop-filter. The content-adaptive parameters are signalled via a specifically-designed Adaption Parameter Set (APS) called Neural Network Filter Update (NNFU). It is reported that applying this approach in the RA configuration yields average coding gains of 1.02% (Y), 2.27% (Cb), and 1.43% (Cr) compared to NNVC-9.1 with NN intra tool ON and LOP3 tool ON.

# Introduction

The content-adaptive method for LOP3 [1] involves overfitting LOP3 filter in NNVC-9.1 separately for each random-access segment. The resulting weight-updates are coded using the MPEG-NNR standard [2, 3] and signaled through a NNFU APS [4].

# Content adaptation method

The content-adaptive LOP3 filter is obtained by overfitting multiplier parameters that are applied after a convolutional block, like:

σW*x+b∙m,
where W is the kernel, * is the convolution operator, x is the input, b is the bias, m is the multiplier and σ is the activation function.

The network architecture is based on JVET-AH0080 [1] (Figure 1) and the difference is that the multiplier layers are added after each convolutional block in the luma and chroma backbone. Similar to content-adaptive loop filter in JVET-AH0096 [4], only subsets of multipliers in the luma and chroma backbones are selectively overfitted. The details of selection and determination of the number of overfitted multiplier layers are described in JVET-AH0096.

Figure 1. LOP3 filter architecture from JVET-AH0080. Head channels: d1: 16, d2: 8, d3: 4, d4: 2, d5: 2, C (=d6): 64. Backbone channels: NY: 14, NUV: 4, CY: 32, CUV: 32, CY1: 144, UV1: 128, C21: 32.

Table 1 shows the details of NN information during the overfitting stage.

Table 1. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Overfitting Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
|  | Framework: | PyTorch 1.12.1 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 100 epochs |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time (for 1 model – 1 segment): | A1 & A2: from 26 min to 2.3h B: from 13 min to 40 min C: from 3 min to 13 min D: from 3 min to 7 min F: from 6 min to 34 min |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | 1e-3 |
|  | Learning rate update strategy | Constant learning rate |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Up-sample 72x72 UV signal to 144x144 blocks Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

# Quantization

The floating-point overfitted models, obtained after decoding the NNR weight updates, were quantized to int16 using the same quantizers as the base models (for convolutional kernels and bias terms). Additionally, the multiplier layers use a fixed quantizer of 13.

# Inference

Table 2 describes the loop-filter information at inference stage.

Table 2. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | CPU only |
|  | Framework: | SADL |
|  | Number of GPUs per Task |  |
|  | Number of Parameters (Each Model) | 209918 |
|  | Total Number of Parameters (All Models) | 209918 |
|  | Parameter Precision (Bits) | Int16 |
|  | Memory Parameter (MB) | 0.4 |
|  | Multiply Accumulate (kMAC/pixel) | 17.3 |
|  | Calculation Method | on a block basis |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128x128 / 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

The number of parameters and complexity are computed using SADL.

# Experimental results

Table 3 shows the coding performance of the proposed approach (content-adaptive LOP3 and NN-intra ON) in the RA configuration over NNVC-9.1 with LOP3 and NN-intra ON.

Table 3. Coding performance over NNVC-9.1 with LOP3 and NN intra ON

|  | BD-rate Over NNVC-9.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 | -2,30 % | -2,30 % | -7,90 % | -3,76 % | -4,20 % | -7,07 % | 197 % | 132 % | 99 % |
| Class A2 | -0,59 % | -2,70 % | -3,19 % | 0,09 % | -3,16 % | -4,28 % | 197 % | 121 % | 100 % |
| Class B | -0,87 % | -8,18 % | -6,23 % | 1,60 % | -8,51 % | -6,00 % | 206 % | 298 % | 99 % |
| Class C | -0,23 % | -0,99 % | -1,67 % | 0,42 % | -1,76 % | -2,32 % | 207 % | 1243 % | 100 % |
| Overall | -0,93 % | -3,99 % | -4,74 % | -0,09 % | -4,78 % | -4,89 % | 203 % | 309 % | 100 % |
| Class D | 0,01 % | 0,03 % | -0,10 % | 0,04 % | 0,08 % | -0,09 % | 206 % | 4631 % | 100 % |
| Class F | -0,13 % | -1,09 % | -1,16 % | 0,05 % | -1,16 % | -1,36 % | 207 % | 2052 % | 100 % |

Table 4 shows the coding performance of the proposed approach (content-adaptive LOP3 and NN-intra ON) in the RA configuration over NNVC-9.1 with all NN tools OFF.

Table 4. Coding performance over NNVC-9.1 with all NN tools OFF

|  | BD-rate Over NNVC-9.1-VTM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |  |
| Class A1 | -10,89 % | -13,89 % | -19,40 % | -12,90 % | -17,19 % | -20,60 % | 246 % | 7109 % | 95 % |  |
| Class A2 | -8,11 % | -14,75 % | -11,04 % | -8,19 % | -15,97 % | -11,77 % | 245 % | 6890 % | 96 % |  |
| Class B | -7,70 % | -20,77 % | -17,40 % | -6,16 % | -22,15 % | -19,23 % | 254 % | 7292 % | 96 % |  |
| Class C | -6,75 % | -14,33 % | -13,38 % | -7,12 % | -15,71 % | -14,78 % | 253 % | 6376 % | 98 % |  |
| Overall | -8,16 % | -16,47 % | -15,46 % | -8,17 % | -18,20 % | -16,82 % | 250 % | 6921 % | 98 % |  |
| Class D | -6,70 % | -11,79 % | -11,22 % | -6,75 % | -11,98 % | -11,91 % | 252 % | 5602 % | 98 % |  |
| Class F | -3,93 % | -8,50 % | -7,13 % | -4,68 % | -11,16 % | -10,20 % | 276 % | 4133 % | 98 % |  |

# Recommendation

It is recommended to adopt the proposed content-adaptive LOP3 into the next version of the NNVC software.

# Reference

- . Liu, J. Ström, M. Damghanian, P. Wennerste, D. Rusanovskyy, Y. Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya. EE1-1.2: Joint LOP model with inputs transformed. JVET-AH0080. April 2024.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis (INCTM), N0179. February 2022.

- M. Santamaria, R. Yang, F. Cricri, M. M. Hannuksela, D. B. Sansli, A. Hallapuro, H. R. Tavakoli, J. Lainema, H. Zhang. EE1-1.4: Content-adaptive loop-filter. JVET-AH0096. April 2024.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).