| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0115-v2 |
| --- | --- |

| Title: | Non-EE2: Regression-based GPM extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ke Jia Jie Chen Xinwei Li Ru-Ling Liao Yan Ye | Tel: Email: | {feijia.jk, jiechen.cj, sid.lxw, ruling.lrl, yan.ye}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to extend the regression-based GPM mode by applying TM and MMVD. For each extension (TM or MMVD), a CU-level flag is signaled to indicate whether the corresponding extension is used or not. The proposed extension was implemented on top of ECM-13.0, and it is reported that the overall performance impact is {-0.10%, -0.17%, -0.17%, 105.9%, 100.2%} in RA and {-0.13%, -0.38%, -0.21%, 107.6%, 99.9%} in LD.

In the 2nd version, the full results are provided and an additional test is added to reduce the runtime increase, it is reported that {-0.07%, -0.13%, -0.16%, 101.8%, 100.0%} in RA and {-0.13%, -0.02%, -0.23%, 101.5%, 99.6%} in LD in this additional test.

Introduction

In ECM-13.0, template matching (TM) based motion vector refinement and merge with motion vector difference (MMVD) was applied to regular geometric partitioning mode (GPM). For GPM-TM, a CU-level flag is signaled to jointly indicate whether the TM is used for the two partitions. That is, when the flag is on, the motion vectors of both two partitions are refined by TM; otherwise, none of the motion vectors are refined. For GPM-MMVD, two partition-level flags are signaled to separately indicate whether the MMVD is used for each partition. When the flag indicates MMVD is used for a partition, the index of the MVD selected by the encoder is further signaled. Both TM and MMVD can improve the motion accuracy for GPM and thus give coding performance increase.

However, in the ECM-13.0, TM and MMVD is only applied to regular GPM, but not applied to regression-based GPM. In this contribution, it is proposed to extend the regression-based GPM by applying TM and MMVD.

Proposed method

It is proposed to apply both TM and MMVD on regression-based GPM.

When regression-based GPM is used for the current block, a TM flag is firstly signaled to indicate whether the motion vector is to be refined by TM. If the TM flag is true, the motion vectors of both partitions are refined. The template used in the TM process consists of above and left reconstructed samples (AL shape). The TM search process is the same as that of regular GPM-TM.

If the TM flag is false, then an MMVD flag is signaled to indicate whether the motion vectors of partitions are refined by the MVD. If the MMVD flag is true, an MVD index is signaled to indicate the MVD. Then the MVD is used to refine the motion vectors of both partitions. The MVD candidates for encoder selection and MVD index signalling are the same as those of regular GPM-MMVD.

The signing chart is shown in Figure 1. P0 and P1 represent the two partitions.

Figure 1. The flowchart of regression GPM with TM and MMVD

Simulation results

The proposed methods are implemented on top of ECM-13.0 and tested under CTC for enhanced compression tool testing. The results are shown in the Table 1.

Table 1. Simulation results of regression GPM with TM and MMVD

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.21% | -0.22% | 103.3% | 99.1% | 100.2% | 100.1% |
| Class A2 | -0.11% | -0.20% | -0.16% | 103.9% | 100.1% | 100.1% | 100.1% |
| Class B | -0.12% | -0.17% | -0.28% | 107.1% | 100.5% | 100.0% | 100.1% |
| Class C | -0.07% | -0.10% | 0.02% | 108.0% | 100.9% | 100.8% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.10% | -0.17% | -0.17% | 105.9% | 100.2% | 100.3% | 100.1% |
| Class D | -0.04% | 0.01% | -0.02% | 107.3% | 101.3% | 101.2% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.17% | -0.48% | -0.25% | 106.6% | 99.7% | 100.4% | 99.9% |
| Class C | -0.12% | -0.40% | -0.17% | 107.9% | 99.7% | 101.3% | 100.0% |
| Class E | -0.07% | -0.19% | -0.20% | 108.8% | 100.5% | 99.8% | 100.0% |
| Overall | -0.13% | -0.38% | -0.21% | 107.6% | 99.9% | 100.6% | 100.0% |
| Class D | -0.13% | -1.09% | -0.43% | 107.9% | 100.2% | 101.7% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

In order to reduce the runtime increase, the preliminary experiment results are shown in the Table 2.

Table 2. Simulation results of complexity redunction

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04% | -0.21% | -0.19% | 101.6% | 98.7% | 100.1% | 99.8% |
| Class A2 | -0.08% | -0.21% | -0.11% | 101.7% | 100.2% | 100.1% | 100.0% |
| Class B | -0.09% | -0.08% | -0.16% | 101.9% | 100.4% | 100.5% | 100.1% |
| Class C | -0.06% | -0.06% | -0.18% | 102.0% | 100.3% | 100.1% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.13% | -0.16% | 101.8% | 100.0% | 100.2% | 100.0% |
| Class D | -0.05% | -0.05% | -0.09% | 101.6% | 100.5% | 100.1% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.17% | -0.33% | -0.22% | 101.1% | 99.3% | 100.1% | 100.0% |
| Class C | -0.21% | -0.33% | -0.79% | 101.6% | 100.3% | 100.5% | 100.0% |
| Class E | 0.05% | 0.89% | 0.51% | 102.1% | 99.0% | 97.2% | 100.1% |
| Overall | -0.13% | -0.02% | -0.23% | 101.5% | 99.6% | 99.5% | 100.0% |
| Class D | -0.07% | -0.07% | -0.48% | 102.1% | 100.6% | 100.9% | 100.0% |
| Class F |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |

Conclusion

In this contribution, the regression GPM with TM and regression GPM with MMVD are proposed. Overall, it gives {-0.10%, -0.17%, -0.17%, 105.9%, 100.2%} in RA and {-0.13%, -0.38%, -0.21%, 107.6%, 99.9%} in LD. Furthermore, a complexity reduction version gives {-0.07%, -0.13%, -0.16%, 101.8%, 100.0%} in RA and {-0.13%, -0.02%, -0.23%, 101.5%, 99.6%} in LD. It is recommended to study the proposed method in EE2.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).