| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0116-v1 |
| --- | --- |

| Title: | AHG9: On object mask auxiliary layer list and mask Id in OMI SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junghak Nam Hendry Tan Jangwon Lee Chul Keun Kim Jaehyun Lim Seung Hwan Kim | Email: | junghak.nam@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution proposes some modifications for the object mask information (OMI) SEI Message. The proposed modifications are as follows:

Aspect 1: regarding the signalling of object mask auxiliary layers information:

Proposal 1: signal a syntax element, i.e., omi_aux_pic_layer_id[ i ], to specify the layer Id of the i-th object mask auxiliary layer associated with the current primary picture layer.

Proposal 2: signal a syntax element, i.e., omi_aux_pic_layer_used_flag[ i ], to specify whether the i-th object mask auxiliary layer associated with the current primary picture layer is used or not.

Aspect 2: regarding the signaling of maskId information:

Proposal 3: clarify the association between mask Ids among different OMI SEI messages in the current CLVS as follows:

- Option 1: specify that when the value of maskId[ i0 ][ j0 ] in an OMI SEI message seiA and maskId[ i1 ][ j1 ] in another OMI SEI message seiB are the same and both seiA and seiB are associated with the same primary picture layer, they are referring to the same object mask.

- Option 2: specify a constraint that value of maskId[ i0 ][ j0 ] in an OMI SEI message seiA and maskId[ i1 ][ j1 ] in another OMI SEI message seiB shall be the same and they refer to the same object mask in the i-th object mask auxiliary layer associated with the primary picture layer when all the following are true:

  - i0 is equal to i1 and j0 is equal to j1.

  - Both seiA and seiB are associated with the same primary picture layer.

Problem Statements

Problem 1

In the current design of object mask information (OMI) SEI message, an OMI SEI message contains object mask information from all object mask auxiliary layers associated with a primary picture layer. The layer Ids of the associated auxiliary layers are currently not signalled nor derived. It would be easier / desirable for implementation if the layer Ids of the associated object mask auxiliary layers are easily obtained (e.g., explicitly signalled in the SEI message).

In addition, it is also asserted that when there are more than one object mask auxiliary layers associated with a primary picture layer, zero or more of those auxiliary layers may not be needed / used for a particular OMI SEI message. It is desirable that OMI SEI message can indicate that and avoid signalling object mask information from the auxiliary layers that is not used.

Problem 2

In the current design, when a particular maskId[ i ][ j] is cancelled (i.e., the value of omi_mask_cancel[ i ][ j ] is equal to 0) in an OMI SEI message, that particular maskId (i.e., maskId[ i ][ j]) may later be present again in another OMI SEI message in the same CLVS. In such situation, it is not clear if these two maskIds in different OMI SEI messages refer to the same object mask or not. In authors’ assertion, it is desirable that the association between maskId and mask object is constant in the CLVS.

If it is agreed to have constant association between maskId and object mask in the CLVS, such association may be expressed in at least two ways:

- Via the value of maskId. When two maskIds, regardless of their i and j indices, in two OMI SEI messages have the same value, they refer to the same object mask.

- Via the signalled position of the maskId. When two maskIds are signalled with the same i and j indices in two OMI SEI messages, it is possible to constraint them to have the same value and refer to the same object mask.

Proposals

Aspect 1

To overcome the asserted problem 1, we propose solutions as follows,

Proposal 1: signal a syntax element, i.e., omi_aux_pic_layer_id[ i ], to specify the layer Id of an i-th object mask auxiliary layer associated with the current primary picture layer.

Proposal 2: signal a syntax element, i.e., omi_aux_pic_layer_used_flag[ i ], to specify whether the i-th object mask auxiliary layer associated with the current primary picture layer is used or not.

Aspect 2

To overcome the asserted problem 2, we propose solutions as follows,

Proposal 3: clarify the association between mask Ids among different OMI SEI messages in the current CLVS as follows:

- Option 1: specify that when the value of maskId[ i0 ][ j0 ] in an OMI SEI message seiA and maskId[ i1 ][ j1 ] in another OMI SEI message seiB are the same and both seiA and seiB are associated with the same primary picture layer, they are referring to the same object mask.

- Option 2: specify a constraint that value of maskId[ i0 ][ j0 ] in an OMI SEI message seiA and maskId[ i1 ][ j1 ] in another OMI SEI message seiB shall be the same and they refer to the same object mask in the i-th object mask auxiliary layer associated with the primary picture layer when all the following are true:

–	i0 is equal to i1 and j0 is equal to j1

–	Both seiA and seiB are associated with the same primary picture layer.

Text implementation

Aspect 1

Newly added text are shown in in yellow highlight font whereas removed text are shown in stricken-red font.

…

8.32.1 Object mask information SEI message syntax

| object_mask_info( payloadSize ) { | Descriptor |
| --- | --- |
| omi_cancel_flag | u(1) |
| if( !omi_cancel_flag ) { |  |
| omi_persistence_flag | u(1) |
| omi_num_aux_pic_layer | ue(v) |
| for( i = 0; i < omi_num_aux_pic_layer; i++ ){ |  |
| omi_aux_pic_layer_id[ i ] | u(6) |
| omi_aux_pic_layer_used_flag[ i ] | u(1) |
| } |  |
| omi_mask_id_length_minus1 | ue(v) |
| omi_mask_sample_value_length_minus8 | ue(v) |
| omi_mask_confidence_info_present_flag | u(1) |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence_length_minus1 | u(4) |
| omi_mask_depth_info_present_flag | u(1) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth_length_minus1 | u(4) |
| omi_mask_label_info_present_flag | u(1) |
| if( omi_mask_label_info_present_flag ) { |  |
| omi_mask_label_language_present_flag | u(1) |
| if( omi_mask_label_language_present_flag ) { |  |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| omi_mask_label_language | st(v) |
| } |  |
| } |  |
| for( i = 0; i < omi_num_aux_pic_layer; i++ ) { |  |
| if( omi_aux_pic_layer_used_flag[ i ] ) { |  |
| omi_mask_pic_update_flag[ i ] | f(1) |
| if( omi_mask_pic_update_flag[ i ] ) { |  |
| omi_num_mask_in_pic_update[ i ] | ue(v) |
| for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) { |  |
| omi_mask_id[ i ][ j ] | u(v) |
| omi_aux_sample_value[ i ][ j ] | u(v) |
| omi_mask_cancel[ i ][ j ] | u(1) |
| if( !omi_mask_cancel[ i ][ j ] ) { |  |
| omi_mask_bounding_box_present_flag[ i ][ j ] | u(1) |
| if( omi_mask_bounding_box_present_flag[ i ][ j ] ) { |  |
| omi_mask_top[ i ][ j ] | u(16) |
| omi_mask_left[ i ][ j ] | u(16) |
| omi_mask_width[ i ][ j ] | u(16) |
| omi_mask_height[ i ][ j ] | u(16) |
| } |  |
| if( omi_mask_confidence_info_present_flag ) |  |
| omi_mask_confidence[ i ][ j ] | u(v) |
| if( omi_mask_depth_info_present_flag ) |  |
| omi_mask_depth[ i ][ j ] | u(v) |
| while( !byte_aligned( ) ) |  |
| omi_bit_equal_to_zero | f(1) |
| if( omi_mask_label_info_present_flag ) |  |
| omi_mask_label[ i ][ j ] | st(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |
| } |  |

8.32.2	Object mask information SEI message semantics

The object mask information (OMI) SEI message provides object mask information of object mask picture in the auxiliary picture layer associated with the primary picture layer, referred to as the current primary picture layer, in which the SEI message is present. If the OMI SEI message is present, it shall be present in a primary picture layer. One primary picture layer can be associated with one or more auxiliary picture layers. For each of the associated auxiliary picture layers containing the object mask picture with nuh_layer_id equal to sdi_layer_id[ i ], the value of sdi_aux_id[ i ] shall be equal to AUX_OBJECT_MASK, for any value of i in range of 0 to sdi_max_layers_minus1, inclusive.

Use of this SEI message requires the definition of the following variables:

–	A cropped picture width and picture height in units of luma samples, denoted herein by CroppedWidth and CroppedHeight, respectively.

–	A conformance cropping window left offset, ConfWinLeftOffset

–	A conformance cropping window top offset, ConfWinTopOffset

–	A chroma format indicator, denoted herein by ChromaFormatIdc, as described in clause 7.3.

The variables SubWidthC and SubHeightC are derived from ChromaFormatIdc as specified by Table 2.

omi_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object mask information SEI message in the same layer, if present, in output order. omi_cancel_flag equal to 0 indicates that object mask information follows.

omi_persistence_flag specifies the persistence of the object mask information provided in this SEI message. omi_persistence_flag equal to 0 specifies that the object mask information applies to the current picture only. omi_persistence_flag equal to 1 specifies that the object mask information applies to the current picture and all subsequent pictures in the same layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in a PU containing an object mask information SEI message is output that follows the current picture in output order.

When a CVS does not contain an SDI SEI message with sdi_aux_id[ i ] equal to AUX_OBJECT_MASK for at least one value of i, the OMI SEI message shall be ignored.

When an AU contains both an SDI SEI message with sdi_aux_id[ i ] equal to AUX_OBJECT_MASK for at least one value of i and an OMI SEI message, the SDI SEI message shall precede the OMI SEI message in decoding order.

omi_num_aux_pic_layer indicates the number of auxiliary picture layers associated with the current primary picture layer. It is a requirement of bitstream conformance that the value of omi_num_aux_pic_layer shall be equal to numAuxLayer where the variable numAuxLayer is derived as follows.

Let omiPrimaryLayerId be the value of nuh_layer_id of the NAL unit that contains the SEI message.

numAuxLayer = 0;
for( i = 0; i <= sdi_max_layers_minus1; i++ )
	if( sdi_aux_id[ i ] = = AUX_OBJECT_MASK )
		for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ )			(xx)
			if( sdi_layer_id[ sdi_associated_primary_layer_idx[ i ][ j ] ]  = =  omiPrimaryLayerId )
				numAuxLayer++;

omi_aux_pic_layer_id[ i ] specifies the layer id of the i-th auxiliary picture layer associated with the current primary picture layer.

omi_aux_pic_layer_used_flag[ i ] equal to 1 specifies that object mask information of the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer is used. omi_aux_pic_layer_used_flag[ i ] equal to 0 specifies that object mask information of the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer is not used.

omi_mask_id_length_minus1 plus 1 specifies the length, in bits, of omi_mask_id[ i ][ j ] syntax elements.

omi_mask_sample_value_length_minus8 plus 8 specifies the length, in bits, of omi_aux_sample_value[ i ][ j ] syntax elements. The value of omi_mask_sample_value_length_minus8 shall be in the range of 0 to 8.

omi_mask_confidence_info_present_flag equal to 1 indicates that omi_mask_confidence[ i ][ j ] syntax elements are present. omi_mask_confidence_info_present_flag equal to 0 indicates that omi_mask_confidence[ i ][ j ]syntax elements are not present.

omi_mask_confidence_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_confidence[ i ][ j ] syntax elements.

omi_mask_depth_info_present_flag equal to 1 indicates that omi_mask_depth[ i ][ j ] syntax elements are present. omi_mask_depth_info_present_flag equal to 0 indicates that omi_mask_depth[ i ][ j ]syntax elements are not present.

omi_mask_depth_length_minus1 plus 1 specifies the length, in bits, of the omi_mask_depth[ i ][ j ] syntax elements.

It is a requirement of bitstream conformance that the value of omi_num_aux_pic_layer, omi_mask_id_length_minus1, omi_mask_sample_value_length_minus8, omi_mask_confidence_info_present_flag, omi_mask_confidence_length_minus1, if present, omi_mask_depth_info_present_flag and omi_mask_depth_length_minus1, if present, shall be the same for all object_mask_info( ) syntax structures within a CLVS.

omi_mask_label_info_present_flag equal to 1 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ][ j ] syntax elements are present. omi_mask_label_info_present_flag equal to 0 indicates that omi_mask_label_language_present_flag and omi_mask_label[ i ][ j ] syntax elements are not present.

omi_mask_label_language_present_flag equal to 1 indicates that omi_mask_label_language syntax element is present. omi_mask_label_language_present_flag equal to 0 indicates that omi_mask_label_language syntax element is not present.

omi_bit_equal_to_zero shall be equal to 0.

omi_mask_label_language contains a language tag as specified by IETF RFC 5646 followed by a null termination byte equal to 0x00. The length of the omi_mask_label_language syntax element shall be less than or equal to 255 bytes, not including the null termination byte. When not present, the language of the label is unspecified.

omi_mask_pic_update_flag[ i ] equal to 1 indicates the object mask information of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer may be updated. omi_mask_pic_update_flag[ i ] equal to 0 indicates there is no change to the mask information of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer. When omi_mask_pic_update_flag[ i ] is equal to 0, the persistence mechanism is used, that is the object mask information is inherited from the last OMI SEI message present in the same layer in decoding order which signals the mask information of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer.

omi_num_mask_in_pic_update[ i ] specifies the number of object masks in object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer. omi_num_mask_in_pic_update [ i ] shall be in the range of 0 to (1<<(omi_mask_id_length_minus1 + 1)) – 1, inclusive.

omi_mask_id[ i ][ j ] indicates the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer. The length of the omi_mask_id[ i ][ j ] syntax element is omi_mask_id_length_minus1 + 1 bits.

The variable maskId[ i ][ j ] specifying the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer is derived as follows:

for( i = 0; i < omi_num_aux_pic_layer; i++ )
	for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) 	(xx)
		maskId[ i ][ j ] = omi_mask_id[ i ][ j ] + (1<<(omi_mask_id_length_minus1 + 1))*i

omi_aux_sample_value[ i ][ j ] specifies the value of the samples within the area of the j-th object mask of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer.

omi_mask_cancel[ i ][ j ] equal to 1 cancels the persistence scope of the j-th object mask of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture. omi_mask_cancel[ i ][ j ] equal to 0 specifies that the information of the j-th object mask of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer is signalled.

It is a requirement of bitstream conformance that when omi_mask_id[ i ][ j ] with a particular value is parsed for the first time in the current CLVS, the value of the corresponding omi_mask_cancel[ i ][ j ] shall be equal to 0.

omi_mask_bounding_box_present_flag[ i ][ j ] equal to1 specifies that the syntax elements omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ], are present. omi_mask_bounding_box_present_flag[ i ][ j ] equal to 0 indicates syntax elements, omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ], are not present.

omi_mask_top[ i ][ j ], omi_mask_left[ i ][ j ], omi_mask_width[ i ][ j ], and omi_mask_height[ i ][ j ] specify the coordinates of the top-left corner and the width and height, respectively, of the bounding box of the j-th object mask in the cropped decoded object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer, relative to the conformance cropping window specified by the active SPS.

The value of omi_mask_left[ i ][ j ] shall be in the range of 0 to ( CroppedWidth / SubWidthC – 1 ), inclusive, CroppedWidth and SubWidthC being associated to the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer. When it is not present, the value of omi_mask_left[ i ][ j ] is inferred to be 0.

The value of omi_mask_top[ i ][ j ] shall be in the range of 0 to ( CroppedHeight / SubHeightC – 1 ), inclusive, CroppedHeight  and SubHeightC  being associated to the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer. When it is not present, the value of omi_mask_top[ i ][ j ] is inferred to be 0.

The value of omi_mask_width[ i ][ j ] shall be in the range of 0 to ( CroppedWidth / SubWidthC − omi_mask_left[ i ][ j ] ), inclusive. When it is not present, the value of omi_mask_width [ i ][ j ] is inferred to be ( CroppedWidth / SubWidthC − omi_mask_left[ i ][ j ] ).

The value of omi_mask_height[ i ][ j ] shall be in the range of 0 to ( CroppedHeight / SubHeightC − omi_mask_top[ i ][ j ] ), inclusive. When it is not present, the value of omi_mask_height [ i ][ j ] is inferred to be ( CroppedHeight / SubWidthC − omi_mask_top[ i ][ j ] ).

The identified object mask is within a bounding box containing luma samples with horizontal coordinates from SubWidthC * ( ConfWinLeftOffset + omi_mask_left[ i ][ j ] ) to SubWidthC * ( ConfWinLeftOffset + omi_mask_left[ i ][ j ] + omi_mask_width[ i ][ j ] ) − 1, inclusive, and vertical coordinates from SubHeightC * ( ConfWinTopOffset + omi_mask_top[ i ][ j ] ) to SubHeightC * ( ConfWinTopOffset + omi_mask_top[ i ][ j ] + omi_mask_height[ i ][ j ] ) − 1, inclusive.

Variable pI[ i ] [ x ][ y ] is the decoded value of the sample at the relative sample location (x, y) in the cropped object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer. The following process is to determine mask area in a auxiliary picture.

for( i = 0; i < omi_num_aux_pic_layer; i++ )
	for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ )
		if( pI[ i ][ x ][ y ]  = =  omi_aux_sample_value [ i ][ j ]
				&&  x  >=  omi_mask_left[ i ][ j ]
				&&  x < omi_mask_left[ i ][ j ] + omi_mask_width[ i ][ j ]	(xx)
				&&  y  >=  omi_mask_top[ i ][ j ]
				&&  y < omi_mask_top[ i ][ j ] + omi_mask_height[ i ][ j ] )
			the sample at location (x, y) is associated with the object mask with the identifier of maskId[ i ][ j ]

omi_mask_confidence[ i ][ j ] specifies the degree of confidence associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer, in units of 2-( omi_mask_confidence_length_minus1 + 1 ), such that a higher value of omi_mask_confidence[ i ][ j ] indicates a higher degree of confidence. The length of the omi_mask_confidence[ i ][ j ] syntax element is omi_mask_confidence_length_minus1 + 1 bits.

omi_mask_depth[ i ][ j ] specifies the object depth associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer. A smaller value of omi_mask_depth indicates a shorter distance to the object. The length of the omi_mask_depth[ i ][ j ] syntax element is omi_mask_depth_length_minus1 + 1 bits.

omi_mask_label[ i ][ j ] specifies the contents of the label associated with the j-th object mask of the object mask picture in the i-th auxiliary picture layer with nuh_layer_id equal to omi_aux_pic_layer_id[ i ] associated with the current primary picture layer. The length of the omi_mask_label[ i ][ j ] syntax element shall be less than or equal to 255 bytes, not including the null termination byte.

Aspect 2

Newly added text are shown in in yellow highlight font whereas removed text are shown in stricken-red font.

Option 1:

…

omi_mask_id[ i ][ j ] indicates the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The length of the omi_mask_id[ i ][ j ] syntax element is omi_mask_id_length_minus1 + 1 bits.

The variable maskId[ i ][ j ] specifying the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer is derived as follows:

for( i = 0; i < omi_num_aux_pic_layer; i++ )
	for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) 	(xx)
		maskId[ i ][ j ] = omi_mask_id[ i ][ j ] + (1<<(omi_mask_id_length_minus1 + 1))*i

When maskId[ i0 ][ j0 ] in an OMI SEI message seiA and maskId[ i1 ][ j1 ] in another OMI SEI message seiB, in the same CLVS, have the same values, they refer to the same object mask.

…

Option 2:

…

omi_mask_id[ i ][ j ] indicates the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer. The length of the omi_mask_id[ i ][ j ] syntax element is omi_mask_id_length_minus1 + 1 bits.

The variable maskId[ i ][ j ] specifying the identifier of the j-th object mask of the object mask picture in the i-th auxiliary picture layer associated with the current primary picture layer is derived as follows:

for( i = 0; i < omi_num_aux_pic_layer; i++ )
	for( j = 0; j < omi_num_mask_in_pic_update[ i ]; j++ ) 	(xx)
		maskId[ i ][ j ] = omi_mask_id[ i ][ j ] + (1<<(omi_mask_id_length_minus1 + 1))*i

It is constrained that the values of omi_mask_id[ i0 ][ j0 ] in an OMI SEI message seiA and omi_mask_id[ i1 ][ j1 ] in another OMI SEI message seiB shall be the same when all the following are true:

–	i0 is equal to i1 and j0 is equal to j1.

–	Both seiA and seiB are associated with the same primary picture layer.

…

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).