| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0124-v1 |
| --- | --- |

| Title: | AHG 14: Alignment of reference picture setting in NNVC with those in VTM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, D. Liu, J. Ström | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The reference picture settings for random access and low delay B configuration in NNVC are not aligned with corresponding settings in VTM and ECM. In VTM and ECM the DPB is better utilized enabling better coding efficiency for both random access and low-delay B.

The BDR impact of deploying the same reference picture settings in NNVC are as follows:

RA(Y/U/V): -0,56%/-0,50%/-0,56%

LDB(Y/U/V): -1,56%/-3,35%/-2,68%

It is suggested to align the reference picture settings in NNVC with those in the VTM reference software.

# Introduction

The reference picture settings for VTM and ECM were updated by JVET-Z0072. However, this was not incorporated into the NNVC reference software.

# Proposal

It is suggested to deploy the same reference picture setting in NNVC as in VTM for random-access and low-delay B configuration.

For random-access configuration the changes can be summarized as adding some additional references for several pictures, removing duplicates for some pictures and thereby removing some references and also putting some references closer to the current picture. In addition to that the QP offset for POC 8 and POC 24 is increased by 1. The changes are high-lighted in yellow.

#        Type POC QPoffset QPOffsetModelOff QPOffsetModelScale CbQPoffset CrQPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 CbTcOffsetDiv2 CbBetaOffsetDiv2 CrTcOffsetDiv2 CrBetaOffsetDiv2                                                       #                                                                                                    #                                                                                    temporal_id  active tot         L0                               active        tot          L1

Frame1  : B   32  -1   0.0     0.0     0  0  1.0  1  0  1  0  1  0  0            2  5       32 64 48 40 36            1  2        32 48

Frame2  : B   16   0  -4.9309  0.2265  0  0  1.0  2  0  2  0  2  0  1      3  5       16 32 48 24 20            1  1       -16

Frame3  : B    8   1  -4.5000  0.1900  0  0  1.0  3  0  3  0  3  0  2       4  5        8 24 16 40 12             2  2        -8 -24

Frame4  : B    4   3  -5.4095  0.2571  0  0  1.0  4  0  4  0  4  0  3       3  3        4  8 20                        3  3        -4 -12 -28

Frame5  : B    2   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4       3  3        2  6 18                        4  4        -2 -6 -14 -30

Frame6  : B    1   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5       1  1        1                                 2  5        -1 -3 -7 -15 -31

Frame7  : B    3   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5       2  2        1  3                             2  4        -1 -5 -13 -29

Frame8  : B    6   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4       3  3        2  4  6                         3  3        -2 -10 -26

Frame9  : B    5   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5       2  2        1  5                             2  4        -1 -3 -11 -27

Frame10 : B    7   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5      2  3        1  3  7                         2  3        -1 -9 -25

Frame11 : B   12   3  -5.4095  0.2571  0  0  1.0  4  0  4  0  4  0  3     3  4        4  8 12  6                    2  2        -4 -20

Frame12 : B   10   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4     4  4        2  4  6 10                    3  3        -2 -6 -22

Frame13 : B    9   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5     2  3         1  5  9                         2  4        -1 -3 -7 -23

Frame14 : B   11   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3         1  3 11                        2  3        -1 -5 -21

Frame15 : B   14   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    4  4         2  4  6 14                    2  2        -2 -18

Frame16 : B   13   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3         1  5 13                        2  3        -1 -3 -19

Frame17 : B   15   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4         1  3  7 15                    2  2        -1 -17

Frame18 : B   24   1  -4.5000  0.1900  0  0  1.0  3  0  3  0  3  0  2    3  3         8 16 24                       1  1        -8

Frame19 : B   20   3  -5.4095  0.2571  0  0  1.0  4  0  4  0  4  0  3    3  3        4 12 20                        2  2        -4 -12

Frame20 : B   18   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    3  3        2 10 18                        3  3        -2 -6 -14

Frame21 : B   17   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3        1  9 17                         2  4        -1 -3 -7 -15

Frame22 : B   19   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3        1  3 19                         2  3        -1 -5 -13

Frame23 : B   22   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    3  3        2  6 22                         3  3        -2 -10 4

Frame24 : B   21   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  3        1  5 21                         2  3        -1 -3 -11

Frame25 : B   23   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4        1  3  7 23                     2  2        -1 -9

Frame26 : B   28   3  -5.4095  0.2571  0  0  1.0  4  0  4  0  4  0  3    4  4        4  8 12 28                    1  1        -4

Frame27 : B   26   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    4  4        2  6 10 26                    2  2        -2 -6

Frame28 : B   25   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4        1  5  9 25                     2  3        -1 -3 -7

Frame29 : B   27   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4        1  3 11 27                    2  2        -1 -5

Frame30 : B   30   5  -4.4895  0.1947  0  0  1.0  5  0  5  0  5  0  4    4  4        2  6 14 30                    1  1        -2

Frame31 : B   29   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  4        1  5 13 29                    2  2        -1 -3

Frame32 : B   31   6  -5.4429  0.2429  0  0  1.0  5  0  5  0  5  0  5    2  5        1  3  7 15 31                1  1        -1

For low-delay B configuration the changes can be summarized as adding two additional reference pictures instead of two of the duplicates from L0 in L1. The number of active reference pictures is unchanged. We also propose to increase the QP offset by 1 and to adjust the QP offset scale parameter for the pictures with odd POC values. The changes are high-lighted in yellow.

#        Type POC QPoffset QPOffsetModelOff QPOffsetModelScale CbQPoffset CrQPoffset QPfactor tcOffsetDiv2 betaOffsetDiv2 CbTcOffsetDiv2 CbBetaOffsetDiv2 CrTcOffsetDiv2 CrBetaOffsetDiv2                                                       #                                                                                                    #                                                                                    temporal_id  active tot         L0                               active        tot          L1

Frame1:  B    1   6       -6.5  0.2450         0  0  1.0  0  0  0  0   0  0  0   4                4         1 9 17 25            4                   4      1 3 5 33

Frame2:  B    2   4       -6.5  0.2590         0  0  1.0  0  0  0  0   0  0  0   4                4         1 2 10 18            4                   4      1 2 4 26

Frame3:  B    3   6       -6.5  0.2450         0  0  1.0  0  0  0  0   0  0  0   4                4         1 3 11 19            4                   4      1 3 5 27

Frame4:  B    4   4       -6.5  0.2590         0  0  1.0  0  0  0  0   0  0  0   4                4         1 4 12 20            4                   4      1 2 4 28

Frame5:  B    5   6       -6.5  0.2450         0  0  1.0  0  0  0  0   0  0  0   4                4         1 5 13 21            4                   4      1 3 5 29

Frame6:  B    6   4       -6.5  0.2590         0  0  1.0  0  0  0  0   0  0  0   4                4         1 6 14 22            4                   4      1 2 6 30

Frame7:  B    7   6       -6.5  0.2450         0  0  1.0  0  0  0  0   0  0  0   4                4         1 7 15 23            4                   4      1 3 7 31

Frame8:  B    8   1        0.0   0.0               0  0  1.0  0  0  0  0   0  0  0   4                4         1 8 16 24           4                   4      1 2 4 32

# Results

The results are compared against NNVC-9.0 [2][3] using gcc 12.2.0. The test is with NN tools off. The encoding and decoding time is not accurate.

|  | Random access Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-9.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | -0,39% | -0,21% | -0,38% | -0,53% | -0,54% | -0,47% | 97% | 95% | 1% |
| Class A2 | -0,45% | -0,44% | -0,47% | -0,46% | -0,55% | -0,55% | 95% | 98% | 1% |
| Class B | -0,62% | -0,66% | -0,61% | -0,59% | -0,67% | -0,65% | 94% | 98% | 1% |
| Class C | -0,68% | -0,55% | -0,70% | -0,70% | -0,60% | -0,78% | 105% | 99% | 1% |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall | -0,56% | -0,50% | -0,56% | -0,59% | -0,60% | -0,63% | 98% | 98% | 1% |
| Class D | -0,45% | -0,28% | -0,41% | -0,44% | -0,32% | -0,44% | 101% | 103% | 1% |
| Class F | -0,41% | -0,37% | -0,31% | -0,52% | -0,47% | -0,35% | 94% | 98% | 1% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-9.0 |  |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT | bit DIFF |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -0,77% | -2,68% | -1,85% | -1,45% | -3,08% | -1,98% | 85% | 80% | 4% |
| Class C | -2,43% | -4,19% | -3,71% | -2,90% | -4,82% | -4,99% | 93% | 87% | 4% |
| Class E | -1,73% | -2,84% | -2,86% | -1,69% | -2,48% | -2,89% | 73% | 61% | 3% |
| Overall | -1,56% | -3,35% | -2,68% | -1,99% | -3,85% | -3,32% | 84% | 77% | 4% |
| Class D | -1,79% | -3,55% | -3,06% | -2,23% | -3,13% | -3,13% | 75% | 66% | 4% |
| Class F | -2,43% | -3,02% | -3,02% | -2,34% | -2,78% | -3,88% | 94% | 92% | 3% |

# Conclusions

It has been showed that aligning reference picture setting with those in VTM can improve coding efficiency for both random access and low delay B configuration. It is therefore suggested to update the reference picture settings in NNVC so it’s aligned with the reference picture settings in VTM reference software.

# References

[1] K. Andersson, R. Sjöberg, R. Yu, L. Litwic, ” AHG10/AHG12: Enhanced reference picture structures for ECM and VTM”, JVET-Z0072, Online meeting, April 2022.

[2] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-AF2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[3] https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM.git

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).