| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0125-v1 |
| --- | --- |

| Title: | Non-EE2: DIMD with filtered-template |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhuoyi Lv Chuan Zhou (vivo) | Email: | zhuoyi.lv@vivo.com chuan.zhou@vivo.com |
| Source: | vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

In previous contributions JVET-AE0130 [1] and JVET-AF0131 [2], the method of DIMD modes derived with the filtered-template was proposed, where the template was filtered using a 3x3 filter operator before obtaining the gradient histogram. At that time, it was not adopted due to high encoding complexity.

In this contribution, the encoder optimization strategy is designed to reduce the encoding time. In addition, the usage of template is modified.

It is reported that on top of ECM-13.0, the overall coding performance impact for {Y, U, V, EncT, DecT} is {  %,   %,   %,   %,   %} for AI and {  %,   %,   %,   %,   %} for RA.

Introduction

In ECM-13.0, when DIMD is applied, up to five intra modes are derived from the reconstructed neighbor samples, and those five predictors are combined with the non-directional predictor (planar or block vector-based predictor) with the weights derived from the histogram of gradients [3].

Proposed methods

Aspect #1. Usage of template

Firstly, compared with previous contributions (JVET-AE0130, JVET-AF0131), the region of neighboring reconstructed samples used for computing the histogram of gradients for this filtered-template DIMD method is modified, depending on the availability of reconstructed samples. Compared to current DIMD, one additional row towards above side, and one additional column towards left side are extended if available. In other words, up to (W×2+5)×5 above reconstructed samples and 5×(H×2) left reconstructed samples of current W×H CB are used as template if available (see Template-Ⅰ in Fig. 1). When there are un-available reconstructed samples, the unfiltered samples in adjacent row and column are used for gradient computation (see Template-Ⅱ in Fig. 2).

Figure 1. Template-Ⅰ

Figure 2. Template-Ⅱ

Secondly, a 3×3 filter is applied on the template.

F=-1-1-1-19-1-1-1-1

Then, the same 3×3 Sobel operators in current DIMD are used to derive the histogram.

Mx=-101-202-101    and    My=-1-2-1000121

Aspect #2. Signalling

Whether the template is filtered or not is signalled with a CABAC coded flag.

Aspect #3. Complexity reduction

In addition, this contribution proposes to reduce the complexity associated with filtered-template DIMD by the following aspects:

- Limited block sizes: Filtered-template DIMD is only applied to block whose area is larger than or equal to 128, and smaller than or equal 1024. And it is applied to I-slice only.

- Early termination: For I-slice, if the cost of filtered-template DIMD mode is larger than 1.5 time of regular angular intra mode in the first pass of LFNST_MTS check, it will be skipped in the remaining passes.

Simulation results

The proposed method is tested on top of ECM13.0 under the common test conditions [4].

Table 1. Simulation results

|  | All intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.07% | -0.15% |  |  |
| Class C | -0.03% | 0.14% | 0.06% | 100.4% | 97.8% |
| Class E | -0.04% | 0.02% | -0.09% |  |  |
| Overall |  |  |  |  |  |
| Class D | -0.01% | 0.03% | -0.01% | 101.0% | 96.1% |
| Class F |  |  |  |  |  |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

References

- C. Zhou, Z. Lv, “Non-EE2: DIMD with filtered template”, JVET-AE0130.

- C. Zhou, Z. Lv, “EE2-2.2: DIMD with filtered template”, JVET-AF0131.

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 13 (ECM 13)”, JVET-AH2025.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-AF2017.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).