| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0126-v2 |
| --- | --- |

| Title: | Non-EE2: Enhanced inter AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Na Zhang1, Yang Wang1, Wenbin Yin1, Lei Zhao1, Li Zhang2 1 No. 27 North Third Ring West Road, Haidian District, Beijing Fangheng Fashion Center, 100098, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to check additional candidates for regular AMVP candidate list, and reorder the affine AMVP candidates. The impact on coding efficiency and runtime of the proposed method over ECM-13.0 are reported for {Y, U, V, EncT, DecT} as below:

RA: { -0.03%, -0.03%, -0.04%, 100.7%, 100.4%}

LDB: { -0.12%, -0.07%, -0.14%, 101.2%, 100.8%}

# Introduction

In ECM-13.0, the regular inter AMVP candidate list is constructed by checking spatial adjacent candidates, temporal collocated candidates, spatial non-adjacent candidates, and HMVP candidates. The regular AMVP candidate list comprises 5 candidates and the candidate with the minimal TM cost is selected as the MVP. On the other hand, the affine AMVP candidate list contains 2 candidates, and the index of the selected affine MVP candidate is signalled in the bitstream.

# Proposed method

This contribution proposes two aspects to enhance inter AMVP. First, non-adjacent positions used in the inter merge mode can be checked to construct the regular inter AMVP candidate list. Second, template based reordering is applied to reorder at most 10 affine AMVP candidates in the affine AMVP candidate list.

# Experimental results

The proposed method was implemented on top of ECM-13.0 and tested following the common test conditions [1]. The simulation results of the proposed method compared to ECM-13.0 are summarized in Table 1.

Table 1: Coding performance of the proposed method

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.00% | -0.04% | 100.5% | 100.4% | 100.0% | 100.0% |
| Class A2 | -0.02% | -0.15% | -0.03% | 100.4% | 100.8% | 100.3% | 100.1% |
| Class B | -0.03% | 0.03% | -0.09% | 100.7% | 100.5% | 99.8% | 99.8% |
| Class C | -0.03% | -0.02% | 0.07% | 101.0% | 100.1% | 99.6% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.03% | -0.04% | 100.7% | 100.4% | 99.9% | 100.0% |
| Class D | -0.04% | -0.12% | 0.19% | 100.8% | 100.6% | 99.9% | 100.0% |
| Class F | -0.01% | 0.16% | -0.02% | 100.7% | 100.8% | 100.0% | 99.9% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.12% | -0.01% | -0.13% | 101.7% | 100.8% | 100.0% | 100.1% |
| Class C | -0.08% | -0.16% | -0.64% | 100.4% | 101.2% | 100.0% | 100.0% |
| Class E | -0.16% | -0.04% | 0.55% | 101.6% | 100.2% | 100.0% | 99.9% |
| Overall | -0.12% | -0.07% | -0.14% | 101.2% | 100.8% | 100.0% | 100.0% |
| Class D | -0.11% | 0.01% | -0.39% | 100.0% | 100.3% | 100.2% | 100.0% |
| Class F | *-0.46% | -0.72% | -0.56% | 100.8% | 101.0% | 100.0% | 100.0% |

*The results of class F ArenaOfValor QP 22 under low delay B configuration is copied from anchor

# Conclusions

The proposed method can achieve decent BD-rate gains with minor runtime change. It is recommended to include the proposed method in EE2 for further investigation.

# References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).