| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0339-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AI0064 (EE2-4.5: Multiple Transform Set Selection for Intra MTS) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Damian Ruiz Coll, Jung-Kyung Lee. | Tel: Email: | duizcoll@ofinno.com, jlee@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

EE2 Test 4.5a and Test 4.5b explore Multiple Transform Set Selection (MTSS) applied to explicit intra MTS, as was proposed in the contribution JVET-AH0307. In this proposal, for the DIMD, TIMD, IntraTMP, MIP, and EIP modes, an alternative primary transform set from the explicit intra-MTS lookup table is considered in addition to the initial primary transform set.

Test 4.5a corresponds to MTSS for intra MTS as initially proposed, and Test 4.5b corresponds to a combination of 4.5a with 4.4a, which corresponds to the EE2 test about MTSS for LFNST/NSPT. The objective of 4.5b is to assess the orthogonality of the two methods.

# Test description

In [1], an alternative transform set for explicit intra-MTS for DIMD, TIMD, IntraTMP, MIP, and EIP modes is proposed. The proposed method uses 6 primary transform pair candidates in the alternative transform set for each mode. A flag is used in addition to the MTS transform index to signal the best transform set to the decoder after performing the rate-distortion optimization process.

The transform set associated with the MIP mode is used as an alternative transform set for the EIP mode. In contrast, the transform set associated with PLANAR is used as an alternative transform set for the other modes.

As some transform pairs are redundant from a primary transform set to another, some transform pair candidates from the alternative transform set can already be contained in the initial transform set. Therefore, it also uses transform sets associated with adjacent TU size in the LUT to feed the alternative set with non-redundant primary transform pairs.

# Experimental results

The software provided for the proponent [2] has been simulated in accordance with the EE2 description [3] and under JVET ECM common test conditions [4] in both AI and RA configurations. The partial results for Test 4.5a and Test 4.5b of the cross-check are detailed in the accompanying spreadsheet and are further summarized in the subsequent tables. It is affirmed that the proponent's reported results in JVET-AI0064 [1] align with the cross-check outcomes presented in this report for RA configuration.  Note that tests for RA configuration are currently under simulation as of the edition of this report. Also, the test runtimes provided in this report may not be slightly accurate due to the cluster noise.

Test 4.5a:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | 0.06% | 0.03% | 104.2% | 105.3% | 100.3% | 100.2% |
| Class A2 | -0.07% | -0.07% | 0.00% | 103.7% | 104.6% | 100.3% | 100.1% |
| Class B | -0.04% | 0.04% | -0.04% | 104.1% | 104.2% | 97.8% | 100.3% |
| Class C | -0.02% | -0.07% | 0.04% | 105.2% | 103.0% | 100.1% | 100.4% |
| Class E | -0.02% | 0.22% | 0.09% | 103.2% | 104.2% | 101.1% | 100.4% |
| Overall | -0.04% | 0.03% | 0.02% | 104.1% | 104.2% | 99.7% | 100.3% |
| Class D | -0.02% | 0.13% | -0.15% | 105.1% | 102.3% | 100.0% | 100.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -0.01% | -0.07% | -0.16% | 103.0% | 99.2% | 100.0% | 100.3% |
| Class C | -0.01% | -0.08% | 0.08% | 104.3% | 101.3% | 100.5% | 100.3% |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class D | 0.02% | -0.42% | -0.43% | 105.5% | 104.0% | 100.2% | 100.4% |
| Class F | 0.01% | 0.15% | 0.11% | 103.9% | 104.9% | 99.9% | 100.3% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Test 4.5b:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.25% | 0.08% | 0.03% | 111.6% | 104.7% | 101.5% | 100.3% |
| Class A2 | -0.20% | 0.11% | 0.12% | 111.1% | 105.4% | 100.3% | 100.4% |
| Class B | -0.20% | 0.09% | -0.06% | 112.6% | 104.6% | 99.1% | 100.5% |
| Class C | -0.22% | 0.14% | 0.21% | 112.2% | 103.3% | 100.2% | 100.5% |
| Class E | -0.25% | 0.05% | -0.03% | 111.0% | 104.4% | 100.8% | 100.5% |
| Overall | -0.22% | 0.10% | 0.05% | 111.8% | 104.4% | 100.2% | 100.4% |
| Class D | -0.20% | -0.02% | -0.04% | 112.0% | 103.2% | 100.1% | 100.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.19% | -0.22% | -0.24% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | -0.08% | 0.00% | 0.09% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -0.13% | 0.02% | -0.18% | 104.1% | 102.6% | 100.0% | 100.6% |
| Class C | -0.08% | 0.11% | 0.11% | 103.3% | 101.0% | 100.2% | 100.4% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.12% | -0.01% | -0.06% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.07% | -0.23% | -0.05% | 103.0% | 100.9% | 100.2% | 100.5% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Conclusion

The cross-checker has confirmed that the “Multiple Transform Set Selection for Intra MTS” proposed in Test 4.5 [3] is implemented on top of ECM-13.0, and the software implementation matches the technical description of the algorithms in JVET-AI0064[1]. The reported EE2 Test-4.5a and Test-4.5b results, as documented in JVET-AH0064 [1], correlate with the simulation outcomes derived from this cross-checking process. The cross-checker considers that EE2-4.5a achieves a better trade-off between coding gain and complexity than the other two tests, recommending adopting it in ECM-14.0.

# References

- Charles Bonnineau, Karam Naser, Saurabh Puri, Fabrice Le Léannec, “EE2-4.5: Multiple Transform Set Selection for Intra MTS,” JVET-AI0064, July 2024.

- EE2-3.2, https://vcgit.hhi.fraunhofer.de/ecm/jvet-ah-ee2/ECM/-/tree/TEST_4.5

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AH2024, April 2024.

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AF2017, 32nd Meeting, Hannover, Oct. 2023.

# Patent rights declaration(s)

Ofinno LLC does not have any current or pending patent rights relating to the technology described in this contribution.