| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0139-v2 |
| --- | --- |

| Title: | Non-EE2: Modified L-shape template in IntraTMP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jinho Lee Jongho Kim Woong Lim Donghyun Kim Sung-Chang Lim 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-42-860-5656 jinosoul@etri.re.kr pooney@etri.re.kr woong.lim@etri.re.kr kimddng@etri.re.kr sclim@etri.re.kr |
| Source: | Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution presents a modified L-shape template in IntraTMP that excludes top-left. This exclusion affects the calculation of SAD and MRSAD as well as the parameter derivation process in FLM mode. The experimental results {Y, U, V, EncT, DecT} of the proposed method based on ECM-13.0 are reported as follows:

AI: {-0.03%, 0.03%, -0.04%, 99.6%, 100.1%};

RA: {-0.03%, -0.01%, -0.01%, 100.1%, 99.7%}.

Introduction

In IntraTMP of ECM-13.0 [1], there are three template types: L-shape, Top, and Left. In the L-shape template, top, left and top-left samples of a current block are used to calculate SAD and MRSAD, and to derive FLM parameters.

Proposal

In this contribution, a modified L-shape template that excludes top-left samples is proposed, as illustrated in Figure 1. The top-left samples can have a large impact on small blocks in finding the best matching block, while their impact on larger blocks is minimal. For small blocks, excluding these samples from the L-shape template could better reflect the current block and increase prediction efficiency. This modification also reduces computational complexity during the calculating SAD and MRSAD, as well as in the FLM parameter derivation process. Additionally, the current ECM-13.0 SIMD implementation already separates top-left and top samples, making it straightforward to exclude the top-left part.

(a) ECM-13.0           (b) Proposal

Figure 1. An example of L-shape template for 4x4 block

Experimental Results

The proposed method was implemented on top of ECM-13.0 and tested under JVET ECM common test conditions [2]. The experimental results of the proposed method are summarized in Table 1.

Table 1. Results of the proposed method over ECM-13.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | 0.07% | 0.04% | 99.8% | 100.1% | 101.8% | 100.1% |
| Class A2 | -0.03% | -0.03% | -0.02% | 100.0% | 100.0% | 99.9% | 100.0% |
| Class B | -0.04% | -0.01% | -0.21% | 99.0% | 100.0% | 101.5% | 100.1% |
| Class C | -0.01% | 0.04% | 0.09% | 99.4% | 100.2% | 100.5% | 100.0% |
| Class E | -0.05% | 0.11% | 0.00% | 100.4% | 100.1% | 101.2% | 100.0% |
| Overall | -0.03% | 0.03% | -0.04% | 99.6% | 100.1% | 101.0% | 100.0% |
| Class D | -0.01% | -0.07% | -0.04% | 100.9% | 100.4% | 100.0% | 100.0% |
| Class F | -0.05% | -0.06% | 0.00% | 99.6% | 100.0% | 99.3% | 100.0% |
| Class TGM | -0.14% | -0.15% | -0.14% | 100.6% | 100.4% | 102.5% | 100.2% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | -0.08% | -0.02% | 100.3% | 99.7% | 100.0% | 99.8% |
| Class A2 | 0.00% | -0.16% | 0.05% | 100.2% | 99.9% | 100.2% | 100.0% |
| Class B | -0.04% | 0.05% | -0.18% | 99.8% | 99.9% | 100.0% | 100.3% |
| Class C | -0.04% | 0.07% | 0.18% | 100.2% | 99.5% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.03% | -0.01% | -0.01% | 100.1% | 99.7% | 100.0% | 100.0% |
| Class D | 0.00% | -0.15% | -0.10% | 100.6% | 99.7% | 99.9% | 100.0% |
| Class F | -0.05% | 0.22% | -0.09% | 100.3% | 99.1% | 100.0% | 99.9% |
| Class TGM | -0.14% | -0.06% | -0.07% | 99.5% | 98.6% | 100.1% | 99.8% |

Conclusions

This contribution proposes a modified L-shape template in IntraTMP that excludes top-left samples. The experimental results show that this modification results in the average luma BD-rate changes of -0.03% for AI and -0.03% for RA, respectively. It is suggested to adopt the proposed method in the next version of ECM.

References

- ECM-13.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-13.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct, 2023.

Patent rights declaration(s)

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).