| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12-19 July 2024 | Document: JVET-AI0145 |
| --- | --- |

| Title: | AHG12: CABAC slice-level model switch |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Lo Bianco F. Galpin | Tel: Email: | Federico.lobianco@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to

- Extend the current CABAC slice-level model switch to also allow the new low-delay (L) model.

- At encoder side, choose the best CABAC model based on the actual CABAC cost instead of using an estimation.

This contribution provides the following test results:

RA {Y xxx, U xxx, V xxx} LDB {Y xxx, U xxx, V xxx}

# Introduction

In current ECM design, CABAC contexts for B-slices and P-slices are initialized using the CABAC parameters either of B-slices or of P-slices, depending on the value of a transmitted flag sh_cabac_init_flag.

[JVET-AH0176] introduced a new set of CABAC parameters specific to low-delay slices (L-slices for short).

# Proposal

This contribution proposes to also allow the use of CABAC parameters of L-slices. The syntax is modified as follows: first a flag sh_cabac_init_flag is signaled. If sh_cabac_init_flag is equal to 0, the CABAC model corresponding to the slice type of the current slice. Otherwise, one more bit is signaled which of the other two models is used. For example, in a B-slice, the following three cases can occur:

- 0 -> use B-model

- 10 -> use P-model

- 11 -> use L-model

In the proposed experiment, the encoder decision is made based on the computation of the actual CABAC encoding cost of the three available models, instead of the currently used approximation.

The proposed experiment uses the retrained CABAC parameters of EE2 Test 6.1a.

# Experimental results

## These are the reported results. The experiment was performed on a heterogeneous cluster, so encoding and decoding times are not reliable.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-13.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class D | -0.07% | -0.27% | 0.20% | 111.7% | 114.8% | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #DIV/0! | #DIV/0! |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over EE2-6.1a |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0.04% | -0.06% | -0.01% | 110.6% | 116.6% | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #NUM! |

# Conclusion

It is suggested to study the proposed tool in next round of Exploration Experiment.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).