| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, Japan, 12–19 July 2024 | Document: JVET-AI0146-v5 |
| --- | --- |

| Title: | AHG9: Moving the SPO and PON SEI messages from VVC to VSEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Bytedance Inc., US Sean McCarthy Dolby Laboratories, Inc., US | Email: | yekui.wang@bytedance.com sean.mccarthy@dolby.com |
| Source: | Bytedance Inc., Dolby Laboratories, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to move the syntax and semantics of the SPO and PON SEI messages from the VVC spec to the VSEI spec, such that the two SEI messages can be later added to AVC and HEVC (and future video coding standards) through referencing to VSEI, instead of repeating the syntax and semantics therein.

To do that, besides moving of the syntax and semantics, it is asserted that only the following two changes are needed:

- In VVC add a subclause on use of these two SEI messages, which includes the specification of the list variable SeiProcessingOrderSeiList for use in the semantics of the SPO SEI message.

- In VSEI slightly update the following sentence in subclause 6.1:

"Technical specifications that reference this Specification for carrying SEI messages shall specify a way to carry the payload syntax of each specified SEI message, to identify which SEI message is conveyed, ..."

to be the following:

"Technical specifications that reference this Specification for carrying SEI messages shall specify a way to carry the payload syntax of each specified SEI message, to identify which SEI message is conveyed through a payloadType variable that indicates the SEI message payload type, ..."

Attached are the detailed text changes based on the January JVET meeting outputs JVET-AG2027-v1 and JVET-AG2034-v1.

In v4 of this contribution, text changes based on the April JVET meeting outputs JVET-AH2005-v1 and JVET-AH2006-v1 are provided. Besides changes included in the attachments of earlier versions of this contribution, comments received from initial discussions at the meeting have been addressed, including a comment regarding the use of sei_message( ) in the PON SEI message and a comment on the constraint on generalized CMP SEI message in the semantics of the PON SEI message being VVC-specific.

In v5 of this contribution, one error in the VSEI text changes has been coorrected; the error was accidental removal of the RWP SEI constraint instead of the GCMP SEI constraint.

# Patent rights declaration(s)

Bytedance Inc. and Dolby Laboratories, Inc. do not have current or pending patent rights on the proposed changes described in this contribution.