| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024 | Document: JVET-AH2005-v1 |
| --- | --- |

| Title: | Additions and corrections for VVC version 4 (Draft 8) |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Draft text |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan Benjamin Bross Miska M. Hannuksela Ye-Kui Wang | Email: | gary.sullivan@dolby.com benjamin.bross@hhi.fraunhofer.de miska.hannuksela@nokia.com yekui.wang@bytedance.com |
| Source: | Editors |  |  |

_____________________________

# Abstract

This document contains the draft text for changes to the Versatile Video Coding (VVC) standard (Rec. ITU-T H.266 | ISO/IEC 23090-3), to specify the SEI processing order (SPO) and processing order nesting (PON) SEI messages.

Changes that have been integrated:

- (JVET-AH0121) Agreed to allow RWP as a process, and generalized cubemap and ERP as properties in SPO. A list of “property SEIs” should be generated.

- (JVET-AH0121) Item 3: Agreed to remove “or alternative processes that can be applied” from the spec. text.

- (JVET-AH0123) Decision: For now, we should specify that all processes and properties in a processing chain that follow an NNPF in an SPO chain are applied to all pictures produced by the NNPF.

- (JVET-AH0132) Editor action item: The editors are asked to check if the existing text could provide further clarification in the identified situation.

- (JVET-AH0159) Decision: Adopt Item 2 Option 2, with possible editorial improvement by the editors.

- (JVET-AH0352) Decision: Adopt JVET-AH0352v2 (resolves item #1 from JVET-AH0159)

Changes to be integrated or checked:

- Elements from JVET-AG1004 (errata), and bug fixes from JVET-AH0002 for tickets #1609 (NoBackwardPredFlag derivation ambiguity), #1617 (Not initialized NumCtusInSlice[ 0 ] to 0), #1624 (Incorrect indexing in computation of motion vector offset), #1628 (Derivation of ModeTypeCondition should say “one or more”), #1630 (Missing equations for applying AmvrShift), #1631 (Should “Motion vector storing process for geometric partitioning mode” store HpelIfIdx? BBross: HpekIfIdx should always be written since it might be accessed later so it is definitely missing in "Motion vector storing process for geometric partitioning mode".), #1632 (Incorrect indexing used for choosing matrix intra sample prediction), #1634 (Matrices QStateTransTable,levelScale,AlfFixFiltCoeff,AlfClassToFiltMap are incorrectly transposed).

- (JVET-AH0121 item 5) fix the ChromaFormatIdc derivation for the use of the colour transform information SEI message as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027 (to carefully check and communicate offline for correctness, e.g., w.r.t. JVET-AH0047).

- (JVET-AH0121 item 6) fix the interface variable derivation for the use of the NNPFC and NNPFA SEI messages as a part of a processing chain in a similar manner as done for the film grain characteristics SEI message in JVET-AG2027 (to carefully check and communicate offline for correctness, e.g., w.r.t. JVET-AH0047).

- Referencing to messages in VSEI: JVET-AH0343 Text description SEI.

- A sentence should be added that a decoder should operate at most one processing chain at one time. Check sentence saying processing chains “can be complementary, i.e., such that more than one processing chain is chosen and applied separately, with each processing chain generating one output”.

- (JVET-AH0350) Breadth-first: All the pictures (in the entire bitstream, for ease of text specification writing purposes) are processed by a processing stage before moving on to the next processing stage. Depth-first: A picture is processed by all the processing stages, before moving on the next picture in output order. It was suggested, and agreed, to document both approaches as TuC text specification to determine whether depth-first has some advantage.

Changes to the specification text:

# Replace subclause D.2.1 with the following:

D.2.1	General SEI payload syntax

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| SeiExtensionBitsPresentFlag = 0 |  |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| if( payloadType = = 0 ) |  |
| buffering_period( payloadSize ) |  |
| else if( payloadType = = 1 ) |  |
| pic_timing( payloadSize ) |  |
| else if( payloadType = = 3 ) |  |
| filler_payload( payloadSize ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| else if( payloadType = = 4 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_registered_itu_t_t35( payloadSize ) |  |
| else if( payloadType = = 5 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| user_data_unregistered( payloadSize ) |  |
| else if( payloadType = = 19 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| film_grain_characteristics( payloadSize ) |  |
| else if( payloadType = = 45 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_packing_arrangement( payloadSize ) |  |
| else if( payloadType = = 47 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| display_orientation( payloadSize ) |  |
| else if( payloadType = = 56 ) /* Specified in ISO/IEC 23001-11 */ |  |
| green_metadata( payloadsize ) |  |
| else if( payloadType = = 129 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| parameter_sets_inclusion_indication( payloadSize ) |  |
| else if( payloadType = = 130 ) |  |
| decoding_unit_info( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 137 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| mastering_display_colour_volume( payloadSize ) |  |
| else if( payloadType = = 142 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| colour_transform_info( payloadSize ) |  |
| else if( payloadType = = 144 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_light_level_info( payloadSize ) |  |
| else if( payloadType = = 145 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| dependent_rap_indication( payloadSize ) |  |
| else if( payloadType = = 147 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| alternative_transfer_characteristics( payloadSize ) |  |
| else if( payloadType = = 148 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| ambient_viewing_environment( payloadSize ) |  |
| else if( payloadType = = 149 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| content_colour_volume( payloadSize ) |  |
| else if( payloadType = = 150 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| equirectangular_projection( payloadSize ) |  |
| else if( payloadType = = 153 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| generalized_cubemap_projection( payloadSize ) |  |
| else if( payloadType = = 154 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sphere_rotation( payloadSize ) |  |
| else if( payloadType = = 155 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| regionwise_packing( payloadSize ) |  |
| else if( payloadType = = 156 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| omni_viewport( payloadSize ) |  |
| else if( payloadType = = 165 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| alpha_channel_info( payloadSize ) |  |
| else if( payloadType = = 168 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| frame_field_info( payloadSize ) |  |
| else if( payloadType = = 177 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| depth_representation_info( payloadSize ) |  |
| else if( payloadType = = 179 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| multiview_acquisition_info( payloadSize ) |  |
| else if( payloadType = = 180 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| multiview_view_position( payloadSize ) |  |
| else if( payloadType = = 200 ) |  |
| sei_manifest( payloadSize ) |  |
| else if( payloadType = = 201 ) |  |
| sei_prefix_indication( payloadSize ) |  |
| else if( payloadType = = 202 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| annotated_regions( payloadSize ) |  |
| else if( payloadType = = 203 ) |  |
| subpic_level_info( payloadSize ) |  |
| else if( payloadType = = 204 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sample_aspect_ratio_info( payloadSize ) |  |
| else if( payloadType = = 205 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| shutter_interval_info( payloadSize ) |  |
| else if( payloadType = = 206 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| extended_drap_indication( payloadSize ) |  |
| else if( payloadType = = 207 ) |  |
| constrained_rasl_encoding_indication( payloadSize ) |  |
| else if( payloadType = = 208 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| scalability_dimension_info( payloadSize ) |  |
| else if( payloadType = = 209 ) /* Specified in ISO/IEC 23090-13 */ |  |
| vdi_sei_envelope( payloadsize ) |  |
| else if( payloadType = = 210 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_characteristics( payloadSize ) |  |
| else if( payloadType = = 211 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| nn_post_filter_activation( payloadSize ) |  |
| else if( payloadType = = 212 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| phase_indication( payloadSize ) |  |
| else if( payloadType = = 213 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| sei_processing_order( payloadSize ) |  |
| else if( payloadType = = 214 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| processing_order_nesting( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| else /* nal_unit_type = = SUFFIX_SEI_NUT */ |  |
| if( payloadType = = 3 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| filler_payload( payloadSize ) |  |
| else if( payloadType = = 132 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| decoded_picture_hash( payloadSize ) |  |
| else if( payloadType = = 133 ) |  |
| scalable_nesting( payloadSize ) |  |
| else if( payloadType = = 214 ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| processing_order_nesting( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| if( SeiExtensionBitsPresentFlag ¦ ¦ more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) |  |
| sei_reserved_payload_extension_data | u(v) |
| sei_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| sei_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

# In subclause D.2.2, make the following changes:

...

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Buffering period | The remainder of the bitstream |
| Picture timing | The AU containing the SEI message |
| DU information | The AU containing the SEI message |
| Scalable nesting | Depending on the scalable-nested SEI messages. Each scalable-nested SEI message has the same persistence scope as if the SEI message was not scalable-nested |
| SEI manifest | The CVS containing the SEI message |
| SEI prefix indication | The CVS containing the SEI message |
| Subpicture level information | The CVS containing the SLI SEI message and up to but not including the next CVS, in decoding order, that contains an SLI SEI message with different content |
| Constrained RASL encoding indication | The CVS containing the SEI message |

...

The list VclAssociatedSeiList is set to consist of the payloadType values 3, 19, 45, 47, 129, 132, 137, 142, 144, 145, 147 to 150, inclusive, 153 to 156, inclusive, 165, 168, 177, 179, 180, 200 to 202, inclusive, and 204 to 214, inclusive.

...

# Replace subclause D.11.2 with the following (where highlighted text indicates the changes):

### D.11.2 Use of the film grain characteristics SEI message

For purposes of interpretation of the film grain characteristics SEI message, the following variables are specified:

If the film grain characteristics SEI message is not applied as part of a processing chain indicated by an SEI processing order SEI message or is applied as the first processing step of the processing chain indicated by an SEI processing order SEI message, the following applies:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples, respectively.

–	ChromaFormatIdc is set equal to sps_chroma_format_idc.

–	BitDepthY and BitDepthC are both set equal to BitDepth.

Otherwise (the film grain characteristics SEI message is applied as the second or a later processing step of the processing chain indicated by an SEI processing order SEI message), prevPic is a picture resulting from the previous processing step of the processing chain indicated by the SEI processing order SEI message, and the following applies:

–	PicWidthInLumaSamples and PicHeightInLumaSamples are set equal to the picture width and picture height of prevPic, respectively.

–	ChromaFormatIdc is set equal to the picture chroma format indicator of prevPic.

–	BitDepthY and BitDepthC are set equal to the luma bit depth and chroma bit depth of prevPic, respectively.

# Add subclause D.12.13 as follows:

### D.12.13 Use of the SEI processing order and processing order nesting SEI messages

For purposes of interpretation of the SEI processing order and processing order nesting (PON) SEI messages, the following are specified:

–	The list SeiProcessingOrderSeiList is set to consist of the payloadType values 3, 4, 5, 19, 137, 142, 144, 147, 148, 149, 150, 153, 155, 165, 177, 210, and 211.

–	The list SpoProcessSeiList is set to consist of the payloadType values 19, 142, 155, 210, and 211.

–	The syntax structrure sei_pon_nested_message( ), the container of SEI messages, is set to be identical to the syntax structure sei_message( ).

The SEI messages contained in a PON SEI message are referred to as PON-nested SEI messages. When a generalized cubemap project SEI message is present with gcmp_persistence_flag equal to 1 that is not a PON-nested SEI message, there shall not be an associated generalized cubemap project SEI message in the same CLVS that is a PON-nested SEI message.