| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0150-v1 |
| --- | --- |

| Title: | AHG11: Report of retraining LOP3 with new sequences from JVET-AH0120 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Jacob Ström, Mitra Damghanian, Per Wennersten (Ericsson) Dmytro Rusanovskyy (Qualcomm) | Email: | {du.liu, jacob.strom, mitra.damghanian, per.wennersten}@ericsson.com dmytror@qti.qualcomm.com |
| Source: | Ericsson, Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the performance of retraining LOP3 with the new sequences proposed in JVET-AH0120. Several trainings were conducted, including using different learning rates, different sequences, and different random seeds. It is observed that the validation losses deteriorate after about 10 epochs. The inference results also confirm that the performance is degraded. It is suggested to further study the effects of training with the new sequences before adding them to the NNVC training dataset.

# Introduction

At the last meeting, new training sequences were proposed in JVET-AH0120 [1]. It was suggested at the NNVC BOG to study the training impact using the new sequences [2].

# Training and inference

In this contribution, the tested training dataset includes the current LOP2 dataset and the new dataset from JVET-AH0120. The training dataset extracted from JVET-AH0120 follows the same procedure as the extraction of the LOP2 dataset, where the same encoder, decoder, and configuration were used.

The dataset from JVET-AH0120 has a size of 52 GB, whereas the current LOP2 dataset has 4.0 TB. Thus, in terms of size, the new dataset adds only a small portion of data to the existing dataset. The validation dataset remains the same as before.

The LOP3 model is retrained using the new dataset only for stage 3. The batch size, epochs, and milestones are the same as what has been used in LOP3. Each training has 60 epochs and at epoch 55 the loss function switches from L1 loss to MSE loss.

Different training configurations were tested, as shown in Table 1.

Test 1 and Test 2 are two runs with identical training configuration, and the training configuration is the same as LOP3. This is to see how training noise affects the training under identical circumstances.

Test 3 has a smaller learning rate 1e-4. Since new training data is added, the training may need to be more cautious by using a smaller learning rate.

Test 4 and Test 5 are two runs with identical training without including the CG sequences in JVET-AH0120. The LOP2 dataset does not include CG sequences. Thus, it makes the new dataset more similar to the LOP2 dataset by removing the CG sequences from JVET-AH0120. Again, two trainings are used to see the impact of training noise.

Test 6 and Test 7 have different random seeds in the training, but are otherwise equivalent to tests 1 and 2. This is to see exclude the possibility that it is the random seed that causes issues.

Table 1. Different trainings of LOP3 using the sequences from JVET-AH0120.

| Tests | Dataset | Learning rate | Random seed | Runs | Color in Figs. 1 and 2 |
| --- | --- | --- | --- | --- | --- |
| Test 1 | LOP2 dataset and all sequences from JVET-AH0120 | 2e-4 | 42 | Run 1 | Green |
| Test 2 | LOP2 dataset and all sequences from JVET-AH0120 | 2e-4 | 42 | Run 2 | Light green |
| Test 3 | LOP2 dataset and all sequences from JVET-AH0120 | 1e-4 | 42 | Run 1 | Yellow |
| Test 4 | LOP2 dataset and non-CG sequences from JVET-AH0120 | 2e-4 | 42 | Run 1 | Purple |
| Test 5 | LOP2 dataset and non-CG sequences from JVET-AH0120 | 2e-4 | 42 | Run 2 | Light purple |
| Test 6 | LOP2 dataset and all sequences from JVET-AH0120 | 2e-4 | 21 | Run 1 | Blue |
| Test 7 | LOP2 dataset and all sequences from JVET-AH0120 | 2e-4 | 10 | Run 1 | Light blue |

The validation loss is shown in Fig. 1 and Fig. 2, for the L1 loss stage. The reference curve is the validation loss of LOP3, which is in black. The tests are in different colors according to Table 1. Fig. 1 is the validation loss of luma, and Fig. 2 is the validation loss of chroma.

It can be observed in Fig. 1 that the validation losses decrease for the first 10 epochs. However, from epoch 15, fluctuations occur, and after about 30 epochs, the validation losses start increasing. Similar trends can also be observed in Fig. 2 that the validation losses in all tests run away from the reference curve.

Fig. 1. Validation loss of luma.

Fig. 2. Validation loss of chroma.

The inference results of the tests are provided below for RA classes C and D. NNVC-9.1rc is used and NN-Intra is enabled. All the tests use float models, and the anchor is NNVC-9.1rc (NN-Intra + LOP3) float.

Test 1

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1rc float |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 1.38% | 5.70% | 8.84% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 1.97% | 4.86% | 8.07% |
| Class F |  |  |  |

Test 2

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1rc float |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 1.61% | 7.26% | 9.95% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 2.08% | 6.16% | 7.94% |
| Class F |  |  |  |

Test 3

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1rc float |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 1.84% | 7.28% | 7.91% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 2.15% | 7.00% | 7.18% |
| Class F |  |  |  |

Test 4

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1rc float |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 1.38% | 7.37% | 8.70% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 1.94% | 7.07% | 8.86% |
| Class F |  |  |  |

Test 5

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1rc float |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 1.57% | 7.21% | 7.94% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 1.89% | 6.68% | 8.15% |
| Class F |  |  |  |

Test 6

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1rc float |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 1.25% | 5.56% | 7.75% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 1.65% | 5.40% | 7.92% |
| Class F |  |  |  |

Test 7

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | BD-rate Over NNVC-9.1rc float |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B |  |  |  |
| Class C | 1.47% | 6.81% | 8.59% |
| Class E |  |  |  |
| Overall |  |  |  |
| Class D | 1.92% | 5.51% | 8.03% |
| Class F |  |  |  |

# Discussions

All the tests show the same issue that the validation losses deteriorate, and inference results are worse than LOP3.

Tests 1 and 2 show that adding the new dataset to the current training data causes performance degradation.

Test 3 shows that it is a little slower in learning due to the smaller learning rate, but it does not solve the issue.

Tests 4 and 5 show that the CG sequences are not the cause of the issue. The chroma validation losses get worse when CG sequences are removed from the dataset.

Tests 6 and 7 show that changing the random seed does not solve the issue.

# Conclusions

Several trainings were conducted to test the new dataset from JVET-AH0120. It is observed that the validation losses deteriorate when using the new dataset and the inference results confirm the degradation of the performance. It is suggested to further study the new sequences before adding them to the NNVC training dataset.

# References

[1] AHG11 Response to Call for training materials JVET-AG2036, JVET-AH0120, F. Galpin, R. Gendrot, E. Francois, V. Allie (InterDigital), Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 1417–24 April 2024.

[2] BoG on NNVC training materials, JVET-AH0354, E. Alshina, F. Galpin, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 34th Meeting, Rennes, FR, 17–24 April 2024.

[3] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-AF2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).