| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 35th Meeting, Sapporo, JP, 12–19 July 2024 | Document: JVET-AI0153-v1 |
| --- | --- |

| Title: | AHG9: Updates on software implementation and text of the object mask information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zixiang Zhang Jie Chen Yan Ye Shurun Wang | Email: | {zhangzixiang.zzx, jiechen.cj, yan.ye, shurun.wsr}@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

This contribution updates software implementation of object mask information SEI message to align with the current VSEI version 4 working draft. As object mask information SEI message was adopted in the VSEI working draft since January 2024, it is suggested to move software implementation of object mask information SEI message from VTM SEI TuC branch to VTM software in this contribution.

Further, to address the editor’s comments noted in the working draft during the editing period and further avoid text ambiguity, the text of object mask information SEI message is updated as follows

- remove reference to syntax element nuh_layer_id

- add a sentence to say when SDI SEI message is not present, the OMI SEI message shall be ignored.

- define the index of each object mask auxiliary picture layer

- replace syntax element omi_num_aux_pic_layer with omi_num_aux_pic_layer_minus1

Background

The object mask information (OMI) SEI message was adopted in working draft (WD) of VSEI version 4 at 33rd JVET meeting in January 2024, and the software implementation was included in VTM SEI TuC branch [1]. At last meeting, JVET-AH0346 [2] updates the mechanism and the syntax signaling of OMI SEI message, and thus the software needs to be updated accordingly.

Besides, during the WD editing, the following editing comments were added by the editor:

The object mask information (OMI) SEI message provides object mask information of object mask picture in the auxiliary picture layer associated with the primary picture layer, referred to as the current primary picture layer, in which the SEI message is present. If the OMI SEI message is present, it shall be present in a primary picture layer. One primary picture layer can be associated with one or more auxiliary picture layers. For each of the associated auxiliary picture layers containing the object mask picture with nuh_layer_id equal [Ed. (JB): Shouldn’t refer directly to nuh_layer_id syntax element in VSEI, but have an input variable. The same problem exists in depth representation info SEI and alpha channel info SEI.] to sdi_layer_id[ i ], the value of sdi_aux_id[ i ] shall be equal to AUX_OBJECT_MASK, for any value of i in range of 0 to sdi_max_layers_minus1, inclusive. [Ed. (JB): Is there a requirement that the SDI SEI be present when OMI SEI is present? Is there a requirement that the object mask aux pictures be associated in the SDI SEI with the primary picture carrying the OMI SEI?]

This contribution is to update the software and address the editor’s comments.

Software updates

The software is updated according to the current VSEI version 4 WD [3], the main changes include syntax elements alignment and separate signaling of OMI SEI message for each primary picture layer.

Same as before, to enable OMI SEI message, the multi-layer coding configuration of VTM should be used, and scalability dimension information (SDI) SEI message need to be enabled.

Here is an example of encoder and decoder command to enable OMI SEI message, wherein “-l0 -omi” specifies configuration file for OMI SEI message applied on primary layer 0 and “-p 0 --SEIObjectMaskInfosFilename” specifies the output file for OMI SEI message applied on primary layer 0 in decoder side.

EncoderApp.exe -c .\cfg\multi-layer\two_layers_independent.cfg -c .\cfg\sei_vui\scalability_dimension_info.cfg -c .\cfg\encoder_lowdelay_vtm.cfg -l0 -c .\cfg\per-sequence\BasketballPass.cfg -l0 -omi .\cfg\sei_vui\object_mask_infos\lowdelay\mask_info -l1 -c .\cfg\per-sequence\BasketballPass.cfg -b test.bin -f 5 -q 63

DecoderApp.exe -p 0 -b test.bin -o dec.yuv --SEIObjectMaskInfosFilename=dec_omi.txt

As OMI SEI message was adopted in the WD of VSEI version 4, it is suggested to move the OMI SEI message software implementation from VTM SEI TuC branch to VTM software.

Text updates

- [Ed. (JB): Shouldn’t refer directly to nuh_layer_id syntax element in VSEI, but have an input variable. The same problem exists in depth representation info SEI and alpha channel info SEI.]

To address this comment, reference to syntax element nuh_layer_id is removed and the text changes are as follows. Please note that “layer identifier” is already defined in VSEI spec. Section 3, so there is no need to have an input variable to replace nuh_layer_id

The object mask information (OMI) SEI message provides object mask information of object mask picture in the auxiliary picture layer associated with the primary picture layer, referred to as the current primary picture layer, in which the SEI message is present. If the OMI SEI message is present, it shall be present in a primary picture layer. One primary picture layer can be associated with one or more auxiliary picture layers. For each of the associated auxiliary picture layers containing the object mask picture with nuh_layer_id with layer identifier equal to sdi_layer_id[ i ], the value of sdi_aux_id[ i ] shall be equal to AUX_OBJECT_MASK, for any value of i in range of 0 to sdi_max_layers_minus1, inclusive.

As this problem also exists in depth representation info SEI message and alpha channel info SEI message, if the proposed change can be agreed by the group, it can also be applied on those two SEI messages.

- [Ed. (JB): Is there a requirement that the SDI SEI be present when OMI SEI is present?]

The SDI SEI message indicates the primary layers and auxiliary layers, and provides auxiliary ID of each layer auxiliary layer. Thus, to enable any type of auxiliary picture layer, SDI SEI message should be present. This is not only for OMI SEI message, but also for depth representation info SEI message and alpha channel info SEI message. However, on the other hand, the parsing of OMI SEI message is independent from SDI SEI message. So instead of having a requirement that the SDI SEI shall be present when OMI SEI is present, the proponents suggest to add the following sentence in the text:

When SDI SEI message is not present in the current CVS, the OMI SEI message shall be ignored.

- [Ed. (JB): Is there a requirement that the object mask aux pictures be associated in the SDI SEI with the primary picture carrying the OMI SEI?]

Currently, there is a constraint as follows:

It is a requirement of bitstream conformance that the value of omi_num_aux_pic_layer shall be equal to numAuxLayer where the variable numAuxLayeris derived as follows.

Let omiPrimaryLayerId be the value of nuh_layer_id of the NAL unit that contains the SEI message.

numAuxLayer = 0;
for( i = 0; i <= sdi_max_layers_minus1; i++ )
	if( sdi_aux_id[ i ] = = AUX_OBJECT_MASK )
		for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ )			(xx)
			if( sdi_layer_id[ sdi_associated_primary_layer_idx[ i ][ j ] ]  = =  omiPrimaryLayerId )
				numAuxLayer++;

It means the OMI SEI message only applies to the object mask aux picture layers that are associated in the SDI SEI message with the primary picture layer carrying the OMI SEI message. If there are no object mask aux picture layers being associated in the SDI SEI message with the primary picture layer carrying the OMI SEI message, omi_num_aux_pic_layer shall be zero. So, there is no need to have such a constraint in proponents’ opinion.

- Define the index of object mask picture layer

In the current text, the index of object mask picture auxiliary layer is referenced, but there is no clear definition. That is, it is not clear which object mask auxiliary picture layer is the i-th object mask auxiliary picture layer. Thus, it is proposed to add such a definition in the text. The proposed text changes are as follows

numAuxLayer = 0;
for( i = 0; i <= sdi_max_layers_minus1; i++ )
	if( sdi_aux_id[ i ] = = AUX_OBJECT_MASK )
		for( j = 0; j <= sdi_num_associated_primary_layers_minus1[ i ]; j++ )			(xx)
			if( sdi_layer_id[ sdi_associated_primary_layer_idx[ i ][ j ] ]  = =  omiPrimaryLayerId ) {
				the layer with layer identifier equal to sdi_layer_id[i] is the numAuxLayer-th 					object mask auxiliary picture layer associated with the current primary picture 					layer;
				numAuxLayer++;
			}

- Replace syntax element omi_num_aux_pic_layer with omi_num_aux_pic_layer_minus1

omi_num_aux_pic_layer indicates the number of object mask auxiliary picture layers associated with the current primary picture layer. So, omi_num_aux_pic_layer equal to zero means there is no object mask auxiliary picture layer associated with the current primary picture and consequently there is no need to signal OMI SEI message. Thus, it is proposed to replace syntax element omi_num_aux_pic_layer with omi_num_aux_pic_layer_minus1. The text changes are as follows

| … |  |
| --- | --- |
| omi_num_aux_pic_layer_minus1 | ue(v) |
| … |  |

omi_num_aux_pic_layer_minus1 plus 1 indicates the number of auxiliary picture layers associated with the current primary picture layer.

Conclusion

The contribution updates the OMI SEI message software to align with the current VSEI working draft and refines the text to address the editor’s comments and improve the quality of the text. Considering the OMI SEI message was adopted into VSEI version 4 WD since January 2024, it is suggested to move the OMI SEI software implementation from VTM SEI TuC branch to VTM software and adopt the proposed text changes to the next WD of VSEI version 4.

References

- https://vcgit.hhi.fraunhofer.de/jvet-tuc/VVCSoftware_VTM

- Hendry, J. Nam, M. M. Hannuksela, J. Chen, Y. Ye, Y. -K. Wang, “AHG9: Modifications for signalling of object mask information SEI message”. JVET-AH0346, Rennes, France, April 2024.

- J. Boyce, J. Chen, S. Deshpande, M. M. Hannuksela, Hendry, S. McCarthy, G. J. Sullivan, Y.-K. Wang, Additional SEI messages for VSEI version 4 (Draft 2), JVET-AH2006, Rennes, France, April 2024

Patent rights declaration(s)

Alibaba may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).